3GPP TSG RAN WG4 Meeting #94-e	R4-2002089

Electronic Meeting, February 24 – March 06, 2020

Agenda Item:	8.15.1.2

Source: 	Ericsson

Title:	On activation of multiple SCells

Document for:	Discussion

# Introduction

At RAN4#93, SCell activation delay for activation of multiple SCells with the same MAC command was discussed further. A WF [1] was agreed, with the following open issues:

- Processing time for MAC PDU carrying activation command for multiple SCells

- Requirement scope: Only define requirement for activation of multiple SCells with one MAC-CE per CG

- Delay extension when interruption occurs on L1-RSRP reporting resource of to-be-activated SCell

- Interruption(s) on other serving cells when multiple SCells are being activated

In this contribution we are providing some further analysis regarding the open issues.

# Discussion

Processing time for MAC PDU carrying activation command for multiple SCells

Processing time for parsing MAC-CE and configuring the physical layer correspondingly has been discussed for almost a year by now. We propose introducing a parameter whose value we can define once feedback has been provided by chipset vendors. The existing requirements for activation of a single SCell are using 3ms, and the discussions has been about whether activation of more SCells by the same MAC-CE, and parsing of additional MAC-CEs in the same MAC PDU, e.g. for TCI state configuration, would require additional time. By introducing a parameter the processing time can be added symbolically until it has been define how the value depends on number of SCells being activated and number of additional MAC-CEs being parsed.

It shall be noted that the definition of TFirstSSB and TFirstSSB_MAX, which are used for defining the activation time, both will be depending on TMAC.

Proposal 1: In the activation delay requirement, as well as in TFirstSSB and TFirstSSB_MAX, the processing time for MAC PDU shall be represented symbolically for later specification.

Requirement scope

In general there will not be a very tight coordination between NR PCell/MCG and NR PSCell/SCG, hence requirements for simultaneous activation of multiple SCells do no need to take into account that activation request are received at the same time from PCell and PSCell. We therefor propose that the SCell activation delay requirements for multiple SCells are conditioned on that no other SCell activations are ongoing at the time a MAC-CE for activation of multiple SCells is received.

Proposal 2: SCell activation delay requirements for activation of multiple SCells shall be conditioned on that there are no other NR SCell activations going on when the activation command is received. It shall further assume that an activation command is only received for one cell group (MCG or SCG).

Delay extension when interruption occurs during L1-RSRP measurement:

The activation time is increased by one SMTC for each SSB that would have been used for L1-RSRP measurement but is interrupted due to ongoing SCell activations on other carriers. However, given that the UE only will handle one unknown first SCell in FR2 for which L1-RSRP reporting may be needed, and the L1-RSRP reporting comes at a point in time (after 24 Trs) where any interruptions caused by any other SCell already have occurred, we really do not have to consider this issue.

Proposal 3: For now there is no need to consider whether delay extension is needed or not when a SSB that would have been used for L1-RSRP measurement is interrupted. The interruptions relating to other SCells have already occurred when the UE starts the L1-RSRP measurement.

Interruption of other serving cells when multiple SCells are being activated:

Interruptions on serving cells (other than those being activated) can be reduced if reconfiguration of the radio is carried out for multiple component carriers at the same time – even when their SMTC windows do not overlap. As reduced interruptions – when known also by the base station scheduler – provide clear system benefits, we propose to further specify that the radio reconfiguration for the SCells being activated by the same command as much as possible shall be co-located in time.

Proposal 4: It shall be specified that radio reconfigurations for the SCells being activated as much as possible shall be co-located in time. We may further look into under which conditions this is suitable.

# Summary and Conclusion

In this contribution we have further analysed the open issues regarding activation delay of multiple SCells. The following proposals are made.

On MAC PDU processing time:

Proposal 1: In the activation delay requirement, as well as in TFirstSSB and TFirstSSB_MAX, the processing time for MAC PDU shall be represented symbolically for later specification.

On multiple SCell activation in NR-DC:

Proposal 2: SCell activation delay requirements for activation of multiple SCells shall be conditioned on that there are no other NR SCell activations going on when the activation command is received. It shall further assume that an activation command is only received for one cell group (MCG or SCG).

On delay extension for interruption during L1-RSRP measurement:

Proposal 3: For now there is no need to consider whether delay extension is needed or not when a SSB that would have been used for L1-RSRP measurement is interrupted. The interruptions relating to other SCells have already occurred when the UE starts the L1-RSRP measurement.

On interruptions on other serving cells:

Proposal 4: It shall be specified that radio reconfigurations for the SCells being activated as much as possible shall be co-located in time. We may further look into under which conditions this is suitable.

# References

- R4-1915931 “Way forward on activation delay extension due to multiple SCell activation”, Apple

- R4-1915058 “On activation of multiple SCells”, Ericsson