3GPP TSG-RAN WG4 Meeting #94-e	R4-2002110

Online, 24th February – 6th March 2020

Source: 	NTT DOCOMO, INC.

Title: 	Clarification on Rx image assumption for intra-band non-contiguous EN-DC

Agenda item:	14

Document for:	Approval

- Introduction

RAN4#93 agreed that 25dBc of Rx image rejection for intra-band contiguous NR CA/EN-DC should be assumed to move the discussion forward about power imbalance requirements in UE demod-session [1]. This paper makes a further clarification on Rx image rejection for intra-band non-contiguous case.

- Discussion

RAN4#92bis discussed the power imbalance requirement in UE demod-session, which is described as a scope of Rel-16 NR performance requirement enhancement [2]. It was identified that the assumed value of Rx image rejection on different CCs of intra-band NR CA/EN-DC was needed to discuss the power imbalance requirement, and it was also agreed that the power imbalance discussion is waiting for the outcome of the discussion about the value of the Rx image requirement in UR RF session.

According to the above guidance of UE demod-session, RAN4#93 discussed the value of Rx image rejection for intra-band NR CA/EN-DC in UE RF session, and the conclusion was that the value of 25dBc of Rx image rejection should be assumed for the power imbalance requirement [1].

However, the WF [1] only focused on intra-band contiguous NR CA/EN-DC case although UE demod-session also will discuss intra-band non-contiguous NR CA/EN-DC case [3]. Whether the power imbalance requirement for intra-band non-contiguous NR CA/EN-DC should be specified or not is depending on the discussion of UE demod-session. However, due to the tight schedule of the Rel-16 time-frame, it is better to fix the value of the Rx image rejection for intra-band non-contiguous NR CA/EN-DC in advance in UE RF session. Therefore, we would like to confirm that 25dBc Rx image rejection can be also assumed for intra-band non-contiguous NR CA/EN-DC case.

Proposal: 25 dBc Rx image rejection for FR1 intra-band non-contiguous NR CA and EN-DC should be assumed for power imbalance requirement for UE demodulation requirements.

- Conclusion

In this contribution, we confirmed Rx image rejection for intra-band non-contiguous NR CA/EN-DC case.

Proposal: 25 dBc Rx image rejection for FR1 intra-band non-contiguous NR CA and EN-DC should be assumed for power imbalance requirement for UE demodulation requirements.

- Reference

[1]: R4-1916135, “WF on Rx image rejection for FR1 intra-band contiguous CA/EN-DC”, NTT DOCOMO, INC.

[2]: RP-192281, “Revised WID on NR performance requirement enhancement”, China Telecom, RAN#85

[3]: R4-2000953, “Further discussion on power imbalance requirement for intra-band EN-DC/NR CA”, NTT DOCOMO, INC.