3GPP TSG-RAN WG4 Meeting #94-e		R4-2002157
Electronic Meeting, 24 Feb. – 6 Mar., 2020

Agenda item:	10.2.3.2

Source:	Keysight Technologies

Title:	Joint Power-Angle-Delay profiles for FR2 NR MIMO OTA channel models validation and PSP evaluation

Document for:	Approval

# Introduction

This contribution presents the joint power-angle-delay (PADP) estimates for NR FR2 MIMO OTA models at FR1 frequency band, i.e., 2600 MHz, scaled down from FR2. The aim is to verify the measurement methodology, estimation algorithm and other procedural requirements involved, as outlined in the previous contribution [2] on the subject matter.

# Discussion

The radio channel models of NR MIMO OTA have been defined in [1]. In [2], joint power-angle-delay profile (PADP) was proposed for the validation of emulated environment in the 3D-MPAC setups for the NR FR2 systems and evaluation of PSP metric for system design. Furthermore, the step-by-step test methodology has also been outlined in [2].  For FR2 DUTs, the power angular spectrum (PAS) of the emulated channels relates directly to its beamforming performance. Furthermore, the system bandwidths of FR2 systems will be much larger, leading to higher delay resolution in power delay profile measurement. The joint PADP offers several attractive advantages over conventional marginal profiles (i.e. spatial correlation and power delay profile). The step 5 of the test methodology in [2] defines the estimation of joint PADP from measurements with the aid of sequential search algorithm. In this contribution, we present the preliminary PADP measurements for NR FR2 MIMO OTA channel models, but in a FR1 chamber with uniform 8 vertically polarized probes and frequency scaled down to 2600 MHz band. At this stage, the limitation to FR1 is primarily due to the ongoing preparatory work for full-scale channel model validation in an FR2 3D-MPAC OTA setup.

Table 1: Measurement parameters and settings

| S.No | Item | Unit | Value |
| --- | --- | --- | --- |
| 1 | Center frequency | MHz | 2600 |
| 2 | Span | MHz | 160 |
| 3 | Number of traces |  | 500 |
| 4 | Number of points |  | 1601 |
| 5 | Averaging |  | 1 |
| 6 | Uniform circular array (UCA) radius | cm | 12 |
| 7 | UCA element spacing | λ | 0.5 |
| 8 | Range length | m | 1.3 |
| 9 | OTA probes |  | 8 |
| 10 | Measurement time for one model | hrs | 12 |

Figure 1: Joint PADP FR2 CDL-A InO model in a vertically polarized 8 probe FR1 OTA setup at 2600 MHz

Figure 2: Joint PADP FR2 CDL-C UMi model in a vertically polarized 8 probe FR1 OTA setup at 2600 MHz

Figures 1 and 2 illustrates the measured joint PADPs for CDL-A InO and CDL-C UMi NR MIMO OTA models in a uniform vertically polarized 8 probe FR1 OTA setup (horizontal plane) at 2600 MHz frequency band. The directional frequency response, S21, corresponding to 16 uniform spatial samples on the test zone is acquired through VNA with the parameter settings as given in Table 1. The measured data is then subjected to sequential search algorithm for estimating PAPDs. The results indicate reasonable accuracy in estimating the power-delay profiles for the target directions in the horizontal plane. The FR1 OTA setup utilized lack probes in the elevation probes, thereby limiting the directional analysis only to the horizontal plane. However, it was observed that the rather significant measurement time is a major challenge with the measurement parameters considered here. One model took approximately 12 hrs for the parameters given in Table 1, which is significant.

However, it was confirmed that a subset of the proposed test approach, i.e., without fading and the Channel Emulator on bypass, could be leveraged for the QoQZ measurements which is certainly beneficial.

Observation 1: For the reasonable measurement parameters required for the estimation, the corresponding measurement times could be large. Therefore, it is crucial to define suitable measurement parameters that are efficient in terms of time and the estimation accuracy is not compromised.

Proposal 1: Further investigation is required for defining suitable measurement parameters. Feedback and proposals are welcomed.

# Conclusion

We present the joint power-angle-delay (PAPD) profile results for NR FR2 MIMO OTA models at FR1 frequency band, i.e., 2600 MHz. The purpose is to verify the testing methodology and the estimation algorithm before full-scale model validation in 3D-MPAC setup at FR2 frequencies. The testing method and accuracy of the estimation algorithm has been successfully verified with an important observation as below.

Observation 1: For the reasonable measurement parameters required for the estimation, the corresponding measurement times could be large. Therefore, it is crucial to define suitable measurement parameters that are efficient in terms of time and the estimation accuracy is not compromised.

Proposal 1: Further investigation is required for defining suitable measurement parameters. Feedback and proposals are welcomed.

# References

- 3GPP TR 38.827 “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)” V1.0.0, December 2019.

- R4-1915077, on NR FR2 MIMO OTA Channel Model Validation, CAICT, Spirent, CMCC, SGS Wireless, 3GPP TSG-RAN WG4 Meeting #93bis, November 2019.