3GPP TSG-RAN WG4 Meeting #94-e	R4-2002292

Electronic Meeting, Feb.24th – Mar.6th 2020

Agenda item:			6.11

Source:	Moderator (Ericsson)

Title:	Email discussion summary for RAN4#94e_#43_NR_NewRAT_RRM_Perf_Part_1

Document for:	Information

# Introduction

The documents in agenda items 6.11.1 and 6.11.2 mainly contains CRs to correct test configuration or test cases. There are following 2 main topics and sub-topics under each main topic:

- Topic #1: Correction to RRM test configuration

  - Issue 1-1: new OCNG pattern

  - Issue 1-2: correction to TRS configuration

- Topic #2: Correction to RRM tests

  - Issue 2-1: Editorial corrections

  - Issue 2-2: Correction to event used in RRM test with FR2

  - Issue 2-2: Correction to AoA setup in FR2 RRM tests

# Topic #1: Correction to RRM test configuration

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2001619 | Huawei, HiSilicon | Cat F CR: define new OCNG pattern for FR2 where OCNG is not transmitted in REs in SSB slots. |
| R4-2001620 | Huawei, HiSilicon | Cat A CR: |
| R4-2001592 | Huawei, HiSilicon | Cat F CR: TRS resources (CSI-RS) location in FR1 is changed to symbol 9 in the slot. |
| R4-2001593 | Huawei, HiSilicon | Cat A CR: |

## Open issues summary

### Sub-topic 1-1

Issue 1-1: new OCNG pattern

- Proposals

  - New OCNG pattern for FR2 is proposed where OCNG is not transmitted in REs in SSB slots.

- Recommended WF

  - Collect feedback from companies if there is any concern on introducing new OCNG pattern and on technical details in the CR in R4-2001619.

### Sub-topic 1-2

Issue 1-2: correction to TRS configuration

- Proposals

  - TRS resources (CSI-RS) location for CSI-RS resource 2 and 4 in FR1 is changed to symbol # 9 in the slot. In the existing configuration the CSI-RS location for CSI-RS resource 2 and 4 in FR1 is symbol # 10.

- Recommended WF

  - Collect feedback from companies if there is any concern on the proposed changes to the CR in R4-2001592.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2001592 | Anritsu As can be seen in TS 38.214 5.1.6.1.1, configuration of symbol is defined as follows. , , or for frequency range 1 and frequency range 2, Therefore the combination of 6&9 as proposed in the CR cannot be configured. Feb.26 To comment from Huawei/HiSilicon We are also fine to change the TRS combination to symbol 5 and 9 in a slot. |
|  | Huawei/HiSilicon |
|  | Thanks for Anritsu’s comments. We are fine to change the TRS combination to symbol 5 and 9 in a slot. |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1-1 | Tentative agreements: Introduce new OCNG pattern for FR2 where OCNG is not transmitted in REs in SSB slots. Candidate options: No other option. Recommendations for 2nd round: Agree CR |
| Sub-topic#1-2 | Tentative agreements: Change the TRS combination to symbol 5 and 9 in a slot. Candidate options: No other option. Recommendations for 2nd round: Revise CR in R4-2001592 to capture the above tentative agreement. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2001619 | Agreeable |
| R4-2001592 | To be revised to capture agreements in sub-topic # 1-2 |

## Discussion on 2nd round (if applicable)

|  | Status summary |
| --- | --- |
| Sub-topic#1-2 | Issue 1-2: correction to TRS configuration 2nd round: capture tentative agreements on TRS configuration in CR CRs/TPs CR/TP number; Decision. R4-2001592; Revised. Capture agreements for issue 1-2. R4-2001593; Return to. Cat A CR for R4-2001592. |

| Company | Comments |
| --- | --- |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2002214 | Agreeable |
| R4-2001593 | Agreeable. Cat A CR to R4-2002214. |

# Topic #2: Correction to RRM tests

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2000293 | Samsung | Cat F CR: Editorial changes |
| R4-2000294 | Samsung | Cat A CR |
| R4-2000515 | ZTE Corporation | Cat F CR: Editorial changes |
| R4-2000516 | ZTE Corporation | Cat A CR |
| R4-2001223 | ANRITSU LTD | Cat F CR: Editorial changes |
| R4-2001225 | ANRITSU LTD | Cat A CR |
| R4-2001365 | Rohde & Schwarz | Cat F CR: Corrected event from event A3 to event A4 in tests in annex A.5 |
| R4-2001366 | Rohde & Schwarz | Cat A CR |
| R4-2001367 | Rohde & Schwarz | Cat F CR: Corrected event from event A3 to event A4 in tests in annex A.7 |
| R4-2001368 | Rohde & Schwarz | Cat A CR |
| R4-2001369 | Rohde & Schwarz | Cat F CR: Correction or addition of AoA setup in different FR2 RRM test cases in annex A.5 |
| R4-2001370 | Rohde & Schwarz | Cat A CR |
| R4-2001371 | Rohde & Schwarz | Cat F CR: Correction or addition of AoA setup in different FR2 RRM test cases in annex A.8 |
| R4-2001372 | Rohde & Schwarz | Cat A CR |

## Open issues summary

### Sub-topic 2-1

Issue 2-1: Editorial corrections

- Proposals

  - Editorial corrections in R4-2000293, R4-2000515 and R4-2001223.

  - Note: some of the changes related to spelling and font size of titles in R4-2000515 and R4-2001223 are supposed to be done by MCC (ETSI edit help)

- Recommended WF

  - Agree CRs in R4-2000293, R4-2000515 and R4-2001223 and their respective cat A CRs unless any concern is raised.

### Sub-topic 2-2

Issue 2-2: Correction to event used in RRM test with FR2

- Proposals

  - The CR changes event A3 to event A4 in the test cases in R4-2001365 and R4-2001367. Ok, it is A4 as A4 threshold and levels are for A4

- Recommended WF

  - The tests in R4-2001365 and R4-2001367 are supposed to use event A4 and A4 threshold is also configured. Mistakenly A3 is mentioned in the tests.

  - Agree CRs in R4-2001365 and R4-2001367 and their respective cat A CRs unless any concern is raised.

### Sub-topic 2-3

Issue 2-2: Correction to AoA setup in FR2 RRM tests

- Proposals

  - The AoA setup to be used in the tests is missing or incorrect in several test cases. The missing AoA setup is added or corrected in the test cases.

- Recommended WF

  - RAN4 has previously agreed with AoA setup for different FR2 RRM test cases.

  - Interested companies to review the AoA setup updated or added in different FR2 RRM test cases in R4-2001369 and R4-2001371.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2001369 | Huawei, HiSilicon: - For CSI-RS RLM test case the AoA setup has been decided in RAN4#91 under R4-1907233. Non-DRX will be setup 3 and DRX will be setup 1 - Do we need same changes for corresponding test cases in section A.7? |
|  | R&S to HW: The intention of the CR was to rather clarify the CR. The decision in R4-1907233 for the CSI-RS RLM TCs could also be implemented in a revision if (especially in case of Setup 3) it is clear which signals are mapped to which AoA. After the changes in A.5 are stable, the clean-up can be done also for a A.7 (in a separate CR). |
|  | Company B |
| R4-2001223 | Anritsu: This CR was withdrawn as advised by Kai-Erik, as the corrections can be done by ETSI Edit help. Similarly the Cat A CR R4-2001225 was withdrawn. |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | Tentative agreements: Editorial changes in cat-F CR in R4-2000293 are agreeable. Editorial changes in cat-F CR in R4-2000515 are agreeable but it also contains spelling mistake which should be corrected by ETSI helpdesk over the entire specification TS 38.133. Proponent of cat-F CR in R4-2001223 has withdrawn the CR as it contains changes to be corrected by ETSI helpdesk. Candidate options: None Recommendations for 2nd round: cat-F CR in R4-2000293 to be agreed and cat-F CR in R4-2000515 to be revised. |
| Sub-topic#2-2 | Tentative agreements: Changing event A3 to event A4 in the test cases as proposed in cat-F CRs in R4-2001365 and R4-2001367. Candidate options: None Recommendations for 2nd round: Cat-F CRs in R4-2001365 and R4-2001367 can be agreed. |
| Sub-topic#2-3 | Tentative agreements: Correct AoA setup in the test cases as proposed in cat-F CRs in R4-2001369 and R4-2001371. Correct also AoA setup in tests cases in section A.7 in this meeting or in the next meeting. Candidate options: None Recommendations for 2nd round: cat-F CRs in R4-2001369 and R4-2001371 can be agreed. Possible correction to AoA setup in tests cases in section A.7. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2000293 | Agreeable |
| R4-2000515 | Revise CR to exclude spelling mistakes |
| R4-2001223 | Withdrawn |
| R4-2001365 | Agreeable |
| R4-2001367 | Agreeable |
| R4-2001369 | Agreeable |
| R4-2001371 | Agreeable |

## Discussion on 2nd round (if applicable)

|  | Status summary |
| --- | --- |
| Topic#2 | Topic #2: Correction to RRM tests CRs/TPs CR/TP number; Decision. R4-2000515; Revised. Exclude spelling mistakes. R4-2000294; Return to. Cat A CR for R4-2000515. |

| Company | Comments |
| --- | --- |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2002215 | Agreeable |
| R4-2002330 | Agreeable. Cat A CR to R4-2002215 (revision of R4-2000516). |