3GPP TSG-WG RAN4 Meeting #94-e	R4-2002634

# Online, 24th February – 6th March, 2020

Source: 	Nokia, Nokia Shanghai Bell, [AT&T]

Title: 					TP to TR 36.716-04-01: CA_2-7-26-66

Agenda Item: 			7.4.2

Document for:	Approval

# Introduction

This TP introduces the following LTE inter-band CA for 4 bands DL with 1 band UL for the CA bands CA_2-7-26-66.

- CA_2A-7A-26A-66A_BCS0

**************************** Start of changes ********************************************

## 5.X	CA_2-7-26-66

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Supported E-UTRA bandwidths per CA configuration for 4-band DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| CA_2A-7A-26A-66A | 2 |  | Yes | Yes | Yes | Yes | Yes | 75 | 0 |
|  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 26 |  | Yes | Yes | Yes | Yes |  |  |  |
|  | 66 |  | Yes | Yes | Yes | Yes | Yes |  |  |

### 5.X.2	∆TIB and ∆RIB values

The ∆TIB and ∆RIB values of CA_2-7-26-66 is proposed to be based on CA_2-7-66 in TS 36.101 for the high bands. The relaxation of the low band, i.e., band 26, is based on the diplexer inserstion loss.

Table 5.X.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2-7-26-66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 26 | 0.3 |
|  | 66 | 0.5 |

Table 5.X.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_2-7-26-66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 26 | 0 |
|  | 66 | 0.5 |

### 5.X.3	REFSENS requirements

There is no REFSENS exception for CA_2A-7A-26A-66A.

**************************** End of changes ********************************************