3GPP TSG-RAN WG4 Meeting #94-e	R4-2002662

Online, 24th February – 6th March 2020

Source:	Ericsson, US Cellular

Title:	TP to TR 37.716.21-21: Addition of DC configurations for DC_2_n12-n260

Agenda item:	9.7.3

Document for:	Approval

# Background

This text proposal for TR 37.716-21-21 to add DC_2_n12-n260 configurations as defined in WID [1].

# Text Proposal

##### ---Start of changes---

## 6.x	DC_2_n12-n260

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n12-n260 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 30 | 40 | 50 | 100 | 200 | 400 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n12A-n260A | DC_2A_n260A DC_2A_n12A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 435 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n12 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  |

### 6.x.3	Co-existence studies

Co-existence analysis for DC_2_n260 shows that there is no impact from DC_2_n260 UL to Band n12 DL.

Co-existence analysis for DC_2_n12 shows that there is no impact from DC_2_n12 UL to Band n260 DL.

### 6.x.4	∆TIB and ∆RIB values

For DC_2_n12-n260 the TIB,c and RIB,c values are same as for DC_12_n2_and are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n12-n260 | 2 | 0.3 |
|  | n12 | 0.3 |
|  | n260 | 0 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n12-n260 | 2 | 0 |
|  | n12 | 0 |
|  | n260 | 0 |

### 6.x.5	MSD

There is no need to define MSD.

##### ---End of changes---

# Reference

[1]	RP-192592, “Revised WID on EN-DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)”, LG Electronics