3GPP TSG-RAN WG4 Meeting #94-e				  											R4-2001288 Online, 24th Feb. 2020 – 6th Mar. 2020

Source: 	Nokia, Verizon

Title: 		TP for 37.716-11-11 to introduce DC_48_n66

Agenda Item: 			9.3.2 [DC_R16_1BLTE_1BNR_2DL2UL-Core]

Document for:	Approval

# 1	Introduction

This contribution is a TP for TR 37.716-11-11 to introduce DC_48_n66.

# 2	Text Proposal

*********************** Start of the TP ***************************************************

### 6.1.x	DC_48_n66

#### 6.1.x.1	Operating bands for DC

Table 6.1.x.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_48_n66 | 48 | n66 | No |

#### 6.1.x.2	Configuration for DC

Table 6.1.x.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_48A_n66A | DC_48A_n66A | 48A | n66A |

#### 6.1.x.3	Maximum output power for DC

The maximum output power for the uplink EN-DC configuration for DC_66A_n48A, is already specified in TS 38.101-3 Table 6.2B.1.3-1 and can be reused for DC_48A_n66A.

Table 6.1. x.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_48A_n66A | 23 | +2/-3 |

#### 6.1.x.4	Spurious emission band UE co-existence for DC

The requirements for the uplink EN-DC configuration for DC_66A_n48A, is already specified in TS 38.101-3 Table 6.5B.3.3.2-1 and can be reused for DC_48A_n66A.

Table 6.1. x.4-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_48_n66 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |

#### 6.1.x.5	MSD analysis for DC

MSD analysis have been conducted for DC_66A_n48A and can be reused for DC_48_n66.

#### 6.1.x.6	∆TIB and ∆RIB values

For DC_48_n66, the TIB,c and RIB,c values are reused from DC_66A_n48A already specified in TS 38.101-3.

Table 6.1.x.6-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48_n66 | 48 | 0.8 |
|  | n66 | 0.6 |

Table 6.1.x.6-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_48_n66 | 48 | 0.5 |
|  | n66 | 0.2 |

#### 6.1.x.7	MSD

Based on the co-existence studies for DC_66A_n48A MSD [R4-1912575] which have been defined in TS 38.101-3 Table 7.3B.2.3.5.1-1 and can be reused for DC_48_n66 the following can be defined.

Table 6.1. x.7-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (two bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_48A_n66A | 48 | 3630 | 20 | 100 | 3630 | N/A | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | 4 | IMD5 |

Further it is in the co-existence studies for DC_66A_n48A MSD [R4-1912575] found that 2nd and 3rd order harmonic may fall into Rx frequency of band 48. As a result, the reference sensitivity exceptions (MSD) due to UL harmonic have been aligned to those of DC_66_n48 [R4-2002571]

Table 6.1. x.7-2: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n66 | 482,13 | 27.3 | 24.4 | 22.4 | 21.2 |  |  |  |  |  |  |  |  |
|  | 483 | 1.9 | 1.4 | 0.9 | 0.4 |  |  |  |  |  |  |  |  |
| NOTE 1: Void NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 13: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the EN-DC band combination: ∆FHD = 10 MHz for DC_1_n77, DC_2_n77, DC_66_n77, DC_3_n77, DC_3_n78 and DC_48_n66 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1. x.7-3: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

|  | E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n66 | 48 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |  |

*********************** End of the TP ***************************************************

# 3	References