3GPP TSG-RAN WG4 Meeting #94-e	R4-2002886

Electronic Meeting, Feb.24th – Mar.6th 2020

Agenda item:			8.12.1

Source:	Moderator (China Telecom)

Title:	Email discussion summary for RAN4#94e_#17_NR_DL256QAM_FR2

Document for:	Information

Introduction

In the last RAN4 #93 meeting, the following agreements were reached in the chairman’s meeting report.

- Agreement:

  - In 256QAM WI, BS Tx EVM will be defined as core requirements

  - Whether to introduce the UE maximum input level as core requirements will be further discussed.

  - In 256QAM WI, UE demod performance requirements and testability will be discussed in the performance part of this WI.

- Agreement:

  - BS TX EVM core requiremetns for DL 256QAM FR2 is agreed as 3.5%

Based on the above agreements and companies’ contributions submitted in this e-meeting, this email discussion will focus on the following topics

- Draft TR

- BS requirements including core and conformance requirements

- UE core requirements

Topic #1 Draft TR

Companies’ contributions summary

| T-doc number | Company | Abstracts / Proposals / Observations |
| --- | --- | --- |
| R4-2000909 | China Telecom | Update TR to implement TPs approved in last meeting. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1: Draft TR

- Recommended draft TR: R4-2000909

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |

CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2000909 |  |

Summary for 1st round

    - Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic 1-1 | No concern on the draft TR |

    - CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2000909 | The draft TR is recommend as approved |

Discussion on 2nd round (if applicable)

Topic #1 is concluded on 1st round, no need discuss on 2nd round.

Summary on 2nd round (if applicable)

Topic #1 is concluded on 1st round, no need discuss on 2nd round.

Topic #2: BS requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Abstracts / Proposals / Observations |
| --- | --- | --- |
| R4-2000910 | China Telecom | Abstract: This TP is intended to capture the BS core requirement for FR2 DL 256QAM |
| R4-2001189 | NTT DOCOMO, INC. | Proposal: Adopt BS TX EVM test requirements for FR2 DL 256QAM as 4.5% |
| R4-2001426 | Nokia, Nokia Shanghai Bell, China Telecom, Verizon, NTT Docomo, T-Mobile | Abstract: FR2 DL 256QAM requirements are introduced to the technical specification |
| R4-2001427 | Nokia, Nokia Shanghai Bell, China Telecom, Verizon, NTT Docomo, T-Mobile | Abstract: FR2 DL 256QAM requirements are introduced to the conformance specification |
| R4-2001729 | Ericsson | Abstract: Add minimum EVM requirement for BS type 2-O carrier |
| R4-2002103 | Ericsson | Abstract: Add conformance requirement for BS type 2-O carrier |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1: BS EVM core requirement CR

This sub-topic will discuss the CRs/TP for BS EVM core requirement. Given we have two CRs overlapping, both the CRs are listed below. Companies can discuss on how to select or merge the CRs.

Issue 2-1-1: BS EVM core requirement CR/TP

- Recommended CR: R4-2001426/R4-2001729

- Recommended TP: R4-2000910

Sub-topic 2-2: BS conformance requirement

This sub-topic will discuss the BS conformance requirement.

The first issue 2-2-1 is BS TX EVM test requirement, for which we have seen differfent proposals from contributions.

The second issue 2-2-2 is CR for BS conformance requirement. Given we have two CRs overlapping, both the CRs are listed below. Companies can discuss on how to select or merge the CRs, if we ccould achieve agreement on the recommended WF on the first issue and no other issues were raised.

Issue 2-2-1: BS TX EVM test requirement

- Proposal

  - Define BS TX EVM test requirements as 4.5% (R4-2001189, R4-2001427)

  - Define BS TX EVM test requirements as 3.5% (R4-2002103)

- Recommended WF/Contribution

  - Define BS TX EVM test requirements as 4.5% (R4-2001189)

Issue 2-2-2: BS conformance requirement CR

- Recommended CR: R4-2001427/R4-2002103

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Issue 2-1-1: It’s ok to merge the CR for core requirement. Issue 2-2-1: There is a type-o in the CR R4-2002103 should be 4.5% since this is conformance and TT need to be taken into account. However, guidance from chairman last meeting as we also submitted a conformance CR last meeting (R4-1914570) only when performance work starts; focus on the core requirements until April. In which case, all conformance CRs should be noted as anyhow these contributions are for discussion/information. Issue 2-2-2: CRs should be noted. …. Others: |
| Nokia, Nokia Shanghai Bell | Issue 2-1-1: The coversheet is not correct in R4-2001729 and therefore R4-2001427 is preferred. Issue 2-2-1: We support the WF of defining 4.5% test requirement. Otherwise the test tolerance is not correctly addressed. Issue 2-2-2: R4-2002103 has not addressed declarations, test tolerance for EVM nor included 256QAM in total power dynamic range. Therefore, our preference is to move forward with R4-2001427 |
| Huawei | Issue 2-2-1: Test requirement should be 4.5% |
| Ericsson | Issue 2-1-1: Should coversheet in R4-2001427 have starting date of the meeting? Since the technical content is exactly the same between the 2 contributions; we ask to be added as co-source on R4-2001427 since it’s the preferred version. Issue 2-2-1/2: Only core related requirements should be handled. All others noted. |
| China Telecom | Issue 2-2-1/2: The conformance test requirement shall be discussed when core requirement is stable, from this point, we prefer to move forward with the proposals/CRs submitted in this meeting. |
| NTT DOCOMO | Issue 2-2-1/2: We should focus on completion of core requirements and corresponding CR in this meeting. For conformance requirements, there seems to be no other issues, but we would like to follow chairman’s guidance. |

CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2000910 | Company A |
|  | Company B |
| R4-2001426 | Company A |
|  | Company B |
| R4-2001427 | Company A |
|  | Company B |
| R4-2001729 | Company A |
|  | Company B |
| R4-2002103 | Company A |
|  | Company B |

Summary for 1st round

    - Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic 2-1 | Tentative agreements: No concern on the CRs’ technical content. No concern on the TP Recommendation for 2nd round: The CR R4-2001426 is recommended to be revised to capture Ericsson’s comment |
| Sub-topic 2-2 | Tentative agreements: No technical concern on the CR R4-2001427. No technical concern on the contribution R4-2001189 1st round views collection for conformance requirement in this meeting: - Technically endorse the BS conformance CR since no technical concern - Note all the conformance requirement related contributions Recommendation for 2nd round: - Continue discuss on the conformance requirement in this meeting. |

    - CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2000910 | The TP is recommended as approved |
| R4-2001189 | The contribution is recommended as return to |
| R4-2001426 | The CR is recommended as to be revised to capture Ericsson’s comment |
| R4-2001427 | The CR is recommended as return to |
| R4-2001729 | The CR is recommended as noted |
| R4-2002103 | The CR is recommended as return to |

Discussion on 2nd round (if applicable)

Open issues summary

Open issues for Topic #2 was captured in the WF, and will be handled in Topic #3, section 3.5

Summary on 2nd round (if applicable)

Summary for Topic #2 will be handled in Topic #3, section 3.6

Topic #3: UE core requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Abstracts / Proposals / Observations |
| --- | --- | --- |
| R4-2000823 | Huawei, HiSilicon | Proposal: UE maximum input level is not defined for 256QAM for Rel-16 WI. |
| R4-2000911 | China Telecom | Abstract: This TP is intended to capture the UE core requirement for FR2 DL 256QAM |
| R4-2000954 | Intel Corporation | Observation #1: Rel-15 Maximum Input Level requirements are defined only for QPSK modulation and definition of only 256QAM core requirements (without 64QAM) looks rather confused. Proposal 1: Introduce FR2 Maximum Input Level core requirements for 256QAM jointly with requirements for 64QAM. |
| R4-2001190 | NTT DOCOMO, INC. | Proposal: Introduce UE maximum input level core requirements for FR2 DL 256QAM. |
| R4-2001425 | Nokia, Nokia Shanghai Bell, China Telecom, Verizon, NTT Docomo, T-Mobile | Abstract: Introduction of UE requirements related to the feature of 256QAM DL transmission in FR2, i.e. maximum input power requirement and RMC for 256QAM. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 3-1: UE core requirements

This sub-topic will discuss the UE core requirements.

The first issue 3-1-1 is UE maximum input level, for which we have seen differfent proposals from contributions.

The second issue are CR and TP for UE core requirements. The CR and TP submitted are listed as recommendation in case we could achieve agreeement on recommended WF on the first issue and no other issues were raised.

Issue 3-1-1: UE maximum input level

- Proposal

  - Will be introduced for 256QAM in Rel-16 WI.

  - Will not be introduced for 256QAM in Rel-16 WI

  - Will be introduced for 256QAM jointly with requirements for 64QAM in Rel-16 WI

- Recommended WF

  - Will be introduced for 256QAM in Rel-16 WI

Issue 3-1-2: UE core requirements CR/TP

- Recommended CR: R4-2001425

- Recommended TP: R4-2000911

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Issue 3-1-1: We support the proposed WF. Issue 3-1-2: We support the proposed WF. |
| Huawei | Issue 3-1-1: We think it will be confused if only introduction of 256 QAM. Hence our preference is not introduced in Rel-16. |
| Intel | Issue 3-1-1: We think that definition of only 256QAM FR2 Maximum Input Level Requirements (i.e. without 64QAM) will be rather confusing in future, because LTE and NR FR1 requirements are defined for both modulations formats. Same time, we understand that definition of FR2 core requirements for 256QAM is beneficial. Therefore, we suggest to return 64QAM requirements, which were removed earlier, and then define 256QAM requirements. Issue 3-1-2: We suggest to discuss this issue once we reach agreement on Issue 3-1-1. |
| Qualcomm | Issue 3-1-1: For consistency of the standard, we would like to see joint introduction of max. input level requirements for 64QAM and 256QAM |
| China Telecom | Issue 3-1-1/2: We think the requirements for 64QAM and 256QAM shall be discussed decoupled, which means defining 256QAM requirement has no impact to 64QAM. In order not to make the spec confusing, the requirements for 64QAM could be supplemented in the next meeting. In this meeting, we prefer to move forward with the CR/TP to draw a technical conclusion at least. |
| NTT DOCOMO | Issue 3-1-1/2: We support recommended WF since there are no feasibility issues. In addition, the requirement for 256QAM can be discussed independently of the requirement for 64QAM. |
| Apple | Issue 3-1-1: RAN5 concludes “the minimum conformance requirements in this test case are not testable due to maximum input level unachievable in IFF OTA test setup. Other test setups have not been analysed. Thus the test case will not be tested as part of UE conformance testing” in 38.521-2. Additionally, DL 256QAM in FR2 is part of objectives of Rel-17 SI on FR2 test methodology enhancement, which includes the study of 256QAM related test methodology. Before RAN5 confirms the testing feasibility and RAN4 draws conclusion out of Rel-17 SI on FR2 test methodology enhancement, we think it is premature and also not urgent for RAN4 to introduce such requirements. |

CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2000911 |  |
| R4-2001425 |  |

Summary for 1st round

    - Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic 3-1 | Candidate options collected in first round for UE maximum input level: - Introduced for 256QAM in Rel-16 WI - Introduce for 256QAM jointly with that for 64QAM in Rel-16 WI - Not introduce for 256QAM in Rel-16 WI - Decide by RAN5 test feasibility and RAN4 conclusion out of Rel-17 SI on FR2 test methodology enhancement Recommendation for 2nd round: - Assign a WF for the discussion for both BS and UE requirements |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on the requirements for FR2 DL 256QAM | China Telecom |

    - CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2000823 | The contribution is recommended as noted |
| R4-2000911 | The TP is recommended as return to |
| R4-2000954 | The contribution is recommended as noted |
| R4-2001190 | The contribution is recommended as noted |
| R4-2001425 | The CR is recommended as return to |

Discussion on 2nd round (if applicable)

Open issues summary

The Open issues for Topic #2 and Topic #3 were captured in R4-2002803 WF on the requirements for FR2 DL 256QAM. Company are encouraged to discussed the WF on the email thread: ”RAN4#94e_#17_NR_DL256QAM_FR2 – draft WF R4-2002803”

Companies views’ collection for Open issues for 2nd round

This table will collect the formal comments for the WF on the requirements for FR2 DL 256QAM after discussion or revison on the email thread ”RAN4#94e_#17_NR_DL256QAM_FR2 – draft WF R4-2002803”, if companies would like to fill in.

| Company | Comments |
| --- | --- |
| NTT DOCOMO | Regarding UE core requirement, we agree to introduce the requirement for 256QAM in Rel-16. However, for 64QAM, the requirement should be introduced from Rel-15 since this is Rel-15 feature. We prefer to clarify that the requirement for 64QAM will be introduced from Rel-15. |
| Apple | Thanks for the WF. Based on the first round discussion, I think the contents in this WF is still premature. Considering the diverse views from interested companies, I suggest we focus on 2nd round discussion and the contents in the WF should be drafted accordingly based on the 2nd round discussion. |
| China Telecom | To Apple, For UE maximum input level, I see several views from interesting companies, which have already captured in the way forward. For your comment that the WF is premature, I think majority companies mentioned that only define 256QAM requirement without 64QAM is a little confusing. Based on this background, we try to compromise to the possible agreement that define both 256QAM and 64QAM requirements. In our understanding, this is the most important part what we need to discuss in the 2nd round. To NTT DOCOMO, I agree with you that 64QAM shall start from Rel-15. In our understanding the work for 64QAM shall be done in the UE maintenance, however it depends on other companies views in which ways to introduce. I can clarify that the requirement for 64QAM will be introduced from Rel-15. |
| Ericsson | Question regarding slide 4, regarding “agreements for core requirements” for BS requirements to technically endorse the core requirement? This has already been technically endorsed in last meeting (Reno), so maybe to avoid confusion we can remove this bullet. As for BS conformance, I think based on chairman’s guidance we should note these contributions until core has been defined. |
| Nokia, Nokia Shanghai Bell | Could you provide the Tdoc reference of CR to 38.104 which was endorsed in the last meeting? As far as I know CR was not technically endorsed. For conformance part, I have not seen any technical concern to the proposed content so not sure what is the issue to technically endorse it? There is no proposal at this time to approve CR. |
| Verizon | There was no detail substance in conclusion from last meeting on this topic! Then, this WF provides particular proposals, including the 256QAM jointly with that for 64QAM and the BS core CR. We do prefer to move forward the WF for Rel-16 from this meeting as companies don't see technical concern so far for the proposed content. |
| Ericsson | Maybe I can help by clarifying. The 3.5% has already been agreed for BS RF EVM DL core requirement based on chairman’s notes: - Agreement: - BS TX EVM core requiremetns for DL 256QAM FR2 is agreed as 3.5% I understood that the core requirement (38.104) would capture this already based on the email discussion (R4-2002690) R4-2001426; The CR is recommended as to be revised to capture Ericsson’s comment. However it seems that in the WF R4-2002803 that companies prefer to agree UE and BS requirements as package. That is ok with us as well. Either method we have no strong opinion. Regarding BS conformance, my understanding is that there is consensus from all companies to have 4.5%. In order to capture this agreement I have revised the text in the WF. Please find the updated (uploaded to https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_94_e/Inbox/Drafts/%2317_NR_DL256QAM_FR2) version with our revisions in red. |
| Nokia, Nokia Shanghai Bell | As indicated in my previous e-mail, CR to 38.104 was not technically endorsed in the last meeting. Therefore, in order to make a progress, we support to technically endorse CR to 38.104 in this meeting. For CR 38.141-2 we are fine to include text on test requirement of 4.5%. However, since CR to 38.141-2 captures also details on manufacturer’s declarations and test models, it is important to confirm no technical concerns have been raised. I have made additional changes, note Nokia and Nokia Shanghai Bell would like to co-sign this WF. https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_94_e/Inbox/Drafts/%2317_NR_DL256QAM_FR2/R4-200xxxx%20WF%20on%20the%20requirements%20for%20FR2%20DL%20256QAM_v1_Ericsson_Nokia.pptx |
| Ericsson | Regarding 1427, there is an error in section 6.6.3.4.1: For BS type 2-O, set the BS to transmit a signal according to the applicable test configuration in subclause 4.8 using the corresponding test models: - NR-FR2-TM2a if 256QAM is supported by BS, or - NR-FR2-TM2 with highest modulation order supported if 64QAM is not supported by BS; Also small spelling error of “applicable” in the NOTE 18. The wording can also be improved as it’s a run on sentence that is a bit strange. The highlighted green text should be 256 QAM not 64 QAM. Although I do understand this was probably an oversight on editing, I would prefer to keep the WF as the previous version submitted. R4-200xxxx WF on the requirements for FR2 DL 256QAM_v1_Ericsson. |
| Apple | I think our concern is still there. Firstly, 64QAM is out of scope of this discussion and we should treat it in a separated agenda. Secondly, I would like to reiterate my round 1 comments on 256QAM that RAN5 concludes “the minimum conformance requirements in this test case are not testable due to maximum input level unachievable in IFF OTA test setup. Other test setups have not been analysed. Thus the test case will not be tested as part of UE conformance testing” in 38.521-2. Additionally, DL 256QAM in FR2 is part of objectives of Rel-17 SI on FR2 test methodology enhancement, which includes the study of 256QAM related test methodology. Before RAN5 confirms the testing feasibility and RAN4 draws conclusion out of Rel-17 SI on FR2 test methodology enhancement, we think it is premature and also not urgent for RAN4 to introduce such requirements. Overall, the maximum input level related contents in this WF are not agreeable to us. |
| Nokia, Nokia Shanghai Bell | I don’t agree with you with this change in green. We think that CR 1427 is correct and it should be like proposed. But I agree that at first glance it looks not perfect and not so logical (the reason is many changes on this text done) and not super clear text right now in specification, the meaning is following as I explain: For BS type 2-O, set the BS to transmit a signal according to the applicable test configuration in subclause 4.8 using the corresponding test models: - NR-FR2-TM2a if 256QAM is supported by BS, or [use TM2a with 256QAM if it is supported] - NR-FR2-TM2 with highest modulation order supported if 64QAM is not supported by BS; [use TM2 with highest supported modulation so either 64QAM/16QAM/QPSK] The issue is that current text in spec is not so clear, but we wanted to avoid big changes. In our opinion these steps in many places in spec require some clear text, but this should be done as separate work, not together with 256QAM introduction. |
| Ericsson | Actually in general I don’t see why the text format should really be different from 1-O which is different in the few lines above this: For BS type 1-O, set the BS to transmit a signal according to the applicable test configuration in subclause 4.8 using the corresponding test models: - NR-FR1-TM2a in TS 38.141-1 [3] subclause 4.9.2.2.4 if 256QAM is supported by BS; - or NR-FR1-TM2 in TS 38.141-1 [3] subclause 4.9.2.2.3 if 256QAM is not supported by BS; The NOTE 18 has strange wording also. Again the guidance was to focus on core requirements for this meeting, and as I understand your intension is why I proposed to have the WF capture also the EVM test requirement and agree on the core CR as I highlighted earlier in the efforts towards progressing the work. Actually, I don’t believe this WF is even needed as we have already agreed to the EVM core requirement for BS RF. |
| China Telecom | To Apple, Basically, the core requirements are used for product design targets and do not have to be testable. In fact, some non-testable requirements have been introduced in FR2 so far. I think your comments for the whether and how to test issue have already been discussed in several meetings. You can find the full discussion in the contribution R4-1909982 and R4-2001190. But of course we have no consensus on this point. How about if we propose the following proposal as a compromise for making the progress as - Introduce UE maximum input level for 256QAM jointly with that for 64QAM in Rel-16 WI - Discuss on how to define the UE maximum input level for 256QAM in the next meeting without considering testing feasibility To Ericsson and Nokia, Given the comment for CR 1427 is also related to a spec clear issue. I think we could endorse the CR to make a progress for 256QAM with a comment that - Technically endorse the CR to 38.141-2 (R4-2001427) since no technical concern Modification may be provided in the next meeting for the CR from Spec clear point |
| Apple | Thanks for the effort to lead the discussion. I wonder why maximum input level for 256QAM should be discussed without testing feasibility considered? I think we have the similar situation in Rel-16 for 64QAM. We eventually decide not to introduce this requirements based on RAN5’s feasibility input. I think we should follow a similar approach for this case. With this, the suggested revision is as follow o Agreements for core requirements:  Discuss on how to define the UE maximum input level for 256QAM in the next meeting without considering testing feasibility  Agree the BS and UE CRs core requirements in a package in Rel-16 WI  Technically endorse the CR to 38.104 (R4-2002802) since no technical concern. |
| China Telecom | Thanks for the further comment. Actually, in our understanding, the maximum input level share the same situation on whether testable or not no matter QPSK, 64 QAM or 256QAM. Further if QPSK was testable, then 256QAM shall be testable given the requirement for 256QAM is intended to be relaxed to some extent. Why not just follow QPSK approach to make the requirements complete. I think the Rel-17 SI for test method is target to solve the common issue which not only exists in DL 256QAM. But anyway we need to further discuss this in next meeting, I will follow Apple’s suggestion to upload the final 2nd round summary and final WF since by now no any other comments. |

CRs/TPs comments collection for 2nd round

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

The contributions which were recommended as return to during the 1st round, comments will be collected in the following table for 2nd round.

| CR/TP number | Status after 1st round | Comments collection for 2nd round |
| --- | --- | --- |
| R4-2001189 | return to | Company A |
|  |  | Company B |
| R4-2002802 | return to replace of R4-2001426 |  |
| R4-2001427 | return to |  |
| R4-2002103 | return to |  |
| R4-2000911 | return to |  |
| R4-2001425 | return to |  |

Summary on 2nd round (if applicable)

    - Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic 3-1 | Tentative agreements: approve the WF as following - Agreements for core requirements: - Discuss on how to define the UE maximum input level for 256QAM in the next meeting - Agree the BS and UE CRs in a package in Rel-16 WI - Technically endorse the CR to 38.104 (R4-2002802) since no technical concern - Agreement for BS conformance requirement: - Technically endorse the CR to 38.141-2 (R4-2001427) since no technical concern - Modification may be provided in the next meeting for the CR from Spec clear point |

    - CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2001189 | The contribution is recommended as approved |
| R4-2002802 | The CR is recommended as technically endorsed |
| R4-2001427 | The CR is recommended as technically endorsed |
| R4-2002103 | The CR is recommended as noted |
| R4-2000911 | The TP is recommended as noted |
| R4-2001425 | The CR is recommended as noted |