3GPP TSG RAN WG4 #94-e			R4-2002767

Electronic Meeting, 24th February – 6th March, 2020

3GPP TSG RAN WG1 #99			R1-1913580

Reno, USA, 18th – 22nd, Nov. 2019

Title:	LS on NR Rel-16 TEI

Response to:

Release:	Rel-16

Work Item:	TEI16

Source:	RAN WG1

To:	RAN WG2, RAN WG4

Cc:	TSG RAN

Contact Person:

#### Name:	Hiroki Harada

E-mail Address:	hiroki.harada.sv (at) nttdocomo.com

Attachments:

1. Overall Description:

According to the guidance from RAN, RAN1 had NR Rel-16 TEI discussion.

RAN1 discussed on a TEI proposal regarding TRS for FR1, and RAN1 made following agreements.

Agreement: Regarding TEI “TRS for FR1” - For FR1 in Rel-16, UE can be configured with one-slot periodic TRS configuration only when no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated - Note: One-slot periodic TRS configuration for FR1 in Rel-16 is an optional UE feature - The TP in R1-1912551 is endorsed in principle.

RAN1 discussed on a TEI proposal regarding downgrading configuration of SRS for antenna switching, and RAN1 made following agreements.

Agreement: Regarding TEI “Enable gNB to configure downgrading configuration of SRS for antenna switching” - Rel-16 UE capability design for SRS antenna switching in conjunction with the existing Rel-15 UE capability should allow UE to indicate support of one of the following combinations - {t1r1, t1r2} - {t1r1, t1r2, t1r4} - {t1r1, t1r2, t2r2, t2r4} - {t1r1, t2r2} - {t1r1, t2r2, t4r4} - {t1r1, t1r2, t2r2, t1r4, t2r4} - Note: Detailed signaling design is up to RAN2 - The TP in R1-1913445 is endorsed in principle - Note: RAN1 spec can be further updated depending on the final RAN2 signalling design

RAN1 discussed on a TEI proposal regarding half-duplex operation in CA, and RAN1 made following working assumption.

Working assumption: Regarding TEI “Half-duplex operation in CA” - Agreements made during email discussion [95-NR-06] on directional collision handling for half duplex CA operation are adopted in Rel-16. - A new UE capability to indicate the support for directional collision handling between reference and other cell(s) is introduced. - Note that UEs that do not indicate this capability should still be able to operate half-duplex TDD CA if network ensures same transmission direction across all the serving cells. - The TP in R1-1913440 is endorsed in principle

RAN1 discussed on a TEI proposal regarding RACH configuration considering TDD configuration for FR1, and RAN1 made following agreements.

Agreement: Regarding TEI “RACH configuration considering TDD configuration for FR1” - Following new RACH configuration entries with subframe number 2 and/or 7 are added into RACH configuration table for Rel-16 UE PRACH Configuration Index; Preamble format; Subframe number; Starting symbol; Number of PRACH slots within a subframe; , number of time-domain PRACH occasions within a PRACH slot; , PRACH duration. new1; 0; 16; 1; 7; 0; -; -; 0. new2; 0; 8; 1; 7; 0; -; -; 0. new3; 0; 4; 1; 7; 0; -; -; 0. new4; 0; 2; 0; 7; 0; -; -; 0. new5; 0; 2; 1; 7; 0; -; -; 0. new6; 0; 2; 0; 2; 0; -; -; 0. new7; 0; 2; 1; 2; 0; -; -; 0. - The new RRC parameter prach-ConfigurationIndexNew is introduced and the value range of prach-ConfigurationIndexNew is [256, 262]. - A new UE capability is not introduced for this TEI, i.e., it is a mandatory UE feature for Rel-16. - The TP in R1-1913466 is endorsed in principle.

RAN1 will work on CR and UE capability aspects for the agreed TEI proposals.

2. Actions:

To RAN WG2 and RAN WG4:

RAN1 respectfully asks RAN2 and RAN4 to take above information into account.

3. Date of Next RAN WG1 Meeting:

TSG-RAN WG1 Meeting #100	24 – 28 Feb 2020			Athens, Greece

# TSG-RAN WG1 Meeting #100bis		20th – 24th April 2020		Busan, Korea