3GPP TSG-RAN WG4 Meeting #94-e	R4-2002769

Electronic Meeting, February 24th – March 6th, 2020

3GPP TSG-RAN WG1 Meeting #99	R1-1913585

Reno, USA, November 18th – 22nd, 2019

# Title:	Reply LS on Tx switching between two uplink carriers

# Response to:	R1-1911810/R4-1913040

# Release:	Rel-16

# Work Item:	NR_RF_FR1

Source:	TSG RAN WG1

To:	TSG RAN WG4

Cc:	TSG RAN WG2

Contact Person:

Name:	Jianchi Zhu

E-mail Address:	zhujc@chinatelecom.cn

1. Overall Description:

RAN1 thanks RAN4 for the LS on switching period between case 1 and case 2. RAN1 has reached the following agreements and conclusions in RAN1#99 meeting:

Agreements: (in response to RAN4 LS)

- There is no RAN1 impact on the length of switching period.

- The determination of length of switching period is up to RAN4.

Agreements:

- In response to the RAN4 LS, RAN1 sees no issues on the location of the switching period.

Agreements: (in response to RAN4 LS)

- There is no RAN1 impact on the transient period.

- The determination of transient period is up to RAN4.

Agreements:

- If the UL switching period does not exist, additional time is not needed for PUSCH preparation procedure.

- If the UL switching period actually exists due to Tx switching

- Additional time is needed for PUSCH preparation procedure time.

- The length of the additional time will be decided in next RAN1 meeting.

Agreements:

- For standalone SUL

  - No such issue of concurrent transmission between 1Tx transmission on carrier 1 and 2Tx transmission on carrier 2.

  - Assume RAN4 is discussing the RRC parameter to activate the Tx switching.

- For inter-band UL CA, UE is not expected to be scheduled 1Tx transmission on carrier 1 and 2Tx transmission on carrier 2 simultaneously.

  - It is captured in RAN1 spec.

  - Assume RAN4 is discussing the RRC parameter to activate the Tx switching.

Agreements:

- For standalone SUL, if UL switching period is configured by RRC

- The switching period is not always applicable on the carrier configured with switching period.

- The switching period is only applicable when the scheduled UL transmissions are switched between 1Tx carrier 1 and 2Tx carrier 2.

  - For each UL transmission occasion on a carrier, the existence of the switching period is determined one time every occasion.

UL transmission occasion refers to PUSCH/PUCCH/SRS/PRACH transmission occasion defined in clause 7 of TS 38.213.

Conclusion:

- The condition of the presence of the switching period for inter-band UL CA and inter-band EN-DC without SUL are to be captured in RAN1.

- RAN1 will continue discussing the related issue and solutions (including the applicability period of switching (in terms of number of slot(s)).

- There is no additional RAN4 impact.

2. Actions:

To RAN WG4:

ACTION:

RAN1 respectfully asks RAN4 to take the above information into account.

3. Date of Next RAN WG1 Meetings:

TSG RAN WG1 Meeting #100               24 - 28 Feb, 2020                    Athens, GR

TSG RAN WG1 Meeting #100bis           20 - 24 Apr, 2020                    Busan, KR