3GPP TSG-RAN WG4 Meeting #94-e	R4-2002775

Electronic Meeting, February 24th - March 6th, 2020

3GPP TSG-RAN WG2 Meeting #108	R2-1916468

Reno, United States, 18 - 22 November 2019

Title:	Reply LS on UL-SL prioritization

Response to:	R2-1914318/R1-1911720

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	RAN2

To:	RAN1, RAN4

Cc:

Contact Person:

#### Name:	Qianxi Lu

- E-mail Address:	qianxi.lu at oppo.com

# 1. Overall Description

RAN2 would like to thank RAN1 for their reply LS in R1-1911720 on UL-SL prioritization.

During RAN2#107bis, RAN2 made the following agreements,

Agreements on prioritization:

1: 	A separate LCH priority thresholds is configured for both NR-UL and NR-SL.

2:	For between SL-data and UL-data/SRB, the SL transmission is prioritized if the highest priority value of UL LCH(s) with available data is larger than the UL priority threshold and the highest priority value of SL LCH(s) with available data is lower than the SL priority threshold. Otherwise the UL transmission is prioritized.

3:	Prioritization between UL SR and SL data transmission could be based on priority of the UL LCH that triggered the UL SR and priority value(s) of SL LCH(s), similar as prioritization between NR UL data and NR SL data transmission.

During RAN2#108, RAN2 made the following agreements,

Agreements on UL/SL prioritization:

1: 	For prioritization between SL-TX and SL-triggered SR, it is based on direct comparison between associated LCH priority.

2:	For prioritization between SL-TX and UL-TX (only for PUSCH), for UL MAC CE, rely on LTE solution, i.e., they are treated as if of priority lower than the UL-threshold, so down-prioritized if SL-TX is higher than SL-threshold, otherwise prioritized.

3:	For LTE-UL/NR-SL and NR-UL/LTE-SL, if the two RATs cannot exchange prioritization-related information prior to time of transmission subject to processing time restriction, it is up to UE implementation to decide whether UL or SL to prioritize.

4:	If the two RATs can exchange prioritization-related information prior to time of transmission subject to processing time restriction, rely on LTE solution for LTE-UL/NR-SL and NR-UL/LTE-SL prioritization.

5:	RAN2 does not need to handle the MCG-SL/SCG-UL collision.

RAN2 assumes how to handle all other physical channels in UL/SL prioritization is up to RAN1.

2. Actions:

RAN2 respectfully asks RAN1 and RAN4 to take the above information into account in their future work.

3. Date of Next RAN2 Meetings:

TSG-RAN2 Meeting #109                        24 - 28 Feb. 2020               Athens, Greece

TSG-RAN2 Meeting #109bis                      20 - 24 Apr 2020               Japan