3GPP TSG-RAN WG4 Meeting #94bis-e                                                          R4-2003113

Online, 20th April-30th April, 2020

Source:	Samsung, KDDI

Title:	TP for TR 36.716-02-01: CA_1A-41C

Agenda item:			5.2.3

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 36.716-02-01 to include CA_1A-41C_ UL _41C according to the request in [1].

# 2. Reference

- RP-200231, Revised WID: Rel16 LTE inter-band CA for 2 bands DL with 1 band UL.

# 3. Text Proposal

<Start of Text Proposal>

## 5.x	CA_1-41

### 5.x.1	Channel bandwidths per operating band for CA

Table 5.x.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

Table 5.x.1-2: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-41C | CA_41C | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

### 5.x.2	Co-existence studies

Table 5.x.2-1 summarizes frequency ranges where harmonics occur due to Band 1 and Band 41 CA with 1 UL.

Table 5.x.2-1: Impact of 1 UL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 1 | 1920 | 1980 | 2110 | 2170 | 3840 | 3960 | 5760 | 5940 |
| 41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 |

It can be seen from Table 5.x.2-1 that there is no harmonic impact towards any of its own downlink bands.

### 5.x.3		ΔTIB,c and ΔRIB,c values

CA_1A-41C has the same ΔTIB,c and ΔRIB,c  as existing combination CA_1A-41A.

Table 5.x.3-1: ΔTIB,c for 2DLs aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-41 | 1 | 0.5 |
|  | 41 | 0.5 |

Table 5.x.3-2: ΔRIB,c for 2DLs aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-41 | 1 | 0 |
|  | 41 | 0 |

<End of Text Proposal>