3GPP TSG-RAN WG4 Meeting #94bis-e                                                         R4-2003131

Online, 20th April-30th April, 2020

Source:	Samsung, KDDI, LGE

Title:	TP for TR 37.716-21-11: DC_18-41_n3

Agenda item:			8.4.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.716-21-11 to include DC_18-41_n3 according to the request in [1].

# 2. Reference

- RP-200271, Revised WID on Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL).

# 3. Text Proposal

<Start of Text Proposal>

## 5.1.x	DC_18-41_n3

### 5.1.x.1		Operating bands for DC

Table 5.1.x.1-1: Band combination EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_18-41_n3 | CA_18-41 | n3 | No |

### 5.1.x.2	Configuration for DC

Table 5.1.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_18A-41A_n3A | DC_18A_n3A DC_41A_n3A | CA_18A-41A | n3A |
| DC_18A-41C_n3A | DC_18A_n3A DC_41A_n3A DC_41C_n3A | CA_18A-41C | n3A |

### 5.1.x.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, the IMD issues for this band combination are as below list:

-  2nd and 3rd order IMDs generated by dual uplink of band 18 and band n3 maybe fall into the Rx of band 41. However, B18 only used in Japan. So specific band combinations can considered, then only IMD3 will be impact to the own Band 41.

-  2nd and 3rd order IMDs generated by dual uplink of band 41 and band n3 maybe fall into the Rx of band 18.

### 5.1.x.4	∆TIB and ∆RIB values

For DC_18-41_n3, the TIB,c and RIB,c values are given in the tables below, based on the values of its consistent 2 Bands fallback mode.

Table 5.1.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18-41_n3 | 18 | 0.3 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.5 |
| NOTE 1: The requirement is specified for the frequency range of 2545-2690MHz. NOTE 2: The requirement is specified for the frequency range of 2496-2545MHz. |  |  |

Table 5.1.x.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_18-41_n3 | 18 | 0 |
|  | 41 | 01/0.52 |
|  | n3 | 0 |
| NOTE 1: The requirement is specified for the frequency range of 2545-2690MHz. NOTE 2: The requirement is specified for the frequency range of 2496-2545MHz. |  |  |

### 5.1.x.5	REFSENS requirements

Table 5.1.x.5-1 shows the required MSD levels for the DC configuration.

Table 5.1.x.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

|  | E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration |  | EUTRA and NR band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode |  |
| DC_18A-41A_n3A DC_18A-41C_n3A |  | 18 | 820 | 5 | 25 | 865 | 5 | N/A | FDD | N/A |
|  |  | n3 | 1725 | 5 | 25 | 1820 | 5 | N/A | FDD | N/A |
|  |  | 41 | 2630 | 5 | 25 | 2630 | 5 | 16.0 | TDD | IMD3 ¦2*fB18-fBn3¦ |
|  |  | 18 | 820 | 5 | 25 | 865 | 5 | 28.9 | FDD | IMD21 ¦fB41-fBn3¦ |
|  |  | n3 | 1765 | 5 | 25 | 1860 | 5 | N/A | FDD | N/A |
|  |  | 41 | 2630 | 5 | 25 | 2630 | 5 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD3 falling down band18 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |  |

<End of Text Proposal>