3GPP TSG-RAN WG4 Meeting #94bis-e                                                         R4-2003133

Online, 20th April-30th April, 2020

Source:	Samsung, KDDI, LGE

Title:	TP for TR 37.716-21-11: DC_18-41_n78

Agenda item:			8.4.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.716-21-11 to include DC_18-41_n78 according to the request in [1].

# 2. Reference

- RP-200271, Revised WID on Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL).

# 3. Text Proposal

<Start of Text Proposal>

## 5.1.x	DC_18-41_n78

### 5.1.x.1		Operating bands for DC

Table 5.1.x.1-1: Band combination EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_18-41_n78 | CA_18-41 | n78 | No |

### 5.1.x.2	Configuration for DC

Table 5.1.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_18A-41A_n78A | DC_18A_n78A DC_41A_n78A | CA_18A-41A | n78A |
| DC_18A-41C_n78A | DC_18A_n78A DC_41A_n78A DC_41C_n78A | CA_18A-41C | n78A |

### 5.1.x.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, there is only IMD5 impact to Band 18 considering Operator’s spectrum, since B18 only was used in Japan.

### 5.1.x.4	∆TIB and ∆RIB values

For DC_18-41_n78, the TIB,c and RIB,c values are given in the tables below, based on the values of its consistent 2 Bands fallback mode.

Table 5.1.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18-41_n78 | 18 | 0.3 |
|  | 41 | 0.3 |
|  | n78 | 0.8 |

Table 5.1.x.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_18-41_n78 | 18 | 0 |
|  | 41 | 0 |
|  | n78 | 0.5 |

### 5.1.x.5	REFSENS requirements

Table 5.1.x.5-1 shows the required MSD levels for the DC configuration.

Table 5.1.x.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| c |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_18A-41A_n78A DC_18A-41C_n78A | 18 | 820 | 5 | 25 | 865 | 3.4 | FDD | IMD5 ¦2*fn78-3*fB41¦ |
|  | n78 | 3527.5 | 10 | 50 | 3527.5 | N/A | FDD | N/A |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | TDD | N/A |

<End of Text Proposal>