3GPP TSG-RAN WG4 Meeting #94bis-e                                                          R4-2003161

Online, 20th April-30th April, 2020

Source:	Samsung, KDDI

Title:	TP for TR 38.716-01-01: CA_n78(2A)_UL_n78(2A)

Agenda item:			8.1.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.716-01-01 to include CA_n78(2A)_UL_n78(2A ) according to the request in [1].

# 2. Reference

- RP-200101, Revised WID on Rel-16 NR intra band Carrier Aggregation for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y).

# 3. Text Proposal

<Start of Text Proposal>

## 6.x	CA_2DL_n78(2A)_ 2UL_n78(2A)

### 6.x.1 Operating band for CA

Table 6.x.1-1: intra-band non-contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n78 | n78 |

### 6.x.2 Channel bandwidths per operating band for CA

Table 6.x.2-1: Supported bandwidth combinations for CA_2DL_n78(2A)_2UL _n78(2A)

| NR CA Configuration | Uplink Configurations | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n78(2A) | CA_n78(2A) | 10, 20, 40, 50, 60, 80, 90, 100 | 10, 20, 40, 50, 60, 80, 90, 100 | 200 | 0 |
|  |  | 10, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 10, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 200 | 1 |

### 6.x.3 UE maximum output power for CA

Table 6.x.3-1 UE Power Class for intra-band non-contiguous CA

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n78(2A) |  |  |  |  | 23 | +2/-3 |  |  |

### 6.x.4 Co-existence studies

### There are no co-existence issues for this combination.

### Table 6.x.4 lists the protected bands required for the 2UL intra-band non-contiguous CA configuration

Table 6.x.4-1: Spurious emission band UE co-existence for CA_n78(2A)

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n78(2A) | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. |  |  |  |  |  |  |  |

### 6.x.5 REFSENS

### There are no REFSENS exceptions for this combination. UL configuration for REFSENS is listed below.

Table 6.x.5-1: Intra-band non-contiguous CA with two uplink configuration for reference sensitivity

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | UL SCC allocation | ΔR2UL_PCC (dB) | ΔR2UL_SCC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n78(2A) | NOTE 1 | NOTE 2 | NOTE 3 | NOTE 4 | 0.0 | 0.0 | TDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.6A.1-3. NOTE 2: All applicable sub-block gap sizes. NOTE 3: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE 4: The SCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. |  |  |  |  |  |  |  |

<End of Text Proposal>