3GPP TSG-RAN WG4 Meeting # 94-e-Bis 	                                        R4-2003207

Electronic Meeting, 20 – 30 Apr., 2020

Agenda Item:	6.8.2.1.1

Source:	Intel Corporation

Title:	Further Discussion on NR PRS RSTD Requirements for UE

Document for:		Discussion

# Introduction

In the last RAN4 meeting, UE PRS RSTD requirements were discussed, and some agreements achieved [1]. In this contribution beside some important requirements for PRS measurement requirements presented in [2] we will also provide further considerations on open issues below [1].

- SINR side condition

- Definition of intra and inter-frequency

- Measurement period requirements

- RSTD report mapping

# SINR Side Condition

In the last meeting the following agreement for PRS RSTD measurement SINR side condition was achieved[1].

- Side condition for PRS-RSTD reference cell in FR1 to be PRS Es/Iot = -6 dB - Side condition for PRS-RSTD in FR2 to be: - Option 1: Same as FR1 - Option 2: PRS Es/Iot = -3 dB for reference cell and PRS Es/Iot = -10 dB for neighbor cells

In principle the SINR side condition applied shall be up to the probability that UE can detect the sufficient neighbor cells for the measurements. Thus from the results of the system level simulation provided in R4#93 [3][4][5] for FR2 more than 95% UEs can hear > 2 strongest cells/TRPs when SINR of reference cell and neighbor cells are up to -6dB and -13dB respectively.

However, in the last meeting, some concerns were raised because of the worse measurement performance with the SINR side condition of Option 1 especially under TDL-C propagation channel. Therefore, we also provide some simulation results for FR2 with SINR below.

- Option 1: PRS Es/Iot = -6 dB for reference and PRS Es/Iot = -13 dB for neighbour

The other simulation assumptions can be found in [6] also.

The simulation results for FR2 is given in Figure 1 below.

Figure 1. RSTD simulation results in FR2

From the results in Figure 1, it was noted the performance of PRS RSTD can be quite diverse depending on UE detection algorithm. For an example, if the first arrived path in case of multiple path can be detected correctly, RSTD performance can achieve significant improvements.

Observation 1: RSTD measurement accuracy will be less than [-40.69ns, 89.51ns] under TDL-C propagation channel.

In comparison with existing LTE requirements (e.g. minimum RSTD error required is about 128ns (4Ts) [7]), NR PRS-RSTD measurement performance with SINR=[-6,-13]dB for reference cell and neighbor cells in FR2 can be much better to meet the LTE’s RSTD measurement accuracy requirements.

Therefore, it is safe to propose that:

Proposal 1: SINR side conditions for PRS-RSTD in FR2 can be

- PRS Es/Iot = -6 dB for reference cell and

- PRS Es/Iot = -13 dB for neighbor cells

# Intra/Inter-frequency PRS Measurement Definition

In the latest RAN4 meeting, the following agreements on the definition of intra/inter frequency PRS measurement were achieved in [1].

- RAN4 to define intra-frequency and inter-frequency RSTD measurements and the corresponding requirements - Candidate options for definition of intra-/inter-frequency: - Option 1. Intra-frequency RSTD measurement is defined as when the neighbor DL PRS resource and the reference DL PRS resource belong to the same positioning frequency layer. Otherwise, the RSTD measurement is inter-frequency. - Option 2. Intra-frequency RSTD measurement is defined when neighbor DL PRS resource and reference DL PRS resource belong to the same positioning frequency layer and the BW of the positioning frequency layer is within the BW of UE’s active DL BWP. Otherwise, the RSTD measurement is inter-frequency. - Option 2a: Intra-frequency RSTD measurement is defined when the BW of the positioning frequency layer is within the BW of UE’s active DL BWP and SCS of PRS is same as the SCS of UEs’s active DL BWP. Otherwise, the RSTD measurement is inter-frequency. - Option 3. Intra-frequency RSTD measurement: the center frequency of PRS BW is the center frequency of an serving cell SSB and has the same SCS as that of the serving cell SSB, otherwise it is inter-frequency. (NOTE: for RSTD, the above conditions are met for both reference and the other DL links) - Option 4. Intra-frequency RSTD measurement is defined if the PRS resources to be measured are in the same positioning frequency layer, or the PRS resource to be measure are not in the same positioning frequency layer but have the same SCS and center frequency, and the bandwidth are all within the active BWP. Otherwise, the RSTD measurement is inter-frequency

Generally, in our views all of these options can clearly define the intra/inter frequency PRS measurement with their own ways. However, in comparison with other candidates, Option 1 can be most simple way, by which the intra-frequency measurement could be identified when the measurement frequency layers for the reference cell and the neighbour cells are same. Similar definitions for LTE and NR SSB measurement can be also found in TS36.300 [8] for CSIR-RS measurement and TS38.133 [9] for SSB measurement. Thus, for the consistence with the previous intra/inter frequency layer definition in LTE and NR Rel15, the similar approach shall be considered.

For CSI-RS based discovery signals measurements, "cell" should be interpreted as "transmission point of the concerned cell" in the following descriptions. Intra-frequency neighbour (cell) measurements, inter-frequency neighbour (cell) measurements and inter-RAT measurements are defined as follows: - Intra-frequency neighbour (cell) measurements: Neighbour cell measurements performed by the UE are intra-frequency measurements when the current and target cell operates on the same carrier frequency. - Inter-frequency neighbour (cell) measurements: Neighbour cell measurements performed by the UE are inter-frequency measurements when the neighbour cell operates on a different carrier frequency, compared to the current cell.

## 9.2 NR intra-frequency measurements ### 9.2.1 Introduction A measurement is defined as a SSB based intra-frequency measurement provided the centre frequency of the SSB of the serving cell indicated for measurement and the centre frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs are also the same.

Observation 2: It is more straightforward to define of intra/inter-frequency PRS measurement with similar approaches of LTE and NR SSB measurement. That is the intra/inter PRS-RSTD measurement can be defined independently with UE active BWP.

At a same time, if RAN4 specifying UE requirements like that of maximum number of legacy measurement frequency layer in NR (e.g. 9.1.3.1a SA: Monitoring of multiple layers using gaps[9]), it is better to be consisted with the “positioning frequency layer” which is consisted with the conclusions of RAN1. As a result, when considering the requirements on the number of layers to be measured with gap, RAN4 can depend on RAN1’s agreements below accordingly.

Agreement:in RAN1#99 UE can be configured for DL PRS processing according to the following table: Description; Maximum numbers for DL PRS resources; Values that can be signaled as part of UE Capability. Max number of frequency layers (X1); X1=4; Values = {1,4} FFS: other values. Max number of TRPs per frequency layer (X2); X2=64. Max number of PRS resource sets per TRP (X3) per frequency layer; X3=2; Values = {1,2}. Max number of Resources per PRS resource set (X4); X4=64; FFS: values. Max number of DL PRS Resources per UE (X5); NA; FFS: values. Max number of TRPs for all frequency layers (X6) per UE; 256; FFS: values. Max number of Resources per frequency layer (X7); NA; FFS: values. Note: This does not constrain in any way how features and feature sets are defined. The values in the table above may or may not be signalled to be different for different features or feature sets.

Observation 3: UE requirements on the number of measured frequency layers for PRS measurement s in RAN4 RRM can be also defined by the number of measurement positioning frequency layer as RAN1 agreed.

Initially the main purpose to define the intra and inter frequency measurement is to differentiate whether a measurement is non-gap or gap-assisted for UE exactly like the scenarios in LTE [8]. Thus, we also give more examples of the typical measurement scenarios regarding to the gap applicability with the intra/inter frequency measurement definition of Option 1 below.

- intra-frequency without gap

From RAN4 perspective, if the reference cell is same as the serving cell in NR positioning measurement the intra-frequency can be given in Figure 2 below.

Figure 2. Non-gap intra-frequency PRS measurement

It shall be noted when PRS bandwidth is larger than the bandwidth of BWP of serving cell, UE can perform the PRS measurement with the best effort over the wider PRS bandwidth if no gap is requested.

- intra-frequency with gap

If the central frequency of reference cell is different with that of the serving cell in NR positioning measurement the intra-frequency can be given in Figure 3 below. That is UE needs to switch its RF operation to other frequency which is different that of UE serving cell. It is normally to request the measurement gap by UE.

Figure 3. Gap-assisted intra-frequency PRS measurement

- inter-frequency with gap

For inter-frequency PRS measurements, there are multiple measurement layers in the different central frequency. UE also needs request the measurement gap during switching to other RF carriers for PRS measurement indeed.

Figure 4. Inter-frequency PRS measurement

In summary, we can conclude that:

Proposal 2: Regarding to the consistent definition for NR measurement as simple as possible, the intra/inter frequency PRS measurement can be defined as:

|  | Definition | Need gap or Not |
| --- | --- | --- |
| intra-frequency | DL PRS resources of the measured neighbor cells/TRPs and the serving/reference cell/TRP have - the same SCS and CP type - the same centre frequency - the same point-A - same configured PRS BW | NO: If the central frequency of reference cell is same as that of the serving cell. |
|  |  | Yes: If the central frequency of reference cell is different with that of the serving cell. |
| inter-frequency | DL PRS resources of the measured neighbor cells/TRPs and the serving/reference cell/TRP is different in any of - SCS and CP type - centre frequency - point-A - configured PRS BW | Yes |

# Measurement Reporting Delay

With respect to the reporting requirements for the OTDoA measurement in E-UTRA [10] below, the delay requirement of DL PRS RSTD will depend on number of factors which will be addressed below: DL PRS Ês/Iot, the number of positioning occasions and the number of consecutive subframes, periodicity of positioning occasions, measurement sampling rate, measurement bandwidth, beam management, and other network assistance information, etc.

##### 8.1.2.5 E-UTRAN FDD Intra-Frequency OTDOA Measurements When the physical layer cell identities of neighbour cells together with the OTDOA assistance data are provided, the UE shall be able to detect and measure intra-frequency RSTD, specified in TS 36.214 [4], for at least n=16 cells, including the reference cell, on the same carrier frequency f1 as that of the reference cell within TRSTD IntraFreqFDD, E-UTRAN ms as given below (see also Figure 8.1.2.5.1-1): TRSTD IntraFreqFDD, E-UTRAN=TPRS ⋅(M-1)+Δ ms , where TRSTD IntraFreqFDD, E-UTRAN is the total time for detecting and measuring at least n cells, TPRS is the cell-specific positioning subframe configuration period as defined in TS 36.211 [16], M is the number of PRS positioning occasions as defined in Table 8.1.2.5.1-1, where each PRS positioning occasion comprises of NPRS (1≤NPRS≤6) consecutive downlink positioning subframes defined in TS 36.211 [16], and Δ = 160⋅nM ms is the measurement time for a single PRS positioning occasion which includes the sampling time and the processing time. Table 8.1.2.5.1-1: Number of PRS positioning occasions within TRSTD IntraFreqFDD, E-UTRAN Positioning subframe configuration period TPRS; Number of PRS positioning occasions M. f1 Note1; f1 and f2 Note2. 160 ms; 16; 32. >160 ms; 8; 16. Note 1: When only intra-frequency RSTD measurements are performed over cells belonging to the serving FDD carrier frequency f1. Note 2: When intra-frequency RSTD and inter-frequency RSTD measurements are performed over cells belonging to the serving FDD carrier frequency f1 and one inter-frequency carrier frequency f2, respectively.. The UE physical layer shall be capable of reporting RSTD for the reference cell and all the neighbor cells i out of at least (n-1) neighbor cells within TRSTD IntraFreqFDD, E-UTRAN provided: PRS Es/Iotref-6 dB for all Frequency Bands for the reference cell, PRS Es/Ioti-13 dB for all Frequency Bands for neighbour cell i, PRS Es/Iotref and PRS Es/Ioti conditions apply for all subframes of at least L=M2 PRS positioning occasions, PRP 1,2¦dBm according to Annex B.2.5 for a corresponding Band PRS Es/Iot is defined as the ratio of the average received energy per PRS RE during the useful part of the symbol to the average received power spectral density of the total noise and interference for this RE, where the ratio is measured over all REs which carry PRS.

Particularly from UE implementation perspective, when considering the measurement period, some complexity aspects should be accounted. Typically, there are several implementation aspects below which shall be taken count into:

- RSTD measurements cannot be processed in real time, thus some limited buffering of data and correlation results is needed.

- Furthermore, as discussed in context of L1 measurement period for the RSTD, it should be set so that it allows UE to do the measurements and calculations also in serial manner, rather than requiring all the cells being processed simultaneously to alleviate the processing and memory consumption requirements.

Therefore, if UE needs to be able to perform intra- and inter-frequency measurements at the same time, the processing complexity and memory consumption can easily become multiple of the number of layers to be considered, unless some limitations are set. Different approaches to alleviate the complexity concern can be considered, but it would appear to be most straight forward that the allowed L1 measurement period is multiplied by the number of layers that need to be measured.

Observation 4: PRS RSTD measurement delay requirement can be determined by:

- the periodicity of DL PRS resource

- the total number of DL PRS resource which UE can process

Therefore, we can propose that:

Proposal 3: The allowed RSTD measurement delay would be multiplied by the number of PRS resource that UE can measure as defined for LTE RSTD measurement.

TRSTD IntraFreqFDD, NR=TPRS ⋅(M-1)+Δ  ms

Where in, TPRS is the periodicity of DL PRS allocation in slots configured per DL PRS resource,

M is the number of PRS positioning resource for all cells (e.g. n=16 in [7]) to be detected and measured,

Δ  is the measurement time for a single PRS positioning resource which includes the sampling time and the processing time.

For each frequency layer configured, both “M” and “Δ”is are highly related to UE capability and implementation, e.g. the length of PRS signal correlation window. Meanwhile, in the last RAN1 meeting the UE processing capability can be defined as:

Agreement: Duration of DL PRS symbols in units of ms a UE can process every T ms assuming 272 PRB allocation is a UE capability

However, in our views such definition is not clear enough to provide UE processing capability enough. For an example, whether the capability is proportinal with BW or not. It also implies that we can define the relevant parameters (e.g. “M” above) until UE capability on PRS processing time was agreed in RAN1.

Proposal 4: The number of PRS positioning resource to be detected and measured can be FFS waiting RAN1 agreements.

# RSTD Measurement Under Cell Change

For RSTD measurement in E-UTRA, when the serving cell is changed (e.g. handover) UE can continue RSTD measurements for those cells if the necessary assistance data is already available for UE. For NR RSTD measurement, the LCS server will forward the assistance data for multiple cells/TRPs which can be heard by UE. Theoretically the target HO cell/TRP is in the list of LCS server to be measured because of its stronger signal quality. In other words, UE can continue PRS measurement in these target cells/TRPs for HO indeed because its good PRS hearablity and available assistance data.

Observation 5: UE can continue PRS measurement in all target cells/TRPs for HO.

However, the ongoing RSTD measurement during HO may be dropped off if the PRS measurement duration is longer than that of HO processing time. The measurement performance will be impacted by HO interruption also. Both of them will increase the overall PRS measurement delay.

Therefore we can propose that:
Proposal 5:  The total measurement delay when serving cell changed (e.g. HO) can defined as:

TRSTD_HO, NR=TRSTD_withoutHO, NR+ TPRS ⋅k+THO  ms

Where

k is the number of times the intra/inter-frequency handover occurs during TRSTD_HO, NR.

THO is the time during which the intra/inter-frequency RSTD measurement may not be possible due to intra/inter-frequency handover.

The maximum ‘THO’ is the sum of the processing delay of intra/inter-frequency HO command and additional delay required by the UE to be able to initiate the measurement on the new serving cell. The processing delay requirements for RRC procedure is typically 15ms according to TS38.331. The HO delay can be up to 30 ms (including 10 ms to account for frame uncertainity).

# Measurement Report Mapping

In the last RAN4 meeting, there were some agreements on RSTD reporting mapping below [1].

- PRS-RSTD report mapping table(s) are FFS. - RAN4 to continue discussion on PRS-RSTD report mapping table(s) in the next meeting considering e.g. the following aspects: - Uniform vs. non-uniform granularity - Number of report mapping tables - Parameter k (T=Tc2k) where k is a configuration parameter - Whether UE always can follow signaled k or may need to adjust k - FFS: whether UE can report its capability of k to NW - Min/max values of k, and whether they depend on FR1/FR2 and center/edge mapping - Note: other relevant aspects are not precluded

Hereby, we provide further considerations on several open issues above in this section.

# RSTD report range in FR2

Obviously, in comparison with other factors the report range is more essential to determine the report signaling overhead. In order to save the report signaling overhead, it is more efficient to reduce the report range as possible.

In principle, RSTD report range can depend on the cell coverage closely. Thus, for FR2, the positioning coverage can be smaller than FR1, due to the shorter network coverage of FR2 in practical deployments .

Proposal 6: The reporting range of PRS RSTD measurement in FR2 can be (± 125 us).

# Number of report mapping tables

In the previous meeting, there are several options were proposed for the necessary number of report mapping tables as below.

Candidate options: - Option 1. One table per SCS per FR - Option 2. One table per FR - Option 3. One table - Option 4. One table per “k” - Option 5: LTE approach – one table for coarse granularity (for this, even the LTE table can be reused) and one table for fine granularity (based on k for NR) Other options are not precluded.

In our view, the main motivation with different mapping table for UE RSTD measurement is simply save the signaling overhead. However, it shall be noted that it is unnecessary to distinguish them with too many parameters because

- The signaling overhead saving among each defined table will be very vanished (e.g. about 1 bit reduction) especially for RRC signaling overhead.

- The more extra standardization efforts are requested.

Observation 6: The gain of signaling overhead reduction from the many report mapping tables is not obvious especially with the significant extra standardization efforts.

More importantly, the drawback of the report table depends on the numerology (e.g. SCS) is the restriction on UE flexibility to improve the positioning accuracy. For an example, with higher over sampling UE could obtain the better RSTD measurement accuracy than that of define in the report table depend on the SCS. And regarding to the UE report its measurement results by the table in which much finer step than its accuracy requirements shall be fine.

Observation 7: The drawback of the report table depends on UE numerology is the restriction on UE flexibility to improve the positioning accuracy.

On the other hand, as we mentioned above, the reporting range for FR1 and FR2 can be quite diverse, which is more fundamental and efficient to save the signaling overhead. And as agreed in RAN1 the reporting granularity for the UE/gNB timing measurements (DL RSTD, the UE Rx-Tx time difference, UL RTOA, gNB Rx-Tx time difference) can be defined as T=Tc2k, the report mapping table can be differentiated by the parameter “k” also.

Proposal 7: The report mapping tables for NR RSTD measurement can be one table per FR and per “k”.

Alternatively, regarding to the tightened timeline for RAN4 works remained, the other more straightforward way to define the table of RSTD measurement is also given below.

Proposal 7a: The report mapping tables for NR RSTD measurement can be one table per FR.

# Minimum granularity in report mapping table

In the last meeting, the candidate options for the minimum granularity in reporting table includes:

- Option 1. Tc/2 (Huawei/Intel) - Option 2. Tc (CATT) - Option 3. 8Tc for FR1 and 2Tc for FR2 (Apple) - Option 4. 16Tc (k=4) for FR1 and Tc (k=0) for FR2 for the center range (Ericsson) - Option 5. 4Tc for FR1 and Tc for FR2 (Qualcomm) - Option 6. Other options not precluded.

In previous both RAN1 and RAN4 meeting, there were some discussions on RSTD measurement reporting resolution and range. Some agreements achieved are self-contained below.

RAN1#99 Agreement: - The reporting granularity for the gNB angle measurements (AoA, ZOA) is defined as 0.1 degree. - RAN1 assumes that the details of the reporting granularity and ranges for the gNB angle measurements (AoA, ZoA) will be determined by RAN4. - The reporting granularity for the set of angles α (bearing angle), β (downtilt angle) and γ (slant angle) for the translation of the GCS to LCS is defined as 0.1 degree. - RAN1 assumes that the reporting granularity, the range of values and mapping for DL PRS RSRP and UL SRS RSRP measurements will be determined by RAN4 - Include above information in an LS to RAN2/RAN3/RAN4 Agreement: - The reporting granularity for the UE/gNB timing measurements (DL RSTD, the UE Rx-Tx time difference, UL RTOA, gNB Rx-Tx time difference) is defined as T=Tc2k, where k is a configuration parameter with a minimum value of at most 0. - Note: RAN4 can determine if -1 can be a minimum value - RAN1 assumes that the details of the reporting granularity and ranges for the UE/gNB timing measurements (DL RTSD, the UE Rx-Tx time difference, UL RTOA, gNB Rx-Tx time difference) will be determined by RAN4, including the potential relation of the parameter k to DL PRS bandwidth. - Include above information in an LS to RAN2/RAN3/RAN4 Agreement: The quality metrics for both DL and UL timing based measurements have a resolution of {0.1m, 1m, 10m, 30m} and are reported with a 5-bit value field

R4#93 Agreement - RSTD reporting range - The Max/Min reported values are same as in LTE for FR1 and FR2

Thus in this RAN4 meeting, the main remain issue for RSTD reporting resolution is to further discuss the minimum value of reporting resolution below

T=Tc2k

And according to RAN1’ agreements above, the quality metrics for both DL and UL timing based measurements have a the minimum resolution of “0.1m” for the accurate positioning. If the error due to reporting granularity is be much less than resolution due to PRS BW, i.e. 10ns in FR1 and 2.5ns in FR2, the reporting granularity which is 10 times less than bandwidth resolution (i.e. 1ns and 0.25ns respectively) is a reasonable assumption for NR positioning to avoid degradation due to granularity error. Therefore, in NR specification, the minimum reporting resolution shall be as 2Tc and Tc/2 for FR1 and FR2 respectively.

Proposal 8: PRS RSTD minimum reporting granularity can 2Tc and Tc/2 for FR1 and FR2 respectively.

# Differential reporting for RSTD

As shown in Figure 5 below, the differential RSTD can be defined as:

diff_RSTD2 = RSTD2-RSTD1 = Trxsubframe_cell2 - Trxsubframe_cellref – (Trxsubframe_cell1 - Trxsubframe_cellref)

= Trxsubframe_cell2 - Trxsubframe_cell1

Particularly it shall be noted that the differential RSTD report may be valid for the measurements being performed on the same pair of TRPs because the ToA results obtained by the UE through different beam measurements (e.g. ToA1-1 and ToA1-2 in Figure 5) can be close.

Figure 5. Differential RSTD reporting

Observation 8: Differential RSTD report can be applicable for the measurements on the same TRP pair.

Meanwhile, it was agreed in RAN2 [11] that:

Confirm (same as current running CR) when a UE is configured to report multiple DL PRS RSTD, PRS RSRP, RxTX measurements with each measurement between a different pair of DL PRS resources or DL PRS resource sets, and those multiple measurements being performed on the same pair of TRPs, the UE reports one full measurement results and additional delta measurement(s).

That is for the differential RSTD measurement, the UE can report some of measurements results with the offset with the absolute report results. In case of LOS propagation condition, such delta report can be independent with the time offset due to the multipath channel since the multiple beams can experience similar propagation path. But in NLOS, the total offset between two ToAs (e.g. ToA1-1 and ToA1-2) shall include both cell phase offset between two beams and the multiple path delay of the receiving beam pair. Regarding the worst case, the maximum report range of the differential RSTD of same TRP pair can include:

- Maximum time error among the beams within a same gNB/TRP and

- Time offset between the different RX-TX beam pair

Observation 9: Differential RSTD report range can rely on

- Time offset between the beams in a same gNB/TRP

- the time offset between the beams of a same a gNB/TRP

Similarly, the maximum time offset within the two beams can be less than the BS TAE error requirements (e.g.65ns for a same carrier) in [12].

For the time offset between the different Rx-Tx beams, as one of example in Figure 6, it can be as “2Tabs-Tabs = Tabs” assuming the absolute ToA is “Tabs”. Therefore, the maximum RSTD report range with differential reporting can be high as that of absolute reporting. Since the coverage of gNB1 would be reduced as half of one with single beam only, we can conclude the range of the time offset between the beams of a same TRP (denoted as Tabs) shall be (± 250us) and (± 62.5us) for FR1 and FR2 respectively.

Figure 6. Maximum range of differential RSTD report

In conclusion, we can conclude that:

Proposal 9: Differential PRS RSTD report range is ± (250us+65ns) and ±(62.5us+65ns) for FR1 and FR2 respectively.

# Conclusion

In this contribution, some considerations on NR PRS RSTD measurement requirements are provided and the following observations and proposals can be drawn:

Observation 1: RSTD measurement accuracy will be less than [-40.69ns, 89.51ns] under TDL-C propagation channel.

Proposal 1: SINR side conditions for PRS-RSTD in FR2 can be

- PRS Es/Iot = -6 dB for reference cell and

- PRS Es/Iot = -13 dB for neighbor cells

Observation 2: It is more straightforward to define of intra/inter-frequency PRS measurement with similar approaches of LTE and NR SSB measurement. That is the intra/inter PRS-RSTD measurement can be defined independently with UE active BWP.

Observation 3: UE requirements on the number of measured frequency layers for PRS measurement s in RAN4 RRM can be also defined by the number of measurement positioning frequency layer as RAN1 agreed.

Proposal 2: Regarding to the consistent definition for NR measurement as simple as possible, the intra/inter frequency PRS measurement can be defined as:

|  | Definition | Need gap or Not |
| --- | --- | --- |
| intra-frequency | DL PRS resources of the measured neighbor cells/TRPs and the serving/reference cell/TRP have - the same SCS and CP type - the same centre frequency - the same point-A - same configured PRS BW | NO: If the central frequency of reference cell is same as that of the serving cell. |
|  |  | Yes: If the central frequency of reference cell is different with that of the serving cell. |
| inter-frequency | DL PRS resources of the measured neighbor cells/TRPs and the serving/reference cell/TRP is different in any of - SCS and CP type - centre frequency - point-A - configured PRS BW | Yes |

Observation 4: PRS RSTD measurement delay requirement can be determined by:

- the periodicity of DL PRS allocation

- the total number of DL PRS resource which UE can process

Proposal 3: The allowed RSTD measurement delay would be multiplied by the number of PRS resource that UE can measure as defined for LTE RSTD measurement.

TRSTD IntraFreqFDD, NR=TPRS ⋅(M-1)+Δ  ms

Where in, TPRS is the periodicity of DL PRS allocation in slots configured per DL PRS resource,

M is the number of PRS positioning resource for all cells (e.g. n=16 in [7]) to be detected and measured,

Δ  is the measurement time for a single PRS positioning resource which includes the sampling time and the processing time.

Proposal 4: The number of PRS positioning resource to be detected and measured can be FFS waiting RAN1 agreements.

Observation 5: UE can continue PRS measurement in all target cells/TRPs for HO.

Proposal 5:  The total measurement delay when serving cell changed (e.g. HO) can defined as:

TRSTD_HO, NR=TRSTD_withoutHO, NR+ TPRS ⋅k+THO  ms

Where

k is the number of times the intra/inter-frequency handover occurs during TRSTD_HO, NR.

THO is the time during which the intra/inter-frequency RSTD measurement may not be possible due to intra/inter-frequency handover.

Proposal 6: The reporting range of PRS RSTD measurement in FR2 can be (± 125 us).

Observation 6: The benefits of signaling overhead reduction from the too many report mapping tables is not obvious especially with the significant extra standardization efforts.

Observation 7: The drawback of the report table depends on UE numerology is the restriction on UE flexibility to improve the positioning accuracy.

Proposal 7: The report mapping tables for NR RSTD measurement can be one table per FR and per “k”.

Proposal 7a: The report mapping tables for NR RSTD measurement can be one table per FR.

Proposal 8: RSTD minimum reporting granularity can 2Tc and Tc/2 for FR1 and FR2 respectively.

Observation 8: Differential RSTD report can be applicable for the measurements on the same TRP pair.

Observation 9: Differential RSTD report range can rely on

- Time offset between the beams in a same gNB/TRP

- the time offset between the beams of a same a gNB/TRP

Proposal 9: Differential PRS RSTD report range is ± (250us+65ns) and ±(62.5us+65ns) for FR1 and FR2 respectively.

# References

- R4-2002328 “WF on NR Positioning UE requirements”

- R4-2000389,” Further Discussion on NR PRS RSTD Requirements for UE”

- R4-1707424, “WF on NR RRM measurement”

- R4-1913487,  “System-level evaluations for PRS-RSTD and PRS-RSRP”

- R4-1913262, “System level Simulation Results for PRS-RSTD and PRS-RSRP Core Requirement”

- R4-1914957, “Further simulation results for NR positioning”

- R4-2002287, “Updated link level simulation assumption for RSTD and RSRP”

- 3GPP TS36.300 v16.0.0, “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2”

- 3GPP TS 38.133 v16.0.0: "NR: Requirements for support of radio resource management "

- 3GPP TS 36.133 v15.7.0: "E-UTRA: Requirements for support of radio resource management "

- R2-2002245, “LS to RAN4 on measurement range and granularity”

- 3GPP TS38.104 v16.0.0, “NR; Base Station (BS) radio transmission and reception”