3GPP TSG-RAN WG4 Meeting # 94-e-Bis                                 R4-2003239

# Electronic Meeting, 20 – 30 April, 2020

Source:	LG Electronics

Title:	TP on NR V2X A-MPR (NS_33) and updated simulation results

Agenda item:	6.4.4.1

Document for:	Approval

- Introduction

In this contribution, we provide the updated simulation results for A-MPR for NS_33 in NR V2X at n47.

- A-MPR and Simulation results

A-MPR for NS_33

The simulation results are based on simulation assumptions which were agreed in last meeting [1]. For fc = 5860MHz, A-MPR should be defined depending on the LCRB and RB start, and for other frequencies, A-MPR could be defined by inner and outer RB allocation for 15kHz and 30kHz SCS as shown in Fig.1~6 and Table 3. Based on simulation results, we propose A-MPR for NR V2X as Table 1 and Table 2.

- Proposal : Update A-MPR for NS_33 with Table 1 and Table 2

Table 1 A-MPR for NS_33 at fc = 5860MHz

| Subcarrier spacing (kHz) | Carrier frequency(MHz) | Resources Blocks (NRB) Note1 | Start Resource Block | A-MPR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 15 | 5860 | 10 | 0 | ≤ 20 |  |  |
|  |  |  | ≥ 1 and ≤ 3 | ≤ 19 |  |  |
|  |  | 10 and 15 | ≥ 26 and ≤ 38 | ≤ 6 |  |  |
|  |  | ≥ 10 and ≤ 20 | ≥ 12 and ≤ 14 | ≤ 11 |  |  |
|  |  |  | ≥ 15 and ≤ 19 | ≤ 9.5 |  |  |
|  |  |  | ≥ 20 and ≤ 25 | ≤ 7.5 |  |  |
|  |  | ≥ 10 and ≤ 30 | 4 and 5 | ≤ 16 |  |  |
|  |  |  | 6 and 7 | ≤ 14.5 |  |  |
|  |  |  | ≥ 8 and ≤ 11 | ≤ 13.5 |  |  |
|  |  | ≥ 20 and ≤ 30 | ≥ 0 and ≤ 3 | ≤ 19 |  |  |
|  |  | 25 and 30 | ≥ 16 and ≤ 21 | ≤ 9.5 |  |  |
|  |  |  | ≥ 22 and ≤ 27 | ≤ 7.5 |  |  |
|  |  | ≥ 25 and ≤ 40 | ≥ 12 and ≤ 15 | ≤ 12 |  |  |
|  |  | 40 and 45 | 0 and 1 | ≤ 19 |  |  |
|  |  |  | 2 and 3 | ≤ 16 |  |  |
|  |  |  | 4 and 5 | ≤ 14.5 |  |  |
|  |  |  | ≥ 6 and ≤ 11 | ≤ 13.5 |  |  |
|  |  | 50 | 0 | ≤ 16 |  |  |
| 30 | 5860 | 10 | ≥ 11 and ≤ 14 | ≤ 9 |  |  |
|  |  | 10 and 15 | 0 | ≤ 19 |  |  |
|  |  |  | 1 and 2 | ≤ 17.5 |  |  |
|  |  |  | 3 | ≤ 16 |  |  |
|  |  |  | 4 | ≤ 14.5 |  |  |
|  |  |  | 5 and 6 | ≤ 13.5 |  |  |
|  |  |  | 7 and 8 | ≤ 12 |  |  |
|  |  |  | 9 and 10 | ≤ 10.5 |  |  |
|  |  | 20 | 0 | ≤ 17.5 |  |  |
|  |  |  | 1 and 2 | ≤ 16 |  |  |
|  |  |  | 3 and 4 | ≤ 14.5 |  |  |
| 60 | 5860 | 10 | 0 | ≤ 17.5 |  |  |
|  |  | 10 | 1 | ≤ 16 |  |  |
| NOTE 1: The resource blocks are limited by LCRB. LCRB = {10,15,20,25,30,40,45,50}. |  |  |  |  |  |  |

Table 2 A-MPR for NS_33 at fc = [5870,5880,5890,5900,5910,5920]MHz

| Subcarrier spacing (kHz) | Carrier frequency(MHz) | RB allocations | QPSK | 16QAM | 64QAM | 256QAM |
| --- | --- | --- | --- | --- | --- | --- |
| 15 and 30 | 5870, 5880, 5890, 5900, 5910, 5920 | Inner | ≤ 2.5 |  | ≤ 3 | ≤ 5.5 |
|  |  | Outer | ≤ 3.5 |  | ≤ 4 | ≤ 5.5 |
| 60 | 5870, 5910, 5920 |  | ≤ 5 |  |  | ≤ 5.5 |
|  | 5880, 5890, 5900 |  | ≤ 3.5 |  |  | ≤ 5.5 |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1 [2].

RBStart,Low = max(1, floor(LCRB /3))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low ≤ RBStart ≤ RBStart,High , and

LCRB ≤ NRB

where ceil(x) is the smallest integer greater than or equal to x.

Simulation results

Figure 1 A-MPR for NR V2X with QPSK and 15kHz SCS

Figure 2 A-MPR for NR V2X with 64QAM and 15kHz SCS

Figure 3 A-MPR for NR V2X with 256QAM and 15kHz SCS

Figure 4 A-MPR for NR V2X with QPSK and 30kHz SCS

Figure 5 4 A-MPR for NR V2X with 64QAM and 30kHz SCS

Figure 6 A-MPR for NR V2X with 256QAM and 30kHz SCS

Table 3 A-MPR for NR V2X with 60kHz SCS

| Modulation order | Resource Blocks (NRB) | Start Resource Block | Carrier Frequency (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5860 | 5870 | 5880 | 5890 | 5900 | 5910 | 5920 |
| QPSK | 10 | 0 | 17.1 | 4.4 | 2.7 | 2.7 | 2.7 | 4.0 | 4.0 |
|  | 10 | 1 | 15.6 | 4.0 | 2.6 | 2.6 | 2.6 | 4.4 | 4.4 |
| 16QAM | 10 | 0 | 17.0 | 4.7 | 2.7 | 2.6 | 2.7 | 4.0 | 4.0 |
|  | 10 | 1 | 15.6 | 4.0 | 2.6 | 2.6 | 2.7 | 4.4 | 4.5 |
| 64QAM | 10 | 0 | 17.2 | 4.4 | 2.7 | 2.7 | 2.7 | 4.0 | 3.8 |
|  | 10 | 1 | 15.4 | 4.1 | 2.6 | 2.6 | 2.6 | 4.4 | 4.4 |
| 256QAM | 10 | 0 | 17.1 | 4.8 | 4.7 | 4.7 | 4.7 | 4.5 | 4.6 |
|  | 10 | 1 | 15.6 | 4.5 | 4.6 | 4.7 | 4.6 | 4.6 | 4.6 |

- Conclusion

In this contribution, we provide the updated simulation results for A-MPR for NS_3 in NR V2X at n47 and propose

- Proposal : Update A-MPR for NS_33 with Table 1 and Table 2

Appendix

------------------- Start of Text Proposal for TR38.886 --------------------

# 8	Transmitter characteristics

## 8.1	NR V2X UE Tx requirements in FR1

------------------      Omit unchanged parts     --------------------

### 8.1.3	UE maximum output power with additional requirements

To comply the EN 302 571 standard  emission mask, RAN4 need to derive A-MPR requirements considering with the additional A-SEM and A-SE requirements in EN 302 571 standard as shown in Table 8.1.3-1 and Table 8.1.3-2.

NR V2X UE shall satisfy the additional SEM and additional SE requirements when NS_33 is configured from pre-configured radio parameters or the cell and the indication from upper layers has indicated was informed.

Table 8.1.3-1: Additional SEM requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm EIRP)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz] | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5] | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0] | 100 kHz |

NOTE 1:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE 2:	Additional SEM for V2X overrides any other requirements in frequency range 5855-5950MHz.

NOTE 3:	The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex G in TS38.101-1.

Table 8.1.3-2: Additional SE requirements

| n47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, 68, 72, 73 NR band n77, n78 , n79 | FDL_low | - | FDL_high | -50 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Frequency range | 5925 | - | 5950 | -30 EIRP | 1 | 38, 40, 43 |
|  | Frequency range | 5815 | - | 5855 | -30 EIRP | 1 | 38, 43 |
| NOTE 38: Applicable when NS_33 or NS_34 is configured by the pre-configured radio parameters. NOTE 40: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 43: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I. |  |  |  |  |  |  |  |

Also, FCC had regulatory requirements for 40MHz in ITS spectrum as shown in Table 8.1.3-3

Table 8.1.3-3: Additional SEM requirements for 40MHz channel bandwidth (fc =5885 MHz)

| ΔfOOB (MHz) | Emission Limit (dBm) | Measurement Bandwidth |
| --- | --- | --- |
| 0 - 2 | -32 | 100kHz |
| 2-10 | -36 | 100kHz |
| 10-20 | -38 | 100kHz |
| 20-40 | -43 | 100kHz |
| 40 - 100 | -50 | 100kHz |

To comply the regional regulatory requirements, RAN4 specifiy the A-MPR requirements according to different network signalling by pre-configured radio parameters or the cell and the indication from upper layers has indicated to NR V2X UE.

The allowed additional Maximum Power Reduction (A-MPR) for the maximum output power due to higher order modulation and transmit bandwidth configuration (resource blocks) will be specified in TS38.101-1 as below.

Table 8.1.3-4: Additional Maximum Power Reduction (A-MPR) for NR V2X

| Network Signalling value | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_33 | Table 8.1.3-1 (A-SEM) Table 8.1.3-2 (A-SE) | n47 | 10 | Table 8.1.3-5 Table 8.1.3-6 |  |
| NS_48 | Table 8.1.3-3 (A-SEM) | n47 | 40 | Table 8.1.3-7 |  |

Table 8.1.3-5: A-MPR for NS_33 at fc = 5860MHz

| Subcarrier spacing (kHz) | Carrier frequency(MHz) | Resources Blocks (NRB) Note1 | Start Resource Block | A-MPR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 15 | 5860 | 10 | 0 | ≤ 20 |  |  |
|  |  |  | ≥ 1 and ≤ 3 | ≤ 19 |  |  |
|  |  | 10 and 15 | ≥ 26 and ≤ 38 | ≤ 6 |  |  |
|  |  | ≥ 10 and ≤ 20 | ≥ 12 and ≤ 14 | ≤ 11 |  |  |
|  |  |  | ≥ 15 and ≤ 19 | ≤ 9.5 |  |  |
|  |  |  | ≥ 20 and ≤ 25 | ≤ 7.5 |  |  |
|  |  | ≥ 10 and ≤ 30 | 4 and 5 | ≤ 16 |  |  |
|  |  |  | 6 and 7 | ≤ 14.5 |  |  |
|  |  |  | ≥ 8 and ≤ 11 | ≤ 13.5 |  |  |
|  |  | ≥ 20 and ≤ 30 | ≥ 0 and ≤ 3 | ≤ 19 |  |  |
|  |  | 25 and 30 | ≥ 16 and ≤ 21 | ≤ 9.5 |  |  |
|  |  |  | ≥ 22 and ≤ 27 | ≤ 7.5 |  |  |
|  |  | ≥ 25 and ≤ 40 | ≥ 12 and ≤ 15 | ≤ 12 |  |  |
|  |  | 40 and 45 | 0 and 1 | ≤ 19 |  |  |
|  |  |  | 2 and 3 | ≤ 16 |  |  |
|  |  |  | 4 and 5 | ≤ 14.5 |  |  |
|  |  |  | ≥ 6 and ≤ 11 | ≤ 13.5 |  |  |
|  |  | 50 | 0 | ≤ 16 |  |  |
| 30 | 5860 | 10 | ≥ 11 and ≤ 14 | ≤ 9 |  |  |
|  |  | 10 and 15 | 0 | ≤ 19 |  |  |
|  |  |  | 1 and 2 | ≤ 17.5 |  |  |
|  |  |  | 3 | ≤ 16 |  |  |
|  |  |  | 4 | ≤ 14.5 |  |  |
|  |  |  | 5 and 6 | ≤ 13.5 |  |  |
|  |  |  | 7 and 8 | ≤ 12 |  |  |
|  |  |  | 9 and 10 | ≤ 10.5 |  |  |
|  |  | 20 | 0 | ≤ 17.5 |  |  |
|  |  |  | 1 and 2 | ≤ 16 |  |  |
|  |  |  | 3 and 4 | ≤ 14.5 |  |  |
| 60 | 5860 | 10 | 0 | ≤ 17.5 |  |  |
|  |  | 10 | 1 | ≤ 16 |  |  |
| NOTE 1: The resource blocks are limited by LCRB. LCRB = {10,15,20,25,30,40,45,50}. |  |  |  |  |  |  |

Table 8.1.3-6: A-MPR for NS_33 at fc = [5870,5880,5890,5900,5910,5920]MHz

| Subcarrier spacing (kHz) | Carrier frequency(MHz) | RB allocations | QPSK | 16QAM | 64QAM | 256QAM |
| --- | --- | --- | --- | --- | --- | --- |
| 15 and 30 | 5870, 5880, 5890, 5900, 5910, 5920 | Inner | ≤ 2.5 |  | ≤ 3 | ≤ 5.5 |
|  |  | Outer | ≤ 3.5 |  | ≤ 4 | ≤ 5.5 |
| 60 | 5870, 5910, 5920 |  | ≤ 5 |  |  | ≤ 5.5 |
|  | 5880, 5890, 5900 |  | ≤ 3.5 |  |  | ≤ 5.5 |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1 [2].

RBStart,Low = max(1, floor(LCRB /3))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low ≤ RBStart ≤ RBStart,High , and

LCRB ≤ NRB

where ceil(x) is the smallest integer greater than or equal to x.

Table 8.1.3-7: A-MPR for NS_48

| Carrier frequency(MHz) | Modulation | A-MPR(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocation |
| 5885 | QPSK | ≤ (9.5 + ΔNote1) | [≤ 8.0] | [≤ 3.5] |
|  | 16QAM |  | [≤ 8.0] | [≤ 3.5] |
|  | 64QAM |  | [≤ 8.5] | [≤ 4.5] |
|  | 256QAM |  | [≤ 8.5] | [≤ 6.0] |
| Note1: Δ is 0, 3, and 5 for 60kHz, 30kHz, and 15kHz SCS, respectively. |  |  |  |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1 [3].

RBStart,Low = max(1, floor(NRB /3.5))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low ≤ RBStart ≤ RBStart,High , and

LCRB ≤ ceil(NRB/3.5)

where ceil(x) is the smallest integer greater than or equal to x.

An Edge RB allocation is the one for which the RB(s) is (are) allocated at the lowermost or uppermost edge of the channel with LCRB ≤ floor(NRB*0.2) RBs.

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation or Edge RB allocation.

------------------ End of Text Proposal for TR38.886 --------------------

Reference

[1] R4-2002783, “A-MPR simulation assumptions and initial results for NR V2X UE at n47,” LG Electronics

[2] TS 38.101-1.