3GPP TSG-RAN WG4 Meeting #94-e-bis		R4-2003347

Electronic meeting, 20 April – 30 April 2020

Source:	Sony

Title:	Enhanced reporting for beam correspondence in poor SNR condition

Agenda item:	6.14.1.2

Document for:	Approval

- Background

In RAN4 #94-e, beam correspondence (BC) enhancement was discussed [1] and a WF [2] was issued though not approved in the end. There were no updates to the WID [3] regarding BC in last plenary meeting (RAN #87-e).

In this contribution we will discuss Topic 4: Additional beam correspondence enhancements and Topic 5: Beam correspondence capability aspects.

Topic 1: Beam correspondence based on SSB, Topic 2: Beam correspondence based on CSI-RS and Topic 3: Initial access beam correspondence will be discussed in other documents (refer topic description to [2]).

# Discussion

The motivation behind the enhancement on the measurement reporting has been thoroughly discussed in [4] and [5]. Here, we briefly remind again the technical issues with Rel-15 UE beam correspondence requirement.

## Beam correspondence in poor SNR condition

In a real network, in a poor SNR and/or poor SINR scenario the probability that the UE may make RSRP estimation errors increases, and thus the UE may fail to select an optimal uplink beam autonomously, regardless of the UE BC capability. The simulation in [5] shows that the standard deviation of the RSRP estimation diverges quickly when the SNR goes below 0 dB.  Therefore, a UE may anyway lose the BC and, thus, it may need network-assisted uplink beam sweeping even if it has signaled UE BC capability bit to 1. On the other hand, the SNR and/or SINR may also be very high, in a real network scenario, and a UE that has set its UE BC capability bit to 0 may be capable of selecting an optimal uplink beam autonomously under such a condition.

Therefore, the beam correspondence can be seen as a dynamic capability of a UE in real networks: Whether the UE can autonomously select a “good enough” uplink beam does not depend on the UE BC capability bit but rather on the SNR condition and DL reference signal configuration. The network cannot know whether the UE can perform the beam correspondence with Rel-15 BC capability.

Observation 1:	Beam correspondence is a dynamic capability in real networks that depends on the network SNR and DL reference signal configuration. It is not enough for the network to only know UE’s beam correspondence capability, regardless of whether the BC UE capability bit is set to 0 or to 1.

The beam correspondence tolerance has been introduced in Rel-15 in order to accommodate the lower capability of some UEs to select the uplink beam autonomously. However, in light of the discussion above, it is questionable whether it is useful for the network to know such a UE capability. Based on the discussion above and detailed analysis in [4] and [5], regardless of the UE capability bit for beam correspondence 1 or 0, a UE will eventually lose the beam correspondence when the DL SNR/SINR becomes low or the configuration of DL reference signal is not optimized.

Therefore, for Rel-16 BC requirement, we suggest removing the bit 1 or 0 for beam correspondence: All UEs that support Rel-16 BC shall also meet the spherical coverage requirement without tolerance.

Proposal 1: 	If a UE supports Rel-16 BC and the UE is Rel-15 BC bit-0 UE, it is an invalid scenario and should not be allowed.

## Rel-16 enhancement to ensure the UE performs beam correspondence

In order to ensure the UE performs beam correspondence based on DL reference signal configured by the network, it is, therefore, necessary for the network to configure a dynamic reporting mechanism to gain knowledge of the UE side SNR/SINR and any other BC related information, to accurately improve beam correspondence performance.

Observation 2: 	It is necessary to have dynamic reporting/signaling from UE in order to ensure the UE performs beam correspondence based on DL reference signals configured by the network.

L1-RSRP and L1-SINR reporting is already available in Rel-16 and possibly well-suited for this purpose. Some benefits of L1-SINR reporting in the beam selection are elaborated in [6], and two possible ways of how network can use the L1-SINR are further described below

- The UE can indicate to the gNB that the DL SINR is too low to perform beam correspondence, and the network can in response configure more DL resource elements to the UE to improve its RSRP estimation accuracy.

- Another alternative would be the UE, in extremely poor SINR condition, indicates the gNB that it cannot perform the BC, and it may require uplink beam sweeping assistance from the gNB. As an alternative, the gNB can also decide to terminate the BC operation and start the UL beam sweeping based on the reported SINR.

To our understanding, such an enhancement on the reporting/signaling of BC is necessary to meet the objective of the Rel-16 beam correspondence enhancement WID, and it is also essential for the network performance. Though the limited time frame in Rel-16 may block introduction of new signaling at current stage, we think the technical feasibility of potential enhancements should still be further discussed in RAN4, which can pave the road to introducing beam correspondence in future releases.

Proposal 2: 	RAN4 continue discuss the technic feasibility of potential enhancements to beam correspondence in Rel-16 timeframe.

# Conclusion

In this contribution we have discussed the importance of having enhancement measurement report on BC in order to meet the target of Rel-16 BC enhancement. The following observations and proposals have been given:

Observation 1:	Beam correspondence is a dynamic capability in real networks that depends on the network SNR and DL reference signal configuration. It is not enough for the network to only know UE’s beam correspondence capability, regardless of whether the BC UE capability bit is set to 0 or to 1.

Observation 2: 	It is necessary to have dynamic reporting/signaling from UE in order to ensure the UE performs beam correspondence based on DL reference signals configured by the network.

Proposal 1: 	If a UE supports Rel-16 BC and the UE is Rel-15 BC bit-0 UE, it is an invalid scenario and should not be allowed.

Proposal 2: 	RAN4 continue discuss the technic feasibility of potential enhancements to beam correspondence in Rel-16 timeframe.

# References

- R4-2002890	“Email discussion summary for RAN4#94e_#21_NR_RF_FR2_req_enh_Part_2”, Moderator (Apple)

- R4-2002822	“WF on remaining issues with Rel-16 beam correspondence”,

- RP-200246	“Revised WID on NR RF Requirement Enhancements for FR2”, Nokia, Nokia Shanghai Bell

- R4-1913521	“Discussion on more enhancement to beam correspondence in Rel-16”, Samsung

- R4-1909225    “Beam Correspondence in poor SNR conditions”, Sony

- R1-1909202	“Discussion on multi-beam enhancement”, NTT DOCOMO, INC.