3GPP TSG-RAN WG4 Meeting #94-e-Bis	R4-2003377

# Electronic Meeting, 20 April. – 30 April, 2020

Title:	LS on RRM relaxation for higher priority inter frequency layer

Response to:	-

Release:	Release 16

Work Item:	NR_UE_pow_sav

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact Person:

Name: Xusheng Wei, vivo

E-mail Address:	xusheng.wei@vivo.com

1	Overall description

During RAN4’s discussion on RRM relaxation for higher priority inter-frequency layer for UE power saving, RAN4 has the following agreements:

- When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, no relaxation on the current measurement requirement is defined for inter-frequency measurement with higher priority

- When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, identical measurement requirements apply for all inter-frequency layers, no matter their priority; the measurement requirements could be either relaxed measurement requirements or normal measurement requirements

Based on aforementioned conclusions RAN4 conclude that the highPriorityMeasRelax indication is not required for RRM measurement relaxation on the higher priority inter-frequency layers.

RAN4 kindly asks RAN2 to take the aforementioned RAN4’s conclusions into account for their further work.

2	Actions

To RAN2 group.

ACTION: RAN4 kindly asks RAN2 to take the aforementioned RAN4’s conclusions into account for their further work.

3	Dates of next TSG RAN4 meetings

RAN4#95-e                     	  25.05 – 05.06, 2020, Online

RAN4 #96			  24.08 – 28.08, 2020, Toulouse, FR