3GPP TSG RAN WG4 Meeting #94bis-e	R4-2003451

Electronic Meeting, 20 – 30 Apr., 2020

Agenda item:	8.4.2

Source:	SoftBank Corp.

Title:	TP for TR 37.716-21-11: EN-DC_1-11_n3

Document for:	Approval

1	Introduction

EN-DC of 2B LTE and 1B NR of DC_1-11_n3 was approved in RAN#87 [1]. This TP is to capture the basic aspects for the EN-DC. REFSENS exception results for IMD are needed to complete the combo.

2	Combination proposed

Combinations proposed in [1] are as below.

Table 1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-11_n3 | CA_1-11 | n3 | DC_1_n3 |

Table 2: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-11A_n3A | DC_1A_n3A DC_11A_n3A | CA_1A-11A | n3A |

3.  Co-existence

When Uplink EN-DC configuration is DC_1A_n3A, (1) IMD3 of (B1 - n3) will fall into Rx band of Band 11. When Uplink EN-DC configuration is DC_11A_n3A, (2) IMD3 of (B11 - n3) will fall into Rx band of Band 1.

Thus, it is proposed to evaluate (1) and (2).

4.  Delta Tib/Rib

The delta Tib/Rib of subset EN-DC/CA are as follows. EN-DC values are picked.

Table 3: ΔTIB of constituent 2 band combo.

| ΔTIB | B1+B11 | B1+n3 | B11+n3 | Proposed Value |
| --- | --- | --- | --- | --- |
| B1 | 0.3 | 0.3 |  | 0.3 |
| B11 | 0.3 |  | 0.8 | 0.8 |
| n3 |  | 0.3 | 0.9 | 0.9 |

Table 4: ΔRIB of constituent 2 band combo.

| ΔRIB | B1+B11 | B1+n3 | B11+n3 | Proposed Value |
| --- | --- | --- | --- | --- |
| B1 | 0 | 0 |  | 0 |
| B11 | 0 |  | 0.3 | 0.3 |
| n3 |  | 0 | 0.5 | 0.5 |

Then the proposal for this 2LTE-1NR EN-DC is:

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n3 | 1 | 0.3 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n3 | 1 | 0 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |

5.  REFSENS

As mentioned in section 3, REFSENS exceptions due to IMD3 are expected. The following values are proposed.

Table 5: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_1A-11A_n3A | 1 | TBD | TBD | TBD | TBD | N/A | FDD | N/A | Yes |
|  | n3 | TBD | TBD | TBD | TBD | N/A | TDD | N/A | Yes |
|  | 11 | TBD | TBD | TBD | TBD | TBD | FDD | IMD3 |  |
| DC_1A-11A_n3A | 11 | TBD | TBD | TBD | TBD | N/A | FDD | N/A |  |
|  | n3 | TBD | TBD | TBD | TBD | N/A | TDD | N/A | Yes |
|  | 1 | TBD | TBD | TBD | TBD | TBD | FDD | IMD3 | Yes |

6.  Conclusion

This paper proposes study results of EN-DC 1-11_n3. REFSENS exception results for IMD are needed to complete the combo. TP is shown in Annex.

7.  Reference

[1]  RP-200271  WID revision: Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL); rapporteur: Huawei

Annex.   Proposed text for TR37.716-21-11:

[Unchanged Parts Skipped]

5.1.x	DC_1-11_n3

### 5.1.x.1	Operating bands for DC_1-11_n3

Table 5.1.x.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-11_n3 | CA_1-11 | n3 | DC_1_n3 |

### 5.1.x.2	Configurations for DC_1-11_n3

Table 5.1.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-11A_n3A | DC_1A_n3A DC_11A_n3A | CA_1A-11A | n3A |

### 5.1.x.3	Co-existence studies

When Uplink EN-DC configuration is DC_1A_n3A, (1) IMD3 of (B1 - n3) will fall into Rx band of Band 11. When Uplink EN-DC configuration is DC_11A_n3A, (2) IMD3 of (B11 - n3) will fall into Rx band of Band 1.

### 5.1.x.4	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.1.x.4-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n3 | 1 | 0.3 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |

Table 5.1.x.4-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n3 | 1 | 0 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |

### 5.1.x.5	REFSENS requirements

As mentioned above, IMD3 of B1 and n3 to Band11 Rx and IMD3 of B11 and n3 to Band1 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_1A-11A_n3A | 1 | TBD | TBD | TBD | TBD | N/A | FDD | N/A | Yes |
|  | n3 | TBD | TBD | TBD | TBD | N/A | TDD | N/A | Yes |
|  | 11 | TBD | TBD | TBD | TBD | TBD | FDD | IMD3 |  |
| DC_1A-11A_n3A | 11 | TBD | TBD | TBD | TBD | N/A | FDD | N/A |  |
|  | n3 | TBD | TBD | TBD | TBD | N/A | TDD | N/A | Yes |
|  | 1 | TBD | TBD | TBD | TBD | TBD | FDD | IMD3 | Yes |

[Unchanged Parts Skipped]