3GPP TSG-RAN WG4 Meeting #94-e-bis				                           R4-2003511

Electronic Meeting, 20 – 30 April, 2020

Agenda Item: 6.8.2.1.5

Source: MediaTek

Title: LLS Evaluation on PRS-RSTD & PRS-RSRP measurement

Document for:	Discussion

# Introduction

In this contribution, we provide our link level simulation results on PRS-RSTD and PRS-RSRP measurement.

# Simulation results for PRS-RSTD

Assumptions:

- PRSs are in the same positioning frequency layer

- PRS Es/Iot for the reference cell: -6dB

- PRS Es/Iot for the neighbor cell: -13dB

- Number of of PRS repetition: 1

- PRS structure: comb-4, 4 symbols

- Network synchronization: Synchronous with time shifts <0, 0, 3 us>

In the following we show 90%-tile of the RSTD errors for one neighbour cell as the RSTD errors of the two neighbour cells are statistically similar.

| Configuration (Channel: AWGN) |  | 90%-tile of the RSTD errors |
| --- | --- | --- |
| SCS 15kHz | BW 10MHz | 65.2ns |
|  | BW 20MHz | 32.6ns |
|  | BW 50MHz | 16.3ns |
| SCS 30kHz | BW 20MHz | 32.6 ns |
|  | BW 50MHz | 16.3 ns |
|  | BW 100MHz | 8.15 ns |
| SCS 120kHz | BW 50MHz | 16.19 ns |
|  | BW 100MHz | 4.05 ns |
|  | BW 200MHz | 2.02 ns |

| Configuration |  | TDL-A (30 ns delay spread, 5Hz) | TDL-B (100 ns delay spread, 200Hz) | TDL-C (300 ns delay spread, 100 Hz) |
| --- | --- | --- | --- | --- |
| SCS 15kHz | BW 10MHz | 65.2ns | 130.4 ns | 244.5 ns |
|  | BW 20MHz | 32.6 ns | 48.9 ns | 163 ns |
|  | BW 50MHz | 16.3ns | 16.3 ns | 65.2 ns |
| SCS 30kHz | BW 20MHz | 32.6 ns | 130.4 ns | 220.5 ns |
|  | BW 50MHz | 16.3 ns | 32.6 ns | 179.3 ns |
|  | BW 100MHz | 8.15 ns | 16.3 ns | 69.36ns |

| Configuration |  | TDL-C (60 ns delay spread, 300 Hz) |
| --- | --- | --- |
| SCS 120kHz | BW 100MHz | 44.53 ns |
|  | BW 200MHz | 40.48 ns |

# Simulation Results for PRS-RSRP

Assumptions:

-	PRSs are in the same positioning frequency layer

-	Number of of PRS repetition: 1

-	PRS structure: comb-4, 4 symbols

Assume PRS Es/Iot = -6dB, in the following we show 90%-tile of the RSRP errors.

| Configuration |  | Es/Iot = -6 dB |
| --- | --- | --- |
| SCS 15kHz | BW 10MHz | 2.8 dB |
|  | BW 20MHz | 2.7 dB |
|  | BW 50MHz | 2.6 dB |
| SCS 30kHz | BW 20MHz | 2.8 dB |
|  | BW 50MHz | 2.7 dB |
|  | BW 100MHz | 2.6 dB |
| SCS 120kHz | BW 50MHz | 2.9 dB |
|  | BW 100MHz | 2.8 dB |
|  | BW 200MHz | 2.7 dB |

Assume PRS Es/Iot = -13dB, in the following we show 90%-tile of the RSRP errors.

| Configuration |  | Es/Iot = -13 dB |
| --- | --- | --- |
| SCS 15kHz | BW 10MHz | 7.1 dB |
|  | BW 20MHz | 7 dB |
|  | BW 50MHz | 6.9 dB |
| SCS 30kHz | BW 20MHz | 7.1 dB |
|  | BW 50MHz | 7 dB |
|  | BW 100MHz | 6.9 dB |
| SCS 120kHz | BW 50MHz | 7.3 dB |
|  | BW 100MHz | 7.2 dB |
|  | BW 200MHz | 7 dB |

Observation 1: Under Es/Iot = -13 dB, the accuracy of PRS-RSRP is not good. The accuracy requirements defined on such side condition may not be meaningful.

We have the following observation:

# Conclusions

In this contribution, we provide our LLS results on PRS-RSTD and PRS-RSRP. The simulation results should be taken into consideration for discussion on accuracy requirement and the corresponding simulation settings.