3GPP TSG-RAN WG4 Meeting #94-e-Bis	R4-2003569

Online, 20 – 30 April, 2020

Agenda item:	6.8.2.1.3

Source: 	Qualcomm Incorporated

Title: 	On UE Rx-Tx timing difference measurements in NR positioning

Document for:	Discussion

# Introduction

UE Rx-Tx timing difference measurement was discussed in RAN4#94-e meeting with WF agreed in [1]. In this paper, we further discuss the following topics:

- Intra-frequency vs. inter-frequency Rx-Tx timing difference measurement

- Measurement report mapping

- Measurement capability and reporting criteria

- Proximity of SRS transmission with PRS

- Measurement requirements

  - Accuracy

  - Side conditions

  - Period

# Intra-frequency vs. inter-frequency Rx-Tx timing difference measurement

Our views and proposal on the definition of intra-frequency and inter-frequency PRS-RSTD is discussed in [2]. For UE Rx-Tx timing difference measurement, an addition factor that needs to be considered is whether reception of PRS and transmission of SRS is on the same or different component carriers (CC).

In our understanding, the baseline and default scenario should be reception of PRS and transmission of SRS on the same CC. This leads to calibration of UE Rx and Tx paths with the same RF configuration which can translate to better accuracy requirements. Reception of PRS and transmission of SRS on difference CCs requires more complicated calibration of UE Rx and Tx paths with different RF configurations, RF filters, and group delays that results in degraded accuracy requirements. Our understanding is that making Rx-Tx timing difference measurements on different CCs for PRS reception and SRS transmission may end up being an additional UE capability.

Since intra-frequency measurements are typically associated with better accuracy requirements, our proposal is to define intra-frequency measurement based on PRS and SRS on the same CC. Hence,

Proposal 1. UE Rx-Tx timing difference measurement is defined as intra-frequency if all the following conditions are met:

- DL PRS resource belongs to a positioning frequency layer whose BW is within the BW of UE’s active DL BWP

- SCS and CP of the DL PRS resource are the same as those of UE’s active DL BWP

- UL SRS resource for positioning is on the same component carrier as that of DL PRS resource

Otherwise, the UE Rx-Tx timing difference measurement is inter-frequency.

# Measurement report mapping

Several proposals were discussed regarding RSTD report mapping table in RAN4#94-e meeting with no progress or agreement:

UE Rx-Tx time difference report mapping table(s):

- UE Rx-Tx time difference report mapping table(s) are FFS.

- RAN4 to continue discussion on UE Rx-Tx report mapping table(s) in the next meeting considering e.g. the following aspects:

  - Uniform vs. non-uniform granularity

  - Number of report mapping tables

  - Range in FR1 and FR2, Impact of parameter NTA,Offset

    - Whether LMF needs to be aware of NTA,Offset

  - Mapping with negative reported values for neighbor cell UE Rx-TX

  - Parameter k (T=Tc2k) where k is a configuration parameter

    - Whether UE always can follow signaled k or may need to adjust k

      - FFS: whether UE can report its capability of k to NW

    - Min/max values of k, and whether they depend on FR1/FR2 and center/edge mapping

    - Whether gNB needs to be aware of k

  - Note: other relevant aspects are not precluded

Before delving into details, two important and distinct considerations about UE Rx-Tx timing difference measurement should be addressed:

- Range of Rx-Tx timing and whether it needs to cover both positive and negative ranges

- Whether NTA,Offset should be reflected in the report mapping

The definition of Rx-Tx timing difference in NR has a fundamental difference compared to its counterpart in LTE as highlighted in yellow below:

Definition of Rx-Tx time difference in LTE (TS 36.214):

| Definition | The UE Rx – Tx time difference is defined as TUE-RX – TUE-TX Where: TUE-RX is the UE received timing of downlink radio frame #i from the serving cell, defined by the first detected path in time. TUE-TX is the UE transmit timing of uplink radio frame #i. The reference point for the UE Rx – Tx time difference measurement shall be the UE antenna connector. For a HD-FDD UE, if the UE does not receive any DL transmission in radio frame #i, it shall compensate for the difference in the received timing of radio frame #i and the radio frame in which it did receive a DL transmission used for TUE-RX estimation. |
| --- | --- |

Definition of Rx-Tx time difference in NR (TS 38.215):

| Definition | The UE Rx – Tx time difference is defined as TUE-RX – TUE-TX Where: TUE-RX is the UE received timing of downlink subframe #i from a positioning node, defined by the first detected path in time. TUE-TX is the UE transmit timing of uplink subframe #j that is closest in time to the subframe #i received from the positioning node. Multiple DL PRS resources can be used to determine the start of one subframe of the first arrival path of the positioning node. For frequency range 1, the reference point for TUE-RX measurement shall be the Rx antenna connector of the UE and the reference point for TUE-TX measurement shall be the Tx antenna connector of the UE. For frequency range 2, the reference point for TUE-RX measurement shall be the Rx antenna of the UE and the reference point for TUE-TX measurement shall be the Tx antenna of the UE. |
| --- | --- |

The above difference means that the reported UE Rx-Tx timing difference can now be negative even for serving cell (e.g., with large TA offset) and that the range of Rx-Tx timing difference is [-0.5, 0.5]ms since measurement is with respect to closes subframe (1ms).

Proposal 2. Range of reported UE Rx-Tx timing difference to be [-0.5, 0.5] ms, i.e., same as RSTD reporting range.

On inclusion of NTA,Offset in the report mapping, our understanding is that even in LTE, this was optionally included by UE. From TS 36.355:

The IE ECID-ProvideCapabilities is used by the target device to indicate its capability to support E-CID and to provide its E-CID location capabilities to the location server.

-- ASN1START

# ECID-ProvideCapabilities ::= SEQUENCE {

ecid-MeasSupported	BIT STRING {	rsrpSup		(0),

rsrqSup		(1),

ueRxTxSup	(2),

nrsrpSup-r14	(3),

nrsrqSup-r14	(4)} (SIZE(1..8)),

...,

[[	ueRxTxSupTDD-r13					ENUMERATED { true }				OPTIONAL

]],

[[	periodicalReporting-r14				ENUMERATED { supported }		OPTIONAL,

triggeredReporting-r14				ENUMERATED { supported }		OPTIONAL,

idleStateForMeasurements-r14		ENUMERATED { required }			OPTIONAL

]]

}

-- ASN1STOP

ueRxTxSupTDD This field, if present, indicates that any UE Rx-Tx time difference measurement reporting for TDD from the target device includes the NTAoffset according to [16], [17] and uses the UE Rx-Tx time difference measurement report mapping for TDD as specified in 3GPP TS 36.133 [18]. This field may only be included if the ueRxTxSup field in ecid-MeasSupported is set to value one.

Observation 1. In LTE, including NTA,Offset in the reporting of UE Rx-Tx timing difference measurement is optional.

Since the positioning measurement can work without including NTA,Offset in the report, it is proposed in NR to not include it.

Proposal 3. Rx-Tx time difference report mapping tables to not include NTA,Offset .

In the last RAN2 meeting, an LS was sent to RAN4 on positioning measurement range and granularity [4]. Based on the agreement from RAN1:

Agreement:

Confirm the working assumption from RAN1#98bis on reporting of multiple Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning. The FFS item in the working assumption is removed.

RAN2 agreed to the following in order to reduce the signaling overhead:

Confirm (same as current running CR) when a UE is configured to report  multiple DL PRS RSTD, PRS RSRP, RxTX measurements with each measurement between a different pair of DL PRS resources or DL PRS resource sets, and those multiple measurements being performed on the same pair of TRPs, the UE reports one full measurement results and additional delta measurement(s).

For Rx-Tx timing differential report mapping, RAN2 has agreed to the following IE:

Multi-RTT

# NR-Multi-RTT-MeasList-r16 ::= SEQUENCE (SIZE(1.. nrMaxTRPs)) OF NR-Multi-RTT-MeasElement-r16

# NR-Multi-RTT-MeasElement-r16 ::= SEQUENCE {

# trp-ID-r16							TRP-ID-r16			OPTIONAL,

nr-DL-PRS-ResourceId-r16			NR-DL-PRS-ResourceId-r16	OPTIONAL,

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-UE-RxTxTimeDiff-r16				INTEGER (0..ffs)	OPTIONAL,	-- FFS on the value range, to be decided in RAN4

nr-TimeStamp-r16					NR-TimeStamp-r16,

nr-MeasQuality-r16					NR-MeasQuality-r16,

nr-PRS-RSRP-Result-r16				INTEGER (FFS)			OPTIONAL, -- FFS, value range to be decided in RAN4.

nr-Multi-RTT-AdditionalMeasurements-r16		NR-Multi-RTT-AdditionalMeasurements-r16,

...

}

NR-Multi-RTT-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF NR-Multi-RTT-AdditionalMeasurementElement-r16

NR-Multi-RTT-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceId-r16			NR-DL-PRS-ResourceId-r16	OPTIONAL,

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-PRS-RSRP-ResultDiff-r16			INTEGER (FFS)			OPTIONAL, -- FFS, value range to be decided in RAN4.

nr-UE-RxTxTimeDiffAdditional-r16				INTEGER (0..ffs)	OPTIONAL,	-- FFS on the value range, to be decided in RAN4

nr-TimeStamp-r16					NR-TimeStamp-r16,

...

}

nrMaxTRPs		INTEGER ::= 256		-- Max TRPs

Based on these agreements in RAN1/2, RAN4 needs to define differential Rx-Tx timing difference report mapping tables. From the above discussions and observations, it is proposed to use identical report mapping tables for UE Rx-Tx timing difference as in RSTD (with the exception of label names). Tables A-1 to A-7 in Appendix are produced for reference.

Proposal 4. Absolute and differential RSTD report mapping tables to be as in Tables A-1 to A-7.

In addition, RAN2 has requested RAN4 on providing the range for NR-AdditionalPath:

In addition, RAN2 has agreed to introduce reporting of additional paths to NR positioning downlink timing measurements, in particular RSTD measurements of DL-TDOA and UE RxTx measurements of multi-RTT. The RAN2 agreement is:

Agreements:

Add support for additional path reporting to LPP for timing-based measurements, similar to LTE.

Recommend RAN3 to support timing-based additional path reporting in NRPPa.

#### –	NR-AdditionalPath

The IE NR-AdditionalPath is used by the target device to provide information about additional paths in association to the TOA measurements associated to NR positioning in the form of a relative time difference and a quality value. The additional path nr-relativeTimeDifference is the detected path timing relative to the detected path timing used for the TOA value, and each additional path can be associated with a quality value nr-path-Quality.

-- ASN1START

# NR-AdditionalPath-r16 ::= SEQUENCE {

nr-relativeTimeDifference-r16	FFS to be decided in RAN4,			OPTIONAL,

nr-path-Quality-r16				NR-TOAMeasQuality-r16				OPTIONAL,

...

}

-- ASN1STOP

In LTE, the same measurement was defined with range of ± 4.17 us:

-- ASN1START

AdditionalPath-r14 ::= SEQUENCE {

relativeTimeDifference-r14    INTEGER (-256..255),

path-Quality-r14                    OTDOA-MeasQuality                          OPTIONAL,

...

}

-- ASN1STOP

AdditionalPath field descriptions

relativeTimeDifference This field specifies the additional detected path timing relative to the detected path timing used for the rstd value in units of 0.5 Ts, with Ts=1/(15000*2048) seconds. A positive value indicates that the particular path is later in time than the detected path used for RSTD; a negative value indicates that the particular path is earlier in time than the detected path used for RSTD.

In NR, it is proposed to use the same range for minimum and maximum. The granularity is proposed to follow the same approach as in other measurements, i.e., UE follows the signaled parameter k.

Proposal 5. The minimum and maximum range of NR-AdditionalPath to be the same as in LTE, i.e., ± 4.17 us. The granularity to follow the signaled parameter k.

In ASN.1 implementation, this leads to:

nr-relativeTimeDifference-r16 CHOICE {

k1m     INTEGER (-16384..16383),

k0		INTEGER (-8192..8191),

k1		INTEGER (-4096..4095),

k2		INTEGER (-2048..2047),

k3		INTEGER	(-1024..1023),

k4		INTEGER	(-512..511),

k5		INTEGER	(-256..255)

}

# Measurement capability and reporting criteria

In [1], the following was captured:

UE Rx-Tx time difference reporting criteria:

- Ecat for UE Rx-Tx time difference is FFS.

  - Option 1. 1 per positioning session

  - Option 2.  In multi-RTT positioning, each UE Rx-Tx time difference measurement reporting criterion corresponds to one frequency layer with Ecat = 1 indicating Ntotal UE Rx-Tx time difference measurement reports according to signaled capabilities

  - Option 3:

    - intra-frequency UE Rx-Tx measurements, 1 report capable of UE Rx-Tx measurements and PRS-RSRP measurements (when configured together with the UE Rx-Tx) on at least TBD PRS resources, per intra-frequency layer

    - inter-frequency UE Rx-Tx measurements, 1 report capable of UE Rx-Tx measurements and PRS-RSRP measurements (when configured together with the UE Rx-Tx) on at least TBD PRS resources, per inter-frequency layer

  - Other options are not precluded

Our views and proposals regarding measurement capability and reporting criteria for PRS-RSTD are expressed in [2]. Same observations and proposals apply to UE Rx-Tx timing difference measurement as well. RAN4 should not define any minimum capability requirements. UE capability signaling should be adhered.

In previous meeting, some companies proposed including PRS-RSRP report in Rx-Tx timing difference measurement capability and reporting criteria. In our understanding, PRS-RSRP reporting in DL-TDOA or multi-RTT positioning methods are optional UE capabilities. The snippet in Section 4 from RAN2 LS also clearly indicate that PRS-RSRP is optionally reported in the measurement report IE.

Observation 2. PRS-RSRP reporting together with UE Rx-Tx timing difference positioning measurements is an optional UE capability.

Hence, it is proposed that:

Proposal 6. In multi-RTT and for each positioning session, Ecat = 1 per positioning frequency layer where the number of UE Rx-Tx timing difference measurement reports per positioning frequency layer is according to UE’s signaled capabilities.

# Proximity of SRS transmission with PRS reception

Proximity of SRS and PRS in UE Rx-Tx time difference measurement:

- FFS:

  - whether the core measurement and performance requirements for UE Rx-Tx time difference applies if the configured SRS-Slot-offset and SRS-Periodicity parameters for SRS resource for positioning are such that any SRS transmission is within [-X, X] msec of at least one DL PRS resource from each of the TRPs in the assistance data.

    - Option 1: the above constraint is not needed, i.e., no limit on X

    - Option 2: the above constraint is needed, X is FFS

Ideally, all measurements used in generating a positioning fix should be made concurrently. If measurements are performed at different points in time for generating a position fix, UE motion as well as changes to UE clock and gNB clocks result in measurement errors that ultimately can produce position errors. For instance, traveling at highway speed of 30 m/s can result in a 1 s * 30 m/s = 30 m measurement error if two measurements are performed 1 second apart.

More importantly, the UE and gNB clock drift can produce errors in timing measurements as explained below. Figure 1 illustrates an example of PRS reception with SRS transmission timeline where PRS periodicity is half of SRS periodicity. For Rx-Tx timing difference measurements corresponding to PRS instance #0, SRS instance #0 is used. Similarly, for Rx-Tx timing difference measurements corresponding to PRS instance #2, SRS instance #2 is used. However, for Rx-Tx timing difference measurements corresponding to PRS instance #1, either SRS instance#0 or #1 should be used which can have degraded accuracy due to long time lapse between PRS reception and SRS transmission and impact of clock drifts.

Figure 1 Proximity of PRS reception and SRS transmission in UE Rx-Tx timing difference measurements

To quantify the impact of clock drift, the transmit signal quality and timing error requirements should be looked at. Transmit timing error requirements in clause 7.1.2 of TS 38.133 is on the order of several Tc. For instance, the UE transmit timing can drift from the beginning of the positioning measurements with Te=-12.64.Tc to the end of the positioning measurement with Te=12.64.Tc at 15 kHz SCS and UE still meets the transmit timing requirements and requiring no timing adjustment.

Observation 3. UE motion, UE clock drift and gNB clock drift can result in significant errors for measurements that are performed apart in time but are all used to generate the same position fix.

Moreover, in TS 38.101-1, the transmit signal quality requirements is:

6.4 Transmit signal quality

6.4.1 Frequency error

The UE modulated carrier frequency shall be accurate to within ±0.1 PPM observed over a period of 1 ms compared to the carrier frequency received from the NR Node B.

This means UE clock drift can be 100 part-per-billion (ppb). With the stricter 3 - 10m accuracy targets for commercial use cases in NR, it is vital to keep the measurement error due to drift and other impairment at least an order of magnitude below the accuracy requirements which is on the order of Tc. Hence, assuming a maximum 0.05Tc error budget for clock drift during the measurement:

X=0.05Tc100.10-9=25 ms

It is thus proposed that measurement requirements be applicable when the SRS transmissions are within [-25, 25] ms of at least one DL PRS resource from each of the TRPs in the assistance data.

Proposal 7. The measurement requirements for UE Rx-Tx timing difference is applicable only if the configured parameters SRS-Slot-offset and SRS-Periodicity for SRS resource for positioning are such that any SRS transmission is within [-25, 25] msec of at least one DL PRS resource of each of the TRPs in the assistance data.

It is further noted that the above proposal improves the accuracy of both UE and gNB Rx-Tx timing difference measurements.

# Measurement requirements

## Measurement Accuracy

UE Rx-Tx time difference accuracy requirements:

- Accuracy requirements under cell change are FFS

- Measurement period/UE behavior/Accuracy requirements with TA adjustment are FFS

In our understanding, the number of samples used for measuring TUE-RX in Rx-Tx timing difference measurement should be the same as the number of samples used for RSTD measurement. In both types of measurements, UE measures TOA of one or two cells. Thus, if number of samples (defined as the time duration spanned by one DL PRS resource after repetition by DL-PRS-ResourceRepetitionFactor) used for RSTD estimation is N, the number of samples for Rx-Tx timing measurement should also be N.

Proposal 8. The number of samples, N, used to define the accuracy requirements of Rx-Tx timing difference measurement to be the same as the number of samples used to define the accuracy requirements for RSTD measurements.

Moreover, it is noted that:

Observation 4. Two factors impact the accuracy requirements of gNB/UE Rx-Tx time difference measurement:

- TUE-RX estimation error from DL PRS (for UE) or UL SRS (for gNB) resources

- Rx-Tx calibration error

In [5], our simulation results for estimation accuracy of TUE-RX is presented based on the same simulation assumptions used for RSTD.

Regarding the Rx-Tx calibration error, the error budget should be viewed holistically from both gNB and UE perspective. The performance and accuracy of multi-RTT depends on Rx-Tx calibration error on both UE and gNB side. Hence, allocating a small error budget on one side (UE or gNB) while permitting a disproportionately larger error budget on the other side is nonsensical.

Observation 5. The performance and accuracy of multi-RTT depends on Rx-Tx calibration error on both UE and gNB. Hence, the error budget should be viewed holistically from both sides. Allocating a small error budget on one side (UE or gNB) while permitting a disproportionately larger error budget on the other side is nonsensical.

## Side conditions

The side conditions for neighbor cell were agreed to be the same as those in RSTD measurement for FR1 and FR2. The remaining open issue is the side conditions for serving cell (if needed) and reference cell (if needed).

In our view, there is no need to define side conditions for serving cell in UE Rx-Tx timing difference measurement. It is noted that regardless of the positioning technique, the presence of PRS reference resource (or set) is necessary as UE needs to know what is the relative timing of DL PRS resources to be able to measure TUE-RX correctly. This is reflected in TS 38.214:

The UE expects to be configured with higher layer parameter DL-PRS-expectedRSTD, which defines the time difference with respect to the received DL subframe timing the UE is expected to receive DL PRS, and DL-PRS-expectedRSTD-uncertainty, which defines a search window around the expectedRSTD.

However, in Rx-Tx timing difference measurement, there is may not measure TUE-RX of reference resource. Hence, only one side condition is needed and applicable to the PRS resources in the assistance data.

Proposal 9. On Rx-Tx timing difference measurement side conditions:

- Serving cell side condition is not needed

- Reference cell side condition is not needed but if strong views exists on its presence, it shall be the same as PRS-RSTD reference cell side condition for all positioning methods.

## Measurement period

The formulation for the RSTD measurement period is presented in [2]. The UE Rx-Tx measurement period is similar except that the signaled capabilities for Rx-Tx measurement are different from RSTD measurement.

# Conclusions

Proposal 1. UE Rx-Tx timing difference measurement is defined as intra-frequency if all the following conditions are met:

- DL PRS resource belongs to a positioning frequency layer whose BW is within the BW of UE’s active DL BWP

- SCS and CP of the DL PRS resource are the same as those of UE’s active DL BWP

- UL SRS resource for positioning is on the same component carrier as that of DL PRS resource

Otherwise, the UE Rx-Tx timing difference measurement is inter-frequency.

Proposal 2. Range of reported UE Rx-Tx timing difference to be [-0.5, 0.5] ms, i.e., same as RSTD reporting range.

Observation 1. In LTE, including NTA,Offset in the reporting of UE Rx-Tx timing difference measurement is optional.

Proposal 3. Rx-Tx time difference report mapping tables to not include NTA,Offset .

Proposal 4. Absolute and differential RSTD report mapping tables to be as in Tables A-1 to A-7.

Proposal 5. The minimum and maximum range of NR-AdditionalPath to be the same as in LTE, i.e., ± 4.17 us. The granularity to follow the signaled parameter k.

Observation 2. PRS-RSRP reporting together with UE Rx-Tx timing difference positioning measurements is an optional UE capability.

Proposal 6. In multi-RTT and for each positioning session, Ecat = 1 per positioning frequency layer where the number of UE Rx-Tx timing difference measurement reports per positioning frequency layer is according to UE’s signaled capabilities.

Observation 3. UE motion, UE clock drift and gNB clock drift can result in significant errors for measurements that are performed apart in time but are all used to generate the same position fix.

Proposal 7. The measurement requirements for UE Rx-Tx timing difference is applicable only if the configured parameters SRS-Slot-offset and SRS-Periodicity for SRS resource for positioning are such that any SRS transmission is within [-25, 25] msec of at least one DL PRS resource of each of the TRPs in the assistance data.

Proposal 8. The number of samples, N, used to define the accuracy requirements of Rx-Tx timing difference measurement to be the same as the number of samples used to define the accuracy requirements for RSTD measurements.

Observation 4. Two factors impact the accuracy requirements of gNB/UE Rx-Tx time difference measurement:

- TUE-RX estimation error from DL PRS (for UE) or UL SRS (for gNB) resources

- Rx-Tx calibration error

Observation 5. The performance and accuracy of multi-RTT depends on Rx-Tx calibration error on both UE and gNB. Hence, the error budget should be viewed holistically from both sides. Allocating a small error budget on one side (UE or gNB) while permitting a disproportionately larger error budget on the other side is nonsensical.

Proposal 9. On Rx-Tx timing difference measurement side conditions:

- Serving cell side condition is not needed

- Reference cell side condition is not needed but if strong views exists on its presence, it shall be the same as PRS-RSTD reference cell side condition for all positioning methods.

# Appendix Measurement report mapping tables

Table A-1 – Absolute (left) and differential (right) UE Rx-Tx timing difference report mapping table for k=-1 (units of Tc). Measured quantity value in differential report mapping table is from smallest UE Rx-Tx timing difference. Absolute and differential Rx-Tx timing difference tables require 22 and 15 bits, respectively.

| Reported value; Measured quantity value. RX_TX_TIMEDIFF_0; T_RX-TX < -983040.0. RX_TX_TIMEDIFF_1; -983040.0 ≤ T_RX-TX < -983039.5. RX_TX_TIMEDIFF_2; -983039.5 ≤ T_RX-TX < -983039.0. …; …. RX_TX_TIMEDIFF_1966081; 0.0 ≤ T_RX-TX < 0.5. …; …. RX_TX_TIMEDIFF_3932160; 983039.5 ≤ T_RX-TX < 983040.0. RX_TX_TIMEDIFF_3932161; T_RX-TX ≥ 983040.0. | Reported value; Measured quantity value. RX_TX_TIMEDIFF_ADD_0; 0.0 ≤ ΔT_RX-TX < 0.5. RX_TX_TIMEDIFF_ADD_1; 0.5 ≤ΔT_RX-TX < 1.0. RX_TX_TIMEDIFF_ADD_2; 1.0 ≤ ΔT_RX-TX < 1.5. …; …. …; …. RX_TX_TIMEDIFF_ADD_31487; 15743.0 ≤ ΔT_RX-TX < 15743.5. RX_TX_TIMEDIFF_ADD_31488; 15743.5 ≤ ΔT_RX-TX < 15744.0. RX_TX_TIMEDIFF_ADD_31489; ΔT_RX-TX ≥ 15744.0. |
| --- | --- |

Table A-2 – Absolute (left) and differential (right) UE Rx-Tx timing difference report mapping table for k=0 (units of Tc). Measured quantity value in differential report mapping table is from smallest UE Rx-Tx timing difference. Absolute and differential Rx-Tx timing difference tables require 21 and 14 bits, respectively.

| Reported value; Measured quantity value. RX_TX_TIMEDIFF_0; T_RX-TX < -983040. RX_TX_TIMEDIFF_1; -983040 ≤ T_RX-TX < -983039. RX_TX_TIMEDIFF_2; -983039 ≤ T_RX-TX < -983038. …; …. RX_TX_TIMEDIFF_983041; 0 ≤ T_RX-TX < 1. …; …. RX_TX_TIMEDIFF_1966080; 983039 ≤ T_RX-TX < 983040. RX_TX_TIMEDIFF_1966081; T_RX-TX ≥ 983040. | Reported value; Measured quantity value. RX_TX_TIMEDIFF_ADD_0; 0 ≤ ΔT_RX-TX < 1. RX_TX_TIMEDIFF_ADD_1; 1 ≤ ΔT_RX-TX < 2. RX_TX_TIMEDIFF_ADD_2; 2 ≤ ΔT_RX-TX < 3. …; …. …; …. RX_TX_TIMEDIFF_ADD_15743; 15742 ≤ ΔT_RX-TX < 15743. RX_TX_TIMEDIFF_ADD_15744; 15743 ≤ ΔT_RX-TX < 15744. RX_TX_TIMEDIFF_ADD_15745; ΔT_RX-TX ≥ 15744. |
| --- | --- |

Table A-3 – Absolute (left) and differential (right) UE Rx-Tx timing difference report mapping table for k=1  (units of Tc). Measured quantity value in differential report mapping table is from smallest UE Rx-Tx timing difference. Absolute and differential Rx-Tx timing difference tables require 20 and 13 bits, respectively.

| Reported value; Measured quantity value. RX_TX_TIMEDIFF_0; T_RX-TX < -983040. RX_TX_TIMEDIFF_1; -983040 ≤ T_RX-TX < -983038. RX_TX_TIMEDIFF_2; -983038 ≤ T_RX-TX < -983036. …; …. RX_TX_TIMEDIFF_491521; 0 ≤ T_RX-TX < 2. …; …. RX_TX_TIMEDIFF_983040; 983038 ≤ T_RX-TX < 983040. RX_TX_TIMEDIFF_983041; T_RX-TX ≥ 983040. | Reported value; Measured quantity value. RX_TX_TIMEDIFF_ADD_0; 0 ≤ ΔT_RX-TX < 2. RX_TX_TIMEDIFF_ADD_1; 2 ≤ ΔT_RX-TX < 4. RX_TX_TIMEDIFF_ADD_2; 4 ≤ ΔT_RX-TX < 6. …; …. …; …. RX_TX_TIMEDIFF_ADD_7871; 15740 ≤ ΔT_RX-TX < 15742. RX_TX_TIMEDIFF_ADD_7872; 15742 ≤ ΔT_RX-TX < 15744. RX_TX_TIMEDIFF_ADD_7873; ΔT_RX-TX ≥ 15744. |
| --- | --- |

Table A-4 – Absolute (left) and differential (right) UE Rx-Tx timing difference report mapping table for k=2 (units of Tc). Measured quantity value in differential report mapping table is from smallest UE Rx-Tx timing difference. Absolute and differential Rx-Tx timing difference tables require 19 and 12 bits, respectively.

| Reported value; Measured quantity value. RX_TX_TIMEDIFF_0; T_RX-TX < -983040. RX_TX_TIMEDIFF_1; -983040 ≤ T_RX-TX < -983036. RX_TX_TIMEDIFF_2; -983036 ≤ T_RX-TX < -983032. …; …. RX_TX_TIMEDIFF_245761; 0 ≤ T_RX-TX < 4. …; …. RX_TX_TIMEDIFF_491520; 983036 ≤ T_RX-TX < 983040. RX_TX_TIMEDIFF_491521; T_RX-TX ≥ 983040. | Reported value; Measured quantity value. RX_TX_TIMEDIFF_ADD_0; 0 ≤ ΔT_RX-TX < 4. RX_TX_TIMEDIFF_ADD_1; 4 ≤ ΔT_RX-TX < 8. RX_TX_TIMEDIFF_ADD_2; 8 ≤ ΔT_RX-TX < 12. …; …. …; …. RX_TX_TIMEDIFF_ADD_3935; 15736 ≤ ΔT_RX-TX < 15740. RX_TX_TIMEDIFF_ADD_3936; 15740 ≤ ΔT_RX-TX < 15744. RX_TX_TIMEDIFF_ADD_3937; ΔT_RX-TX ≥ 15744. |
| --- | --- |

Table A-5 – Absolute (left) and differential (right) UE Rx-Tx timing difference report mapping table for k=3 (units of Tc). Measured quantity value in differential report mapping table is from smallest UE Rx-Tx timing difference. Absolute and differential Rx-Tx timing difference tables require 18 and 11 bits, respectively.

| Reported value; Measured quantity value. RX_TX_TIMEDIFF_0; T_RX-TX < -983040. RX_TX_TIMEDIFF_1; -983040 ≤ T_RX-TX < -983032. RX_TX_TIMEDIFF_2; -983032 ≤ T_RX-TX < -983024. …; …. RX_TX_TIMEDIFF_122881; 0 ≤ T_RX-TX < 8. …; …. RX_TX_TIMEDIFF_245760; 983032 ≤ T_RX-TX < 983040. RX_TX_TIMEDIFF_245761; T_RX-TX ≥ 983040. | Reported value; Measured quantity value. RX_TX_TIMEDIFF_ADD_0; 0 ≤ ΔT_RX-TX < 8. RX_TX_TIMEDIFF_ADD_1; 8 ≤ ΔT_RX-TX < 16. RX_TX_TIMEDIFF_ADD_2; 16 ≤ ΔT_RX-TX < 32. …; …. …; …. RX_TX_TIMEDIFF_ADD_1965; 15728 ≤ ΔT_RX-TX < 15736. RX_TX_TIMEDIFF_ADD_1968; 15736 ≤ ΔT_RX-TX < 15744. RX_TX_TIMEDIFF_ADD_1969; ΔT_RX-TX ≥ 15744. |
| --- | --- |

Table A-6 – Absolute (left) and differential (right) UE Rx-Tx timing difference report mapping table for k=4 (units of Tc). Measured quantity value in differential report mapping table is from smallest UE Rx-Tx timing difference. Absolute and differential Rx-Tx timing difference tables require 17 and 10 bits, respectively.

| Reported value; Measured quantity value. RX_TX_TIMEDIFF_0; T_RX-TX < -983040. RX_TX_TIMEDIFF_1; -983040 ≤ T_RX-TX < -983024. RX_TX_TIMEDIFF_2; -983024 ≤ T_RX-TX < -983008. …; …. RX_TX_TIMEDIFF_61441; 0 ≤ T_RX-TX < 16. …; …. RX_TX_TIMEDIFF_122880; 983024 ≤ T_RX-TX < 983040. RX_TX_TIMEDIFF_122881; T_RX-TX ≥ 983040. | Reported value; Measured quantity value. RX_TX_TIMEDIFF_ADD_0; 0 ≤ ΔT_RX-TX < 16. RX_TX_TIMEDIFF_ADD_1; 16 ≤ ΔT_RX-TX < 32. RX_TX_TIMEDIFF_ADD_2; 32 ≤ ΔT_RX-TX < 48. …; …. …; …. RX_TX_TIMEDIFF_ADD_983; 15712 ≤ ΔT_RX-TX < 15728. RX_TX_TIMEDIFF_ADD_984; 15728 ≤ ΔT_RX-TX < 15744. RX_TX_TIMEDIFF_ADD_985; ΔT_RX-TX ≥ 15744. |
| --- | --- |

Table A-7 – Absolute (left) and differential (right) UE Rx-Tx timing difference report mapping table for k=5 (units of Tc). Measured quantity value in differential report mapping table is from smallest UE Rx-Tx timing difference. Absolute and differential Rx-Tx timing difference tables require 16 and 9 bits, respectively.

| Reported value; Measured quantity value. RX_TX_TIMEDIFF_0; T_RX-TX < -983040. RX_TX_TIMEDIFF_1; -983040 ≤ T_RX-TX < -983008. RX_TX_TIMEDIFF_2; -983008 ≤ T_RX-TX < -982976. …; …. RX_TX_TIMEDIFF_30721; 0 ≤ T_RX-TX < 32. …; …. RX_TX_TIMEDIFF_61440; 983008 ≤ T_RX-TX < 983040. RX_TX_TIMEDIFF_61441; T_RX-TX ≥ 983040. | Reported value; Measured quantity value. RX_TX_TIMEDIFF_ADD_0; 0 ≤ ΔT_RX-TX < 32. RX_TX_TIMEDIFF_ADD_1; 32 ≤ ΔT_RX-TX < 64. RX_TX_TIMEDIFF_ADD_2; 64 ≤ ΔT_RX-TX < 96. …; …. …; …. RX_TX_TIMEDIFF_ADD_491; 15680 ≤ ΔT_RX-TX < 15712. RX_TX_TIMEDIFF_ADD_492; 15712 ≤ ΔT_RX-TX < 15744. RX_TX_TIMEDIFF_ADD_493; ΔT_RX-TX ≥ 15744. |
| --- | --- |

# References

[1] R4-2002328

[2] R4-2003567

[3] R4-2000733

[4] R2-2002245

[5] R4-2003571