3GPP TSG-RAN WG4 Meeting #94bis-e	R4-2003629

Electronic Meeting, April 20 – 30, 2020

Agenda item:	6.17.2.2.2	 [NR_HST-Perf]

Source: 		NTT DOCOMO, INC.

Title: 	Views on NR PRACH for high speed

Document for:	Approval

- Introduction

In RAN4 #94, WF for NR BS high speed was approved [1]. In this contribution, we provide our views on NR PRACH requirements for HST.

2.	Discussion

2.1	Organisation of requirement sections and tables for PRACH

For PRACH, the following options on organisation of HST sections were agreed in [1]:

RAN4 #94 - Organisation of high-speed train requirement sections for PRACH in specifications - 350kph: - Option 1: - Current section for non-HST - New table long format restricted set type A - New table long format restricted set type B - Option 2: - New section for HST - New table format 0 restricted set type A - New table format 0 restricted set type B - Other options not precluded. - 500kph: - Option 1: - Current section for non-HST - Re-use tables short format and add high speed requirements (currently 500kph only). - Option 2: - Current section for non-HST - New tables (per SCS) short format high speed requirements (currently 500kph only). - Option 3: - New section for HST used for 350kph - New tables (per SCS) short format high speed requirements (currently 500kph only). - Other options not precluded

According to TS 38.211, the type of restricted set can be determined by the higher-layer parameter restrictedSetConfig (unrestricted, restricted type A, restricted type B). Restricted sets can be configured for PRACH preamble long formant, but not for short formats as below:

Table 6.3.3.1-1: PRACH preamble formats for  and  kHz.

| Format |  |  |  |  | Support for restricted sets |
| --- | --- | --- | --- | --- | --- |
| 0 | 839 | 1.25 kHz |  |  | Type A, Type B |
| 1 | 839 | 1.25 kHz |  |  | Type A, Type B |
| 2 | 839 | 1.25 kHz |  |  | Type A, Type B |
| 3 | 839 | 5 kHz |  |  | Type A, Type B |

Table 6.3.3.1-2: Preamble formats for  and  kHz where .

| Format |  |  |  |  |  |  | Support for restricted sets |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | 139 | 1151 | 571 |  |  |  | - |
| A2 | 139 | 1151 | 571 |  |  |  | - |
| A3 | 139 | 1151 | 571 |  |  |  | - |
| B1 | 139 | 1151 | 571 |  |  |  | - |
| B2 | 139 | 1151 | 571 |  |  |  | - |
| B3 | 139 | 1151 | 571 |  |  |  | - |
| B4 | 139 | 1151 | 571 |  |  |  | - |
| C0 | 139 | 1151 | 571 |  |  |  | - |
| C2 | 139 | 1151 | 571 |  |  |  |  |

For PRACH, the organization of the long format and short format requirements tables should be considered separately because of the different set of restricted sets.

For long format, requirements for unrestricted set were introduced in Rel.15, and requirements for restricted set type A and B were agreed to be introduced in Rel.16. Since the sequence generation depends on the type of restricted set, separate tables should be introduced for each types of restricted sets (i.e., unrestricted set, restricted set type A and restricted set type B). Note that LTE also has separate tables for normal mode, restricted set type A and B.

For short format, on the other hand, no restricted sets can be configured. The sequence generation does not changed even if high-speed conditions are assumed. Therefore, it is reasonable to merge new HST requirements into the existing short format requirement tables.

Proposal 1:

For PRACH long format, introduce new requirements for restricted set type A and B in new tables in the existing PRACH section.

For PRACH short format, introduce new requirements with large frequency offset condition into the existing short format requirement tables in the existing PRACH section.

Note: For PRACH requirements, do not consider the supported velocity.

2.2	Declaration

For PRACH, the following options on declaration were agreed in [1]:

RAN4 #94 - High speed support declaration for HST PRACH - Option 1: Allow BS to declare support for either 350kph, or 500kph, or both, and to test requirements accordingly. A BS that only declares to support 500kph does not need to pass 350kph test, with long format or other format. A BS that declares to support both 350kph and 500kph needs to test both. - Option 2: Allow BS to declare support for either 350kph, or 500kph, but not both. A BS that declares to support 500kph and passes the tests for 500kph with short format, it can also consider the tests for 350kph with long format as passed (i.e., skip 350kph). - Option 3: Allow BS to declare support for either 350kph, or 500kph, but not both. A BS that declares to support 500kph needs to test with both 500kph and 350kph with long format (i.e., no skipping). - Other options not precluded.

We summarized feasible PUSCH and PRACH HST configuration sets in below table:

Table 1: Feasible configuration sets for PUSCH and PRACH HST

| Supported PUSCH/PRACH SCS | Supported maximum Doppler shift for PUSCH HST | Supported PRACH format | Supported Restricted set type | Maximum frequency offset for PRACH | Feasible configuration sets |
| --- | --- | --- | --- | --- | --- |
| 15kHz | 1340Hz (350km/h) | Short | - | 1740Hz | X |
|  |  | Long | A | 1340Hz | X |
|  |  |  | B | 2334Hz | X |
|  | 1740Hz (500km/h) | Short | - | 1740Hz | X |
|  |  | Long | A | 1340Hz |  |
|  |  |  | B | 2334Hz | X |
| 30kHz | 2334Hz (350km/h) | Short | - | 3334Hz | X |
|  |  | Long | A | 1340Hz |  |
|  |  |  | B | 2334Hz | X |
|  | 3334Hz (500km/h) | Short | - | 3334Hz | X |
|  |  | Long | A | 1340Hz |  |
|  |  |  | B | 2334Hz |  |

According to Table 1, it can be seen that it is feasible to use PRACH short format for both 350km/h and 500km/h scenarios and PRACH long format with restricted set type B for both 350km/h and 500km/h scenarios. In the current options, it is unclear which velocity the format is associated with. If option 1, 2 or 3 are adopted, some configurations sets are not allowed as below examples.

Example case 1:

Based on option 1 or option 3, if a BS that declares support “350km/h”, only long format is tested and short format test is skipped, so it is not allowed to support PRACH short format for HST with 350km/h scenario.

Example case 2:

Based on option 2, if a BS that declares support “500km/h”, the long format test is skipped even if the BS only supports long format, so it is not allowed to support PRACH long format with restricted set type B for HST with 500km/h and 15kHz SCS.

To avoid such a limitation, we should not use velocity based declarations for PRACH, so we disagree with Option 1, 2 and 3. In our views, it is better to declare supported type of restricted set for long format form "unrestricted set", “restricted set type A”, or “restricted set type B”. Note that this declaration is independent from the PUSCH HST declaration since all types of restricted sets can be used for any velocity conditions. In addition, as mentioned in the previous section, the type of restricted set can be configured from “unrestricted”, “restricted type A”, “restricted type B” by the higher-layer parameter restrictedSetConfig.

Proposal 2: PRACH HST declaration is declared independently with PUSCH HST declaration and do not consider supported velocity.

Proposal 3: For long format, introduce a new declaration to allow a BS to declare support for “unrestricted set”, “restricted set type A” or “restricted set type B”.

| Declaration identifier | Declaration | Description | Applicability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H | BS type 1-O |
| D.1xx | PRACH restricted set | Declaration of the supported PRACH restricted set: unrestricted, restricted set type A or restricted set type B. | x | x | x |

For short formats, there is no specific configuration for HST such as restricted set, so no new declaration for short format is needed.

Proposal 4: For short format, no additional declaration is needed.

2.2	Channel model

For channel model, TDL-C channel model for PRACH HST was agreed in RAN4 #92bis [2], but it was changed to FFS in RAN4 #94 [1].

RAN4 #92bis - Channel model - AWGN - TDL-C fading channel, Frequency offset is 400Hz RAN4 #94 - TDLC300-100 propagation conditions for short preamble formats and long preamble formats - Do not introduce TDLC300-100 fading channel with frequency offset of 400Hz requirements for short preamble formats as they are already defined in “normal mode” PRACH. Remove the cases from the simulation result summary template. - FFS on whether not to introduce TDLC300-100 fading channel with frequency offset of 400Hz requirements for long preamble formats.

In the last meeting, the necessity of test cases for TDL-C for short formats were discussed, and for short formats, it was concluded that do not introduce TDL-C for HST since there is no restricted set configuration for short formats and the performance under TDL-C can be confirmed in “normal mode”  PRACH tests. However, for long format, situation is different from short formats. When restricted set type A or B are configured, the sequence generation will be changed as mentioned in section 2.1. If test cases for TDL-C is not introduced for restricted set, the performance of restricted set under Doppler condition is not confirmed. To ensure the performance under Doppler condition, RAN4 should introduce TDL-C for long format.(Keep original agreements.)

Proposal 5: Introduce test cases with TDLC300-100 for PRACH restricted set type A and B (Keep original agreement).

Based on above proposals, the detailed text example is shown below:

------------------------------------------------------------------------------------------------------------------------------------------------

Table 8.4.2.2-1: PRACH missed detection requirements for Normal Mode, 1.25 kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Frequency offset | SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 |
| 1 | 2 | AWGN | 0 | -14.5 |
|  |  | TDLC300-100 Low | 400 Hz | -6.6 |
|  | 4 | AWGN | 0 | -16.7 |
|  |  | TDLC300-100 Low | 400 Hz | -11.9 |
|  | 8 | AWGN | 0 | -18.9 |
|  |  | TDLC300-100 Low | 400 Hz | -15.8 |

Table 8.4.2.2-2: PRACH missed detection requirements for Normal Mode, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Frequency offset | SNR (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format A1 | Burst format A2 | Burst format A3 | Burst format B4 | Burst format C0 | Burst format C2 |
| 1 | 2 | AWGN | 0 | -9.3 | -12.6 | -14.2 | -16.8 | -6.3 | -12.5 |
|  |  | TDLC300-100 Low | 400 Hz | -2.1 | -4.8 | -6.6 | -8.8 | 0.8 | -4.9 |
|  |  | AWGN | 1740 Hz | - | TBD | - | TBD | - | TBD |
|  | 4 | AWGN | 0 | -11.6 | -14.3 | -16.0 | -19.0 | -8.7 | -14.1 |
|  |  | TDLC300-100 Low | 400 Hz | -7.3 | -10.3 | -11.7 | -13.8 | -4.3 | -10.2 |
|  |  | AWGN | 1740 Hz | - | TBD | - | TBD | - | TBD |
|  | 8 | AWGN | 0 | -13.8 | -16.7 | -18.2 | -21.2 | -11.1 | -16.6 |
|  |  | TDLC300-100 Low | 400 Hz | -11.0 | -13.9 | -15.2 | -17.3 | -8.1 | -13.9 |
|  |  | AWGN | 1740 Hz | - | TBD | - | TBD | - | TBD |

Table 8.4.2.2-3: PRACH missed detection requirements for Normal Mode, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Frequency offset | SNR (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format A1 | Burst format A2 | Burst format A3 | Burst format B4 | Burst format C0 | Burst format C2 |
| 1 | 2 | AWGN | 0 | -9.1 | -12.0 | -13.8 | -16.5 | -6.1 | -11.9 |
|  |  | TDLC300-100 Low | 400 Hz | -2.8 | -5.7 | -7.4 | -9.9 | 0.1 | -5.6 |
|  |  | AWGN | 3334 Hz | - | TBD | - | TBD | - | TBD |
|  | 4 | AWGN | 0 | -11.4 | -14.2 | -15.9 | -19.0 | -8.6 | -14.1 |
|  |  | TDLC300-100 Low | 400 Hz | -7.2 | -10.4 | -12.0 | -14.5 | -4.5 | -10.4 |
|  |  | AWGN | 3334 Hz | - | TBD | - | TBD | - | TBD |
|  | 8 | AWGN | 0 | -13.7 | -16.6 | -18.1 | -21.1 | -11.0 | -16.5 |
|  |  | TDLC300-100 Low | 400 Hz | -10.7 | -13.7 | -15.1 | -17.6 | -7.8 | -13.7 |
|  |  | AWGN | 3334 Hz | - | TBD | - | TBD | - | TBD |

Table 8.4.2.2-4: PRACH missed detection requirements for high speed train, restricted set type A, 1.25 kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Frequency offset | SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 |
| 1 | 2 | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | 625 Hz | TBD |
|  |  | AWGN | 1340 Hz | TBD |
|  | 4 | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | 625 Hz | TBD |
|  |  | AWGN | 1340 Hz | TBD |
|  | 8 | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | 625 Hz | TBD |
|  |  | AWGN | 1340 Hz | TBD |

Table 8.4.2.2-5: PRACH missed detection requirements for high speed train, restricted set type B, 1.25 kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Frequency offset | SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 |
| 1 | 2 | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | 625 Hz | TBD |
|  |  | AWGN | 2334 Hz | TBD |
|  | 4 | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | 625 Hz | TBD |
|  |  | AWGN | 2334 Hz | TBD |
|  | 8 | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | 625 Hz | TBD |
|  |  | AWGN | 2334 Hz | TBD |

------------------------------------------------------------------------------------------------------------------------------------------------

3.	Conclusion

In this contribution, we discuss on remaining issues for PUSCH HST. The following proposals are obtained.

Organization of the requirement tables and section

Proposal 1:

For PRACH long format, introduce new requirements for restricted set type A and B in new tables in the existing PRACH section.

For PRACH short format, introduce new requirements with large frequency offset condition into the existing short format requirement tables in the existing PRACH section.

Note: For PRACH requirements, do not consider the supported velocity.

Declaration

Proposal 2: PRACH HST declaration is declared independently with PUSCH HST declaration and do not consider supported velocity.

Proposal 3: For long format, introduce a new declaration to allow a BS to declare support for “unrestricted set”, “restricted set type A” or “restricted set type B”.

| Declaration identifier | Declaration | Description | Applicability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H | BS type 1-O |
| D.1xx | PRACH restricted set | Declaration of the supported PRACH restricted set: unrestricted, restricted set type A or restricted set type B. | x | x | x |

Proposal 4: For short format, no additional declaration is needed.

Channel model

Proposal 5: Introduce test cases with TDLC300-100 for PRACH restricted set type A and B (Keep original agreement).

References

- R4-2002405, “WF on Rel.16 NR HST BS demodulation requirements”, Nokia, RAN4 #94-e, February 2020

- Re-1912729, “Way forward on NR HST PRACH”, Samsung, RAN4 #92bis, October 2019.