3GPP TSG-RAN WG4 Meeting #94-e-Bis		R4-2003830

Electronic Meeting, April 20th – April 30th, 2020

Agenda item:	6.17.2.1.3

Source:	Ericsson

Title:	Simulation results for NR UE HST Single tap

Document for:	Discussion

# Introduction

In this paper we provide simulation results based on the assumptions found in [1].

# Simulation results

## FDD

Figure 1 FDD Throughput curve MCS17

Table 1 SNR at 70% maximum throughput

| MCS | Doppler [Hz] | SNR @ 70% TP mark |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 2Rx |  | 4Rx |  |
|  |  | 1Tx | 2Tx | 1Tx | 2Tx |
| 17 | 875 | 6.41 | 6.41 | 3.41 | 3.41 |
| 17 | 1250 | 6.41 | 6.41 | 3.4 | 3.41 |

From a doppler estimation point of view, we can see that the demodulation performance does not degrade when comparing 1250Hz doppler with 875Hz for FDD test cases.

Observation 1: FDD 15kHz SCS at 1250Hz doppler does not degrade demodulation performance compared to 875Hz doppler.

Proposal 1: We suggest setting 1250Hz doppler for FDD.

## TDD

Figure 2 TDD throughput curve MCS17

Table 2 SNR at 70% maximum throughput

| MCS | Doppler [Hz] | SNR @ 70% TP mark |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 2Rx |  | 4Rx |  |
|  |  | 1Tx | 2Tx | 1Tx | 2Tx |
| 17 | 1667 | 6.53 | 6.55 | 3.53 | 3.55 |

Observation 2: 1Tx antenna does not change demodulation performance significantly compared to 2Tx antennas.

# Conclusions

We provide simulation results for HST single tap for both FDD, and TDD test cases. The SNR at the 70% maximum throughput mark are provided in table 1, and table 2.

Observation 1: FDD 15kHz SCS at 1250Hz doppler does not degrade demodulation performance compared to 875Hz doppler.

Observation 2: 1Tx antenna does not change demodulation performance significantly compared to 2Tx antennas.

Proposal 1: We suggest setting 1250Hz doppler for FDD.

# References

- R4-2002418, “WF on UE demodulation for NR HST”, CMCC