3GPP TSG-RAN WG4 Meeting #94-e- Bis                 	R4-2003897

Online, , 20th Apr 2020 - 30th Apr 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	NR Rel-16 performance requirement enhancement BS demodulation simulation results

Agenda item:	6.18.2.1

Document for:	Discussion

Introduction

In this contribution we deliver our simulation results for 30% TPUT test points for use in the NR_perf_enh BS demodulation specification writing.

We observe that in RAN4#943, there was no simulation collection template for 30% TPUT in NR_perf_enh BS demodulation.

There is currently no simulation collection template for 30% TPUT in NR_perf_enh BS demodulation.

Simulation delivery 30% TPUT

Unless stated otherwise in the following tables, the simulation setup follows the standard PUSCH setup, as captured in [1]. This is a revision and extension of the results, provided by Nokia in [2].

Table 1: Simulation summary 30%TPUT SINR.

| PUSCH 1T2R |  |  |  |  |  | SNR (dB) @ |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal 70% TP | Ideal 30% TP |
| CP-OFDM | FR1 Mappying type A | 15kHz, 10MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.79 | 0.55 |
|  |  | 30kHz, 40MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.58 | 0.49 |
|  | FR1 Mappying type B | 15kHz, 10MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.86 | 0.58 |
|  |  | 30kHz, 40MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.59 | 0.48 |
|  | FR2 with PTRS | 60kHz, 100MHz | MCS16 | TDLA30-300 | DMRS 1 | 8.84 | 0.68 |
|  |  |  |  |  | DMRS 1+1 | 8.16 | 0.7 |
|  |  | 120kHz, 100MHz | MCS16 | TDLA30-300 | DMRS 1 | 7.77 | 0.82 |
|  |  |  |  |  | DMRS 1+1 | 7.67 | 0.76 |
|  | FR2 without PTRS | 60kHz, 100MHz | MCS16 | TDLA30-300 | DMRS 1 | 8.43 | 0.59 |
|  |  |  |  |  | DMRS 1+1 | 7.67 | 0.4 |
|  |  | 120kHz, 100MHz | MCS16 | TDLA30-300 | DMRS 1 | 7.24 | 0.47 |
|  |  |  |  |  | DMRS 1+1 | 7.1 | 0.42 |
| CP-OFDM | FR1 Mappying type A | 15kHz, 5MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.81 | 0.59 |
|  |  | 15kHz, 20MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.79 | 0.45 |
|  |  | 30kHz, 10MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.7 | 0.63 |
|  |  | 30kHz, 20MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.66 | 0.53 |
|  |  | 30kHz, 100MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.58 | 0.49 |
|  | FR1 Mappying type B | 15kHz, 5MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.88 | 0.61 |
|  |  | 15kHz, 20MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.87 | 0.51 |
|  |  | 30kHz, 10MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.72 | 0.62 |
|  |  | 30kHz, 20MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.7 | 0.51 |
|  |  | 30kHz, 100MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.58 | 0.49 |
|  | FR2 with PTRS | 60kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1 | 8.59 | 0.92 |
|  |  |  |  |  | DMRS 1+1 | 8.14 | 0.81 |
|  |  | 120kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1 | 7.8 | 1.14 |
|  |  |  |  |  | DMRS 1+1 | 7.48 | 0.96 |
|  |  | 120kHz, 200MHz | MCS16 | TDLA30-300 | DMRS 1 | 7.99 | 0.83 |
|  |  |  |  |  | DMRS 1+1 | 7.76 | 0.72 |
|  | FR2 without PTRS | 60kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1 | 8.34 | 0.58 |
|  |  |  |  |  | DMRS 1+1 | 7.63 | 0.46 |
|  |  | 120kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1 | 7.19 | 0.62 |
|  |  |  |  |  | DMRS 1+1 | 6.87 | 0.47 |
|  |  | 120kHz, 200MHz | MCS16 | TDLA30-300 | DMRS 1 | 7.45 | 0.53 |
|  |  |  |  |  | DMRS 1+1 | 7.21 | 0.43 |

Conclusion

In this contribution we have delivered our simulation results for 30% TPUT test points.
We have made the following observation:

- There is currently no simulation collection template for 30% TPUT in NR_perf_enh BS demodulation.

- References

- R4-1907241, Way forward on NR PUSCH demodulation requirements, Nokia, Nokia Shanghai Bell, RAN4#91.

- R4-1911196, NR Rel-16 performance requirement enhancement BS demodulation simulation results, Nokia, Nokia Shanghai Bell, RAN4#92-Bis.