3GPP TSG-RAN WG4 Meeting # 94-e-Bis	R4-2004004

Electronic Meeting, 20 – 30 Apr., 2020

Agenda item:			6.17.2.1.2

Source:				　NTT DOCOMO, INC.

Title:		Views on HST-SFN

Document for:	Discussion

1. Introduction

At the previous meeting, we agreed to start dissolution about release independent for UE demodulation and CSI reporting requirements [1]. This contribution presents our views for HST-SFN.

2. Discussion

2.1. Target maximum Doppler frequency

At the last RAN4 meeting, we agreed options on maximum Doppler shift values as follows.

- Maximum Doppler frequency - For TDD 30 KHz SCS, 500km/h - Option 1: 1667Hz - Option 2: 1500Hz - For FDD 15 KHz SCS, 500km/h - Option 1: 851Hz - Option 2: 870Hz

From our analysis for FDD 15 kHz [2], even taking into account the UE oscillator error, 1667Hz is well covered.

Observation 1: For FDD 15 kHz, 1667 Hz can be compensated.

Proposal 1: Target Doppler frequency in the HST-SFN test as follows.

- Maximum Doppler frequency

  - For TDD 30 KHz SCS, 500km/h

    - Option 1: 1667Hz

  - For FDD 15 KHz SCS, 500km/h

    - Option 2: 870Hz

2.2. Target speed of 350km/h

RAN4 made the agreements to introduce 500km/h requirements and it is motivated, e.g., for maglev trains. Such use cases for 500km/h are very limited but extremely important for some countries. On the other hand, we consider that 350km/h requirements is also important and it would include more common use cases, e.g. high-speed railway. These two scenarios are focused on different use cases and required MCS level would be different as well. In the sense, we prefer to introduce 350km/h requirement in addition to one for 500km/h.

Proposal 2: Introduce the requirement for 350 km/h

3. Conclusion

In this contribution, we present our views on HST-SFN. Observation and proposals are presented as follows.

Observation 1: For FDD 15 kHz, 1667 Hz can be compensated.

Proposal 1: Target Doppler frequency in the HST-SFN test as follows.

- Maximum Doppler frequency

  - For TDD 30 KHz SCS, 500km/h

    - Option 1: 1667Hz

  - For FDD 15 KHz SCS, 500km/h

    - Option 2: 870Hz

Proposal 2: Introduce the requirement for 350 km/h

References

- R4-2002418 , CMCC , “WF on UE demodulation for NR HST,” Mar. 2020

- R4- 2000949, NTT DOCOMO, “Views on HST-SFN for NR”, Mar. 2020