3GPP TSG-RAN WG4 Meeting # 94-e-Bis	R4-2004009

Electronic Meeting, 20 – 30 Apr., 2020

Agenda item:			6.18.1.4

Source:				     NTT DOCOMO, INC.

Title:		Views on power imbalance requirement

Document for:	Discussion

- Introduction

At the last RAN4 meeting, we discussed power imbalance requirements and RAN4 agreed a WF [1]. Based on this WF, the objectives in WID was revised in the last RAN meeting [2]. This contribution presents our views on power imbalance requirement.

- Discussion

  - FR1 intra-band contiguous CA

In RAN4 94-e meeting, we agreed some options on CBW and PDSCH RB allocation as follows.

- CBW - Option 1: Specify the following CA configurations. FFS necessity of further down selection - Option 1A: 50+60, 50+80, 50+100, 60+60, 60+80, 60+100, 80+80 and 80+100 MHz - Other options are not precluded - Further discuss after Rel-16 core spec is finalized. - Option 2: Define requirements for 5+5 MHz bandwidth for FDD+FDD CA, 10+10 MHz bandwidth for TDD+TDD CA, with the following test applicability - The test is done for any one of the supported bandwidth combination, by using performance requirement for 5+5 MHz FDD+FDD CA or 10+10 MHz TDD+TDD CA. - The tested PRBs shall be placed in the highest part for the CC with lower carrier frequency, and placed in the lowest part for the CC with higher carrier frequency. - Option 3: Define generic methodology for selection of CBW combination among all CBW combinations in supported CA configurations - Other options are not precluded - PDSCH RB allocation - Option 1: Full allocation - Option 2: 25 PRBs for 15kHz FDD, 24 PRBs for 30kHz TDD - There are 25 PRBs for 5MHz CBW with 15kHz SCS, and 24 PRBs for 10MHz CBW with 30kHz SCS - Other options are not precluded

Channel bandwidth combination (BCS) for intra-band NR CA with contiguous 2CCs are specified in TS 38.101-1 [3]. Since there are multiple CBW combinations per 1 CA configuration, it is difficult to choose a few generic CBW combination for such test. Based on that, Option 2 for both CBW and PDSCH RB allocation is reasonable

Proposal 1:

Define requirements for 5+5 MHz bandwidth for FDD+FDD CA, 10+10 MHz bandwidth for TDD+TDD CA, with the following test applicability

- The test is done for any one of the supported bandwidth combination, by using performance requirement for 5+5 MHz FDD+FDD CA or 10+10 MHz TDD+TDD CA.-

- The tested PRBs shall be placed in the highest part for the CC with lower carrier frequency, and placed in the lowest part for the CC with higher carrier frequency.

Proposal 2:

25 PRBs for 15kHz FDD, 24 PRBs for 30kHz TDD

- There are 25 PRBs for 5MHz CBW with 15kHz SCS, and 24 PRBs for 10MHz CBW with 30kHz SCS

In addition, there are some remaining topics, e.g., TDD pattern, PDCCH allocation and MIMO configuration. Regarding TDD pattern and PDCCH allocation, 7D1S2U and Symbol #0 and #1 are applied to most of Rel.15 UE demodulation requirements respectively. On the other hand, shifted 7D1S2U pattern, i.e., DDDSUDDDD is also introduced for a few UE demodulation requirements. We slightly prefer shifted 7D1S2U to expand the scope of it. Regarding MIMO configuration, we prefer 2x2 /2x4 since it is more realistic configuration than 1x2 /1x4.

Proposal 3: For power imbalance test for intra-band contiguous NR-CA, the following test parameters are applied.

| Parameters | Value |
| --- | --- |
| TDD pattern | 1st priority: Option 3: DDDSUUDDDD (S=6d4s4u) 2nd priority: Option 1: 7D1S2U(S=6d4s4u) |
| PDCCH allocation | Symbol #0 and #1 |
| MIMO configuration | 2x2 / 2x4 MIMO |

  - FR1 intra-band contiguous and non-contiguous EN-DC

From Rel-15, some band combinations for intra-band EN-DC were introduced, e.g. DC_3_n3, DC_41_n41, DC_42_n78. Note that the requirements of intra-band EN-DC are applied for DC_42_n78 since LTE band 42 (3.4-3.6GHz) is completely contained in the NR band n78 (3.3-3.8GHz). There are some original features and assumptions on intra-band EN-DC in Rel-15 as follows:

- Only co-located scenario is considered for both contiguous and non-contiguous cases.

- UE capability was introduced to inform whether UE supports only non-contiguous case or both cases, i.e., intraBandENDC-Support [4] [5]. It means that UE always supports intra-band non-contiguous EN-DC if UE supports such band combinations for intra-band EN-DC.

intraBandENDC-Support               ENUMERATED {non-contiguous, both}   OPTIONAL,

- Regarding DC_42_n78, RAN4 agreed that non-contiguous EN-DC shall be supported as mandatory in November meeting [6]. It is the same situation as other intra-band EN-DC band combinations.

Observation 1: There are some original features and assumptions on intra-band EN-DC in Rel-15 as follows:

- Only co-located scenario is considered for both contiguous and non-contiguous cases.

- UE capability was introduced to inform whether UE supports only non-contiguous case or both cases, i.e., intraBandENDC-Support. It means that UE always supports intra-band non-contiguous EN-DC if UE supports such band combinations for intra-band EN-DC.

- Regarding DC_42_n78, RAN4 agreed that non-contiguous EN-DC shall be supported as mandatory in November meeting. It is the same situation as other intra-band EN-DC band combinations.

Although only co-located scenario is assumed for intra-band EN-DC as mentioned in observation 1, the reception power of LTE and NR would be different according to deployment scenario. Roughly speaking, the following three deployment scenarios could be considered for intra-band EN-DC (Figure 1):

- Scenario #1: LTE and NR have different antenna/beam pattern, e,g., advanced beamforming technology for NR

- Scenario #2: LTE and NR have same antenna pattern, but the transmission precoding matrix (TPMI) is different.

- Scenario #3: LTE and NR have same antenna pattern and transmission precoding matrix by strict coordination of LTE/NR schedulers at eNB/gNB

The same beam pattern and tight scheduler coordination such as Scenario #3 would be ideal from the system performance perspective, however non-ideal scenarios such as Scenario #1 and #2 should be also allowed in practical. In the Rel-15 specification, NSA scenario was firstly specified as early drop and this enables that NR cells are deployed into the existing LTE network and smooth migration from LTE to NR in the future. Since it is not likely that the NR equipment in the initial deployment are totally renewed when NSA are migrated to SA, such advanced beamforming technology in Scenario #1 and #2 would be introduced from the initial deployment of NSA. In addition, non-scheduler coordination between LTE and NR is one of practical solution from the view point of minimization of LTE development cost. Therefore, Scenario #1 and #2 are possible scenarios in the future.

Figure 1: Deployment scenario for intra-band EN-DC

The reception power of LTE and NR would be the similar in the scenario #3, however it would be different for other scenarios. Especially, the scenario #1 would be the worst case from power imbalance perspective. We submitted the detailed analysis considering worst case in RAN4 94-e [7]. Based on the background mentioned above, it is important to introduce the requirements for FR1 intra-band EN-DC with power imbalance between LTE and NR carrier.

Observation 2: For intra-band EN-DC scenario in FR1, UE may suffer significant power imbalance between LTE and NR carriers even in co-located scenario if beam pattern is different between LTE and NR.

Proposal 4: Power imbalance requirement should be introduced to ensure correct UE implementation for intra-band EN-DC scenario in FR1.

In addition, we discussed which contiguous or non-contiguous EN-DC should be introduced in RAN4#94-e meeting. Our view is that both cases are needed since non-contiguous EN-DC is mandatory as mentioned in observation 1 and contiguous EN-DC is also possible scenario. Regarding contiguous EN-DC case, since it is very similar situation as CA case, which has already been agreed to introduce, test parameters including 6dB power imbalance value can be reused. Regarding non-contiguous EN-DC case, test parameters could be also reused from contiguous CA case except for power imbalance value. The value of power imbalance for non-contiguous EN-DC would be larger than 6dB according to our analysis, however we can take 6dB for compromise considering the remaining time budget for this WI.

Proposal 5: Both contiguous and non-contiguous EN-DC cases should be introduced for power imbalance requirement.

Proposal 6: Power imbalance value for non-contiguous EN-DC is 6dB.

On the other hand, UE does not always need to perform both tests even if both tests are introduced. Our preference is to introduce test applicability rule for these tests according to existing UE capability, i.e., intraBandENDC-Support, as follows:

- UE shall test power imbalance test for intra-band non-contiguous EN-DC as mandatory

- If UE supports intra-band contiguous EN-DC and UE inform “both” in intraBandENDC-Support, UE also tests power imbalance test for intra-band contiguous EN-DC.

Proposal 7: UE should be tested according to the following applicability rules.

- UE shall test power imbalance test for intra-band non-contiguous EN-DC as mandatory

- If UE supports intra-band contiguous EN-DC and UE inform “both” in intraBandENDC-Support, UE also tests power imbalance test for intra-band contiguous EN-DC.

For information, the intra-band contiguous and non-contiguous Rx image was already agreed at 25dB in [8] and [9], respectively. Therefore, we has already readied for the discussion.

Observation 3: Rx image is 25dB for both intra-band contiguous and non-contiguous.

3. Conclusion

In this contribution, we present our views power imbalance requirements. Observation and proposals are presented as follows.

Proposal 1:

Define requirements for 5+5 MHz bandwidth for FDD+FDD CA, 10+10 MHz bandwidth for TDD+TDD CA, with the following test applicability

- The test is done for any one of the supported bandwidth combination, by using performance requirement for 5+5 MHz FDD+FDD CA or 10+10 MHz TDD+TDD CA.-

- The tested PRBs shall be placed in the highest part for the CC with lower carrier frequency, and placed in the lowest part for the CC with higher carrier frequency.

Proposal 2:

25 PRBs for 15kHz FDD, 24 PRBs for 30kHz TDD

- There are 25 PRBs for 5MHz CBW with 15kHz SCS, and 24 PRBs for 10MHz CBW with 30kHz SCS

Proposal 3: For power imbalance test for intra-band contiguous NR-CA, the following test parameters are applied.

| Parameters | Value |
| --- | --- |
| TDD pattern | 1st priority: Option 3: DDDSUUDDDD (S=6d4s4u) 2nd priority: Option 1: 7D1S2U(S=6d4s4u) |
| PDCCH allocation | Symbol #0 and #1 |
| MIMO configuration | 2x2 / 2x4 MIMO |

Observation 1: There are some original features and assumptions on intra-band EN-DC in Rel-15 as follows:

- Only co-located scenario is considered for both contiguous and non-contiguous cases.

- UE capability was introduced to inform whether UE supports only non-contiguous case or both cases, i.e., intraBandENDC-Support. It means that UE always supports intra-band non-contiguous EN-DC if UE supports such band combinations for intra-band EN-DC.

- Regarding DC_42_n78, RAN4 agreed that non-contiguous EN-DC shall be supported as mandatory in November meeting. It is the same situation as other intra-band EN-DC band combinations.

Observation 2: For intra-band EN-DC scenario in FR1, UE may suffer significant power imbalance between LTE and NR carriers even in co-located scenario if beam pattern is different between LTE and NR.

Proposal 4: Power imbalance requirement should be introduced to ensure correct UE implementation for intra-band EN-DC scenario in FR1.

Proposal 5: Both contiguous and non-contiguous EN-DC cases should be introduced for power imbalance requirement.

Proposal 6: Power imbalance value for non-contiguous EN-DC is 6dB.

Proposal 7: UE should be tested according to the following applicability rules.

- UE shall test power imbalance test for intra-band non-contiguous EN-DC as mandatory

- If UE supports intra-band contiguous EN-DC and UE inform “both” in intraBandENDC-Support, UE also tests power imbalance test for intra-band contiguous EN-DC.

Observation 3: Rx image is 25dB for both intra-band contiguous and non-contiguous.

References

- R4-2002396, NTT DOCOMO, Way forward on UE  FR1 CA power imbalance requirements, RAN4 #94-e

- RP-200472, China Telecom, Revised WID on NR performance requirement enhancement, RAN #87-e

- TS 38.101-1

- TS38.306

- TS38.331

- R4-1913130, Qualcomm, LS on UE capability of intraBandENDC-Support, RAN4#93

- R4-2000953, NTT DOCOMO, Further discussion on power imbalance requirement for intra-band EN-DC/NR CA, RAN4 #94-e

- R4-1916135, NTT DOCOMO, WF on Rx image rejection for FR1 intra-band contiguous CA/EN-DC, RAN4#93

- R4-2002110, NTT DOCOMO, Clarification on Rx image assumption for intra-band non-contiguous EN-DC, RAN4 #94-e