TSG-RAN Working Group 4 (Radio) meeting #94bis-E	R4-2004040

Electronic Meeting, April 20th – 30th 2020

Source:	Ericsson

Title:	TP to TR 37.941: MU budget format for RC

Agenda item:	6.19.4

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#94-E) a draft version of TR 37.941 was created [1]. The information in this version was coming from TR 37.840, TR 37.842, TR 37.843 and TR 38.817-02.

Unfortunately, the information in legacy TRs was not complete. The error source list relevant for RC was missing.

In this contribution we have created a test proposal with additional information to complete the MU evaluation description for RC. At the of this contribution a text proposal is attached and presented for approval.

# Discussion

In the text proposal a table with all relevant error source for RC have been created.

# Conclusion

The attached text proposal is presented for approval.

# References

[1]	R4-2002430, “Draft TR 37.941”, Huawei

TEXT PROPOSAL:

### 11.2.5	Reverberation chamber

#### 11.2.5.1	Measurement system description

Measurement system description is captured in subclause 7.7.

#### 11.2.5.2	Test procedure

##### 11.2.5.2.1	Stage 1: Calibration

Calibration procedure for the Reverberation chamber is captured in subclause 8.7.

##### 11.2.5.2.2	Stage 2: BS measurement

TRP calculation in an RC does not rely on a coordinate system and as such no reference coordinate system is defined for the RC method, and no alignment between BS and test system is required. The BS should be placed/mounted in the working volume of the RC. It is advised (but not mandatory) to orient the BS is such a way that its antenna boresight is pointing towards a well-stirred part of the chamber (such as towards the stirrer).

The testing procedure consists of the following steps:

1. Calculate Ambient Power Level:

1)	Connect the RX antenna to a calibrated receiver test equipment using the same cables as in the calibration step. Turn on the BS control. Keep the BS RF power off. Terminate the REF TX ant in a 50Ω load. See figure 8.7-1 (b).

2)	Measure the voltage data Uamb and calculate the ambient power level as Pamb=Uamb250

3)	Calculate the ambient TRP level as TRPamb=ηM1rPrPamb

2. Calculate BS TRP:

1)	Turn on the BS RF power and measure the received voltage UBS, see figure 8.7-1 (c).

2)	Calculate the BS power as:PEUT=UEUT250
PBS=UBS250

3)	Calculate the TRP from the BS as
TRPBS=ηM1(r)P(r)PBS

4)	The following tests shall be performed on the measurement data:

-	The dynamic range TRPEUTTRPambTRPBS / TRPamb must be at least 20 dB.

-	The number of uncorrelated samples, calculated via the auto-correlation function, see [35], shall be at least 250.

There is no need for additional directional measurements to the stirring cycle. In a well-stirred chamber, all directional components will be contained in the samples collected during the measurement cycle and accounted for correctly in the final TRP calculation.

NOTE:	A reverberation chamber does not represent a real-life deployment for the BS and can be considered as a hostile environment due to the potentially high field strengths. Therefore, it is important to make sure that the BS is operating properly throughout the entire measurement.

#### 11.2.5.3	Uncertainty budget format

Table 11.2.5.3-1: Reverberation uncertainty contributions
for TRP measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Uncertainty of the measurement equipment | C1-1 |
| 2 | Impedance mismatch in the receiving chain | A6-1 |
| 3 | Random uncertainty | A6-2 |
| Stage 1: Calibration measurement |  |  |
| 4 | Reference antenna radiation efficiency | A6-3 |
| 5 | Mean value estimation of reference antenna radiation efficiency | A6-4 |
| 6 | Uncertainty of the Network Analyzer | C1-3 |
| 7 | Influence of the reference antenna feed cable | A6-5 |
| 8 | Mean value estimation of transfer function | A6-6 |
| 9 | Uniformity of transfer function | A6-7 |

#### 11.2.5.4	MU value derivation, FR1

Table 11.2.5.4-1 captures derivation of the expanded measurement uncertainty values for OTA BS output power measurements in Reverberation Chamber (Normal test conditions, FR1).

Table 11.2.5.4-1: Reverberation chamber MU value derivation for OTA BS output power, FR1

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.

#### 11.2.5.5	MU value derivation, FR2

Table 11.2.5.5-1 captures derivation of the expanded measurement uncertainty values for OTA BS output power measurements in Reverberation Chamber (Normal test conditions, FR2).

Table 11.2.5.5-1: Reverberation chamber MU value derivation for OTA BS output power, FR2

Editor’s note: placeholder for the MU table based on the Excel spreadsheet.