3GPP TSG-RAN WG4 Meeting #94bis-e			  											R4-2004228

Electronic Meeting, 20th Apr. 2020 – 24th Apr. 2020

Source: 	Nokia, Optus

Title: 		TP for 37.716-21-21 to introduce DC_1-3_n40-n78

Agenda Item: 			8.7.2 [DC_R16_xBLTE_2BNR_yDL2UL-Core]

Document for:	Agreement

# 1	Introduction

This contribution is a TP for TR 37.716-21-21 to introduce DC_1-3_n40-n78.

# 2	Text Proposal

*********************** Start of the TP ***************************************************

## 7.x	DC_1-3_n40-n78

### 7.x.1	Operating bands for DC

Table 7.x.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
|  | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.x.2	Co-existence studies

Co-existence studies are captured in TR 38.716-02-00, TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

### 7.x.3	∆TIB and ∆RIB values

The TIB,c and RIB,c values are given in the tables below based on lower order combinations including the same bands.

Table 7.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n40-n78 | 1 | 0.5 |
|  | 3 | 0.6 |
|  | n40 | 0.36 |
|  | n78 | 0.86 |
| NOTE 6: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 7.x.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3_n40-n78 | 1 | 0 |
|  | 3 | 0.2 |
|  | n40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

### 7.x.4	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

*********************** End of the TP ***************************************************

# 3	References