3GPP TSG-RAN WG4 Meeting # 94-e-Bis 	R4-2004268

# Electronic Meeting, 20 – 30 Apr., 2020

Agenda item:	6.1.5.9

Source: 	Huawei, HiSilicon

Title: 	Discussion on BWP switch for NR-U

Document for:	Discussion

- Introduction

In last meeting RAN4 94e meeting, the RRM measurement for NR-U were discussed [1]. It was agreed to introduce the BWP switch triggered by consistent UL LBT failure. We further present our views about this topic.

- Discussion

The consistent UL LBT failure recovery procedure was introduced in RAN2 107bis meeting. It could be observed that it is an important feature to handle the consistent UL LBT failure, hence we proposed to define the corresponding requirement for the BWP switch in our previous contribution [2]. In the last RAN4#94e meeting, it was agreed to introduce UL BWP switch requirements (delay and interruption) triggered by consistent UL LBT failures [1], and the exact requirements are still open.

- Introduce UL BWP switch requirements (delay and interruption) triggered by consistent UL LBT failures - UL BWP switch requirements triggered by consistent UL LBT failures - Option 1: Follow DCI and timer-based BWP switch - Option 2: FFS

From our understanding, the requirements of UL BWP switch triggered by consistent UL LBT failures shall follow DCI and timer-based BWP switch.

Proposal 1: The requirements of UL BWP switch triggered by consistent UL LBT failures shall follow DCI and timer-based BWP switch.

For the delay requirements, the introduction of the UL BWP switch is added in our accompanied CR [3] for the active BWP switch as a new subsection 8.6.4, and the applicability is explicitly stated at 8.6.1. Also updating of interruption requirements of BWP switch is necessary. We provide the interruption requirements caused by UL BWP switch of consistent LBT failure for both EN-DC and SA section in TS 38.133 in our accompanied CR [4]. The details could be found in the individual files.

- Conclusion

Proposal 1: The requirements of UL BWP switch triggered by consistent UL LBT failures shall follow DCI and timer-based BWP switch.

References

[1] R4-2002335 WF on NR-U RRM Requirements (Part 2), MediaTek inc

[2] R4-1914874 Discussion on Active BWP switching requirements in NR-U, Huawei, HiSilicon

[3] R4-2004263

[4] R4-2004261