3GPP TSG-RAN WG4 Meeting # 94-e-Bis 	R4-2004487

# Electronic Meeting, 20 – 30 Apr., 2020

Agenda item:	6.5.1.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on R16 IAB MT RF features

Document for:	Approval

- Introduction

RAN#87e has briefly discussed how to handle IAB capabilities and some general agreements have been made:

1. RAN WGs to investigate which of the mandatory Rel-15 UE features (as defined in TR 38.822) can be optional for basic operation of (and if found useful, for different classes of IAB-MTs as defined by RAN4). 2. RAN WGs should strive to minimize specification impact.

In this contribution we presents our views on MT features from RF perspective.

- Discussion

Before going to detailed discussion on whether a specific feature should be mandatory or not, we should first understand the motivation of this discussion. As we know, the existing UE capabilities as captured in TS 38.306/38.331 and TR 38.822 were designed for UEs which are equipped in devices like smartphones, and some features are only useful for mobile phones. When we discussing IAB which is a network node, we agreed previously that IAB-MT can follow the legacy procedure and signalling defined in Rel-15, but some of features which are mandatory for legacy UEs may not be useful for IAB-MTs. Implementing and testing this kind of mandatory features on IAB would require a lot of efforts for vendors and it would delay the commercialization of IAB.

Observation 1: The motivation of RAN to revisit Rel-15 mandatory features is to minimize the mandatory features for IAB-MT to support in order to speed up IAB industrialization and commercialization.

With this motivation beard in mind, we look through the features listed in TR 38.822 clause 4.3, which are divided into 3 groups: System parameter, UE RF and Baseband.

System parameters

For system parameter group, 64QAM for PUSCH and 64QAM for FR2 PDSCH are the parameters which are mandatory without capability signalling and pi/2-BPSK for PUCCH is mandatory with capability signalling for FR2. Normally IAB node is expected to deploy in good channel condition to achieve high spectrum efficiency. High order modulation is desired for IAB MT. There is no need to mandate some low order modulation for IAB. The same approach as DU should be adopted that the vendor declares the modulation scheme. The support of subcarrier spacing shares similar situation. IAB DU shall support at least one SS SCS that can be configured for a band and declare the support of other SCS. The IAB MT could have the same capability as IAB DU hence it is unnecessary to mandate the capability on support of SCS.

Observation 2: The modulation scheme and the support of SCS can be declared by the vendors

RF parameters

For RF related parameters, mandatory-without-capability features are multiple frequency band indication and multiple NS/P-Max. The two feature are applicable for the roaming UE. For IAB MT it is not necessary to mandate the capability since it is deployed by the operator and the requirements should be known beforehand. And for channel bandwidths the agreements had been reached in previous meeting that the channel bandwidth is a declared parameter.

Observation 3: Multiple frequency band indication and Multiple NS/P-Max may not applicable to IAB MT or at least it should be not be mandatory without capability.

Baseband Parameters

For baseband parameters, all the features are optional with/without capability signaling which are used for performance improvements.

Based on the observations above, from RF perspective, we think IAB should be treated as a network node. There is no need to mandate the capability which might only applicable to mobile phones.

Proposal: no mandatory feature is needed for IAB MT from RF perspective

- Conclusion

Proposal: no mandatory feature is needed for IAB MT from RF perspective

References

[1] TR 38.822