3GPP TSG-RAN WG4 Meeting #94-e-bis	R4-2004522

# Online, 20 – 30 April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | <CR#> | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on 36.133 LTE conditional handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMob-Core |  |  |  |  |  | Date: |  |  | 2020-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Capture the agreement in RAN4#94-e meeting |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove CHO in section 5.1.2.1 and add the clarification in the section 5.1.1 for E-UTRAN handover introduction - Correct reference and add inter-F cases in section 5.1.2.6 - Add CHO requirements for TDD-TDD in section 5.1.2.9 - Update the reference to TDD requirements in section 5.1.2.7 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements are not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1, 5.1.2.1, 5.1.2.6, 5.1.2.7, 5.1.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of change 1>>

## 5.1	E-UTRAN Handover

### 5.1.1	Introduction

The requirements in this clause are applicable to handover and conditional handover.

### 5.1.2	Requirements

#### 5.1.2.1	E-UTRAN FDD – FDD

The requirements in this clause are applicable to both intra-frequency and inter-frequency handovers.

<<End of change 1>>

<<Start of change 2>>

#### 5.1.2.6	E-UTRAN FDD – FDD conditional handover

The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handovers.

##### 5.1.2.6.1	Handover delay

Procedure delays for all procedures that can command a conditional handover are specified in TS 36.331 [2].

When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command.

Where:

Dhandover equals the maximum RRC procedure delay to be defined in clause 11.2 in TS 36.331 [2] plus the measurements time stated in clause 5.1.2.6.2 plus the conditional execution preparation time in clause 5.1.2.6.3 plus the interruption time stated in clause 5.1.2.6.4.

##### 5.1.2.6.2	Measurement time

The measurement time delay is defined as the time between the RRC procedure delay and the point when the UE executes a handover to a target cell and interruption time starts.

The measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_intradefined in clause 8.1.2.2.1 for intra-frequency conditional handover or Tidentify_inter defined in clause 8.1.2.3.1 for inter-frequency conditional handover. When TTT or L3 filtering is used an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_intra defined in clause 8.1.2.2.1 or Tidentify_inter defined in clause 8.1.2.3.1 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers a handover, the measurement time delay shall be less than TMeasurement_Period, Intra or TMeasurement_Period, Inter provided the timing to that cell has not changed more than ± 50 Ts and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured or the UE is performing reception and/or transmission for ProSe Direct Discovery and/or ProSe Direct Communication, or the UE is configured to perform SRS carrier based switching, an additional delay can be expected.

##### 5.1.2.6.3	Preparation time

Preparation time is the time from when after measurement time and the time during which the UE prepares the target cell for conditional handover.

TCHO_execution is the time needed for preparing the conditional handover to the target cell.

TCHO_execution can be up [FFS].

##### 5.1.2.6.4	Interruption time

The interruption time is the time between when the conditional handover execution is triggered in the UE and the time when the UE executes the conditional handover to the target cell.

For intra-frequency or inter-frequency conditional handover, the interruption time shall be less than

Tinterrupt = TIU + 20 ms

Where:

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to 30 ms.

NOTE:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in Clause 8.1.2.2.1 for intra-frequency conditional handover and Clause 8.1.2.3.1 for inter-frequency conditional handover.

#### 5.1.2.7	E-UTRAN FDD – TDD conditional handover

The requirements in this clause are applicable to conditional handover from FDD to TDD. The requirements in this clause shall apply to UE supporting FDD and TDD.

The requirements in clause 5.1.2.9 apply for this section.

#### 5.1.2.8	E-UTRAN TDD – FDD conditional handover

The requirements in this clause are applicable to conditional handover from TDD to FDD. The requirements in this clause shall apply to UE supporting FDD and TDD.

The requirements in clause 5.1.2.6 apply for this section.

#### 5.1.2.9	E-UTRAN TDD – TDD conditional handover

The requirements in this clause are applicable to handover from TDD to TDD. The requirements in this clause shall apply to UE supporting TDD.

The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handovers.

##### 5.1.2.9.1	Handover delay

Procedure delays for all procedures that can command a conditional handover are specified in TS 36.331 [2].

When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command.

Where:

Dhandover equals the maximum RRC procedure delay to be defined in clause 11.2 in TS 36.331 [2] plus the measurements time stated in clause 5.1.2.9.2 plus the conditional execution preparation time in clause 5.1.2.9.3 plus the interruption time stated in clause 5.1.2.9.4.

##### 5.1.2.9.2	Measurement time

The measurement time delay is defined as the time between the RRC procedure delay and the point when the UE executes a handover to a target cell and interruption time starts.

The measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_intra defined in clause 8.1.2.2.2 for intra-frequency conditional handover or Tidentify_inter defined in clause 8.1.2.3.2 for inter-frequency conditional handover. When TTT or L3 filtering is used an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_intra defined in clause 8.1.2.2.2 or Tidentify_inter defined in clause 8.1.2.3.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers a handover, the measurement time delay shall be less than TMeasurement_Period, Intra or TMeasurement_Period, Inter provided the timing to that cell has not changed more than ± 50 Ts and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured or the UE is performing reception and/or transmission for ProSe Direct Discovery and/or ProSe Direct Communication, or the UE is configured to perform SRS carrier based switching, an additional delay can be expected.

##### 5.1.2.9.3	Preparation time

Preparation time is the time from when after measurement time and the time during which the UE prepares the target cell for conditional handover.

TCHO_execution is the time needed for preparing the conditional handover to the target cell.

TCHO_execution can be up [FFS].

##### 5.1.2.9.4	Interruption time

The interruption time is the time between when the conditional handover execution is triggered in the UE and the time when the UE executes the conditional handover to the target cell.

For intra-frequency or inter-frequency conditional handover, the interruption time shall be less than

Tinterrupt = TIU + 20 ms

Where:

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to 30 ms.

NOTE:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in Clause 8.1.2.2.2 for intra-frequency conditional handover and Clause 8.1.2.3.2 for inter-frequency conditional handover.

<<End of change 2>>