3GPP TSG-RAN WG4 Meeting #94bis-e 	R4-2004537

Online, 20 Apr - 01 May 2020

Source: 	Huawei

Title: 	TP to TR 37.941 - Spurious emissions MU value derivation sub-clause 12 update (FR1)

Agenda Item:	6.19.5

Document for:	Approval

# Introduction

This TP updates the MU value derivation sections for the FR1 out of band spurious emissions requirements.

It uses the pre-approved tables agreed in R4-2004527.

# TP to TR 39.941 v1.0.0

--- Start of changes ---

#### 12.2.2.3	MU value derivation, FR1

For FR1 a general directional chamber and reverberation chamber was analysed for the MU derivation. Table 12.2.2.3-1 captures derivation of the expanded measurement uncertainty values for OTA TX spurious emissions measurements in general directional chamber (Normal test conditions, FR1).

Table 12.2.2.3-1: General directional chamber MU value derivation for the TX spurious emissions, FR1

| UID | Uncertainty source | Uncertainty value |  |  | Distribution of the probability |  | Divisor based on distribution shape |  | ci |  |  | Standard uncertainty ui [dB] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 30MHz<f≤6 GHz | 6GHz<f ≤19GHz | 19GHz<f ≤26GHz |  |  |  |  |  |  |  |  | 30MHz<f≤6 GHz |  |  |  | 6GHz<f ≤19GHz |  | 19GHz<f ≤26GHz |  |  |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A5-1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | 0.03 |  | Rectangular |  | 1.73 |  |  | 1 |  | 0.02 |  |  |  | 0.02 |  | 0.02 |  |  |
| A5-2 | Pointing misalignment between the AAS BS and the receiving antenna | 0.00 | 0.00 | 0.00 |  | Rectangular |  | 1.73 |  |  | 1 |  | 0.00 |  |  |  | 0.00 |  | 0.00 |  |  |
| A5-3 | Quality of quiet zone | 0.10 | 0.10 | 0.10 |  | Gaussian |  | 1.00 |  |  | 1 |  | 0.10 |  |  |  | 0.10 |  | 0.10 |  |  |
| A5-4a | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | 0.01 |  | Rectangular |  | 1.73 |  |  | 1 |  | 0.01 |  |  |  | 0.01 |  | 0.01 |  |  |
| A5-5a | Mutual coupling between the AAS BS and the receiving antenna | 0.00 | 0.00 | 0.00 |  | Rectangular |  | 1.73 |  |  | 1 |  | 0.00 |  |  |  | 0.00 |  | 0.00 |  |  |
| A5-6a | Phase curvature | 0.05 | 0.05 | 0.05 |  | Gaussian |  | 1.00 |  |  | 1 |  | 0.05 |  |  |  | 0.05 |  | 0.05 |  |  |
| C3-3 | Transmitter mandatory spurious emissions - Conducted Uncertainty (minus miss match) | 1.00 | 2.00 | 2.00 |  | Gaussian |  | 1.00 |  |  | 1 |  | 1.00 |  |  |  | 2.00 |  | 2.00 |  |  |
| A5-7 | Impedance mismatch in the receiving chain | 0.20 | 0.45 | 0.45 |  | U-shaped |  | 1.41 |  |  | 1 |  | 0.14 |  |  |  | 0.32 |  | 0.32 |  |  |
| A5-8 | Random uncertainty | 0.10 | 0.10 | 0.10 |  | Rectangular |  | 1.73 |  |  | 1 |  | 0.06 |  |  |  | 0.06 |  | 0.06 |  |  |
| A5-17 | Measurement antenna frequency variation | 0.10 | 0.10 | 0.10 |  | Rectangular |  | 1.73 |  |  | 1 |  | 0.06 |  |  |  | 0.06 |  | 0.06 |  |  |
| A5-18 | FSPL estimation error | 0.00 | 0.00 | 0.00 |  | Gaussian |  | 1.00 |  |  | 1 |  | 0.00 |  |  |  | 0.00 |  | 0.00 |  |  |
| A5-16 | Test system frequency flatness | 0.25 | 0.25 | 0.25 |  | Gaussian |  | 1.00 |  |  | 1 |  | 0.25 |  |  |  | 0.25 |  | 0.25 |  |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A5-9 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | 0.05 |  | U-shaped |  | 1.41 |  |  | 1 |  | 0.04 |  |  |  | 0.04 |  | 0.04 |  |  |
| A5-10 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | 0.01 |  | Rectangular |  | 1.73 |  |  | 1 |  | 0.01 |  |  |  | 0.01 |  | 0.01 |  |  |
| A5-11 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | 0.05 |  | U-shaped |  | 1.41 |  |  | 1 |  | 0.04 |  |  |  | 0.04 |  | 0.04 |  |  |
| A5-3 | Quality of quiet zone | 0.10 | 0.10 | 0.10 |  | Gaussian |  | 1.00 |  |  | 1 |  | 0.10 |  |  |  | 0.10 |  | 0.10 |  |  |
| A5-4b | Polarization mismatch for reference antenna | 0.01 | 0.01 | 0.01 |  | Rectangular |  | 1.73 |  |  | 1 |  | 0.01 |  |  |  | 0.01 |  | 0.01 |  |  |
| A5-5b | Mutual coupling between the reference antenna and the receiving antenna | 0.00 | 0.00 | 0.00 |  | Rectangular |  | 1.73 |  |  | 1 |  | 0.00 |  |  |  | 0.00 |  | 0.00 |  |  |
| A5-6b | Phase curvature | 0.05 | 0.05 | 0.05 |  | Gaussian |  | 1.00 |  |  | 1 |  | 0.05 |  |  |  | 0.05 |  | 0.05 |  |  |
| C1-3 | Uncertainty of the network analyzer | 0.13 | 0.20 | 0.20 |  | Gaussian |  | 1.00 |  |  | 1 |  | 0.13 |  |  |  | 0.20 |  | 0.20 |  |  |
| A5-12 | Influence of the reference antenna feed cable | 0.05 | 0.05 | 0.05 |  | Rectangular |  | 1.73 |  |  | 1 |  | 0.03 |  |  |  | 0.03 |  | 0.03 |  |  |
| A5-13 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | 0.06 |  | Gaussian |  | 1.00 |  |  | 1 |  | 0.06 |  |  |  | 0.06 |  | 0.06 |  |  |
| A5-14 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | 0.05 |  | Rectangular |  | 1.73 |  |  | 1 |  | 0.03 |  |  |  | 0.03 |  | 0.03 |  |  |
| C1-4 | Uncertainty of the absolute gain of the reference antenna | 0.50 | 0.43 | 0.43 |  | Rectangular |  | 1.73 |  |  | 1 |  | 0.29 |  |  |  | 0.25 |  | 0.25 |  |  |
| A5-15 | Uncertainty of the absolute gain of the receiving antenna | 0.00 | 0.00 | 0.00 |  | Rectangular |  | 1.73 |  |  | 1 |  | 0.00 |  |  |  | 0.00 |  | 0.00 |  |  |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  |  |  | 1.11 |  |  |  |  | 2.08 |  |  | 2.08 |  |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  |  |  | 2.17 |  |  |  |  | 4.07 |  |  | 4.07 |  |
| TRP Summation error |  |  |  |  |  |  |  |  |  | 0.75 |  |  |  |  | 0.75 |  |  | 0.75 |  |
| Total MU |  |  |  |  |  |  |  |  |  | 2.29 |  |  |  |  | 4.14 |  |  | 4.14 |  |

--- Next change ---

#### 12.2.4.3	MU value derivation

Table 12.2.3.4-1 captures derivation of the expanded measurement uncertainty values for OTA TX spurious emissions measurements in Reverberation Chamber (Normal test conditions).

Table 12.2.4.3-1: Reverberation Chamber  value derivation for TX spurious emissions, 380 MHz – 26 GHz

| UID | Uncertainty source |  | Uncertainty value |  |  |  |  |  |  |  |  |  | Distribution of the probability |  | Divisor based on distribution shape |  |  |  | ci |  |  | Standard uncertainty ui [dB] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 380 MHz < f ≦ 3 GHz | 3 GHz < f ≦ 6 GHz | 6 GHz < f ≦ 12.75 GHz | 12.75 GHz < f ≦ 19 GHz | 19GHz<f ≤26GHz |  |  |  | 380 MHz < f ≦ 3 GHz | 3 GHz < f ≦ 6 GHz | 6 GHz < f ≦ 12.75 GHz | 12.75 GHz < f ≦ 19 GHz | 19GHz<f ≤26GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C1-1 | RF power measurement equipment (e.g. spectrum analyzer, power meter) |  | 0.14 | 0.26 |  | 0.26 |  |  | 0.37 |  | 0.37 |  | Gaussian |  | 1.00 |  |  |  | 1.00 |  |  | 0.14 |  |  | 0.26 |  |  |  | 0.26 |  |  | 0.37 |  |  | 0.37 |  |  |
| A6-1 | Impedance mismatch in the receiving chain |  | 0.20 | 0.20 |  | 0.45 |  |  | 0.45 |  | 0.45 |  | U-shaped |  | 1.41 |  |  |  | 1.00 |  |  | 0.14 |  |  | 0.14 |  |  |  | 0.32 |  |  | 0.32 |  |  | 0.32 |  |  |
| A6-2 | Random uncertainty |  | 0.10 | 0.10 |  | 0.10 |  |  | 0.1 |  | 0.10 |  | Rectangular |  | 1.73 |  |  |  | 1.00 |  |  | 0.06 |  |  | 0.06 |  |  |  | 0.06 |  |  | 0.06 |  |  | 0.06 |  |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A6-3 | Reference antenna radiation efficiency |  | 0.50 | 0.50 |  | 0.50 |  |  | 0.5 |  | 0.50 |  | Gaussian |  | 1.00 |  |  |  | 1.00 |  |  | 0.50 |  |  | 0.5 |  |  |  | 0.5 |  |  | 0.5 |  |  | 0.5 |  |  |
| A6-4 | Mean value estimation of reference antenna radiation efficiency |  | 0.27 | 0.27 |  | 0.27 |  |  | 0.27 |  | 0.27 |  | Gaussian |  | 1.00 |  |  |  | 1.00 |  |  | 0.27 |  |  | 0.27 |  |  |  | 0.27 |  |  | 0.27 |  |  | 0.27 |  |  |
| C1-3 | Uncertainty of the network analyzer |  | 0.13 | 0.20 |  | 0.20 |  |  | 0.2 |  | 0.20 |  | Gaussian |  | 1.00 |  |  |  | 1.00 |  |  | 0.13 |  |  | 0.2 |  |  |  | 0.2 |  |  | 0.2 |  |  | 0.2 |  |  |
| A6-5 | Influence of the reference antenna feed cable |  | 0.20 | 0.20 |  | 0.20 |  |  | 0.2 |  | 0.20 |  | Gaussian |  | 1.00 |  |  |  | 1.00 |  |  | 0.20 |  |  | 0.2 |  |  |  | 0.2 |  |  | 0.2 |  |  | 0.2 |  |  |
| A6-6 | Mean value estimation of transfer function |  | 0.27 | 0.27 |  | 0.27 |  |  | 0.27 |  | 0.27 |  | Gaussian |  | 1.00 |  |  |  | 1.00 |  |  | 0.27 |  |  | 0.27 |  |  |  | 0.27 |  |  | 0.27 |  |  | 0.27 |  |  |
| A6-7 | Uniformity of transfer function |  | 1.50 | 1.50 |  | 1.50 |  |  | 1.5 |  | 1.50 |  | Gaussian |  | 1.00 |  |  |  | 1.00 |  |  | 1.50 |  |  | 1.5 |  |  |  | 1.5 |  |  | 1.5 |  |  | 1.5 |  |  |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.66 |  |  | 1.68 |  |  | 1.70 |  |  | 1.72 |  |  | 1.72 |  |  |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.25 |  |  | 3.29 |  |  | 3.34 |  |  | 3.38 |  |  | 3.38 |  |  |

--- Next change ---

### 12.2.5	Maximum accepted test system uncertainty

Whilst the TRP estimation does not require far-field

Considering the methodology described in subclause 5.1, Test Tolerance values for OTA transmitter spurious emissions were derived based on values captured in subclause 12.2.3.

Frequency range specific Test Tolerance values for the OTA TX spurious emissions test are defined in table 12.4.4-1.

Hence, we have the following MU values for the whole spurious emissions range (for FR1 and FR2 cases):

Table 12.2.5-1: Spurious emissions MU values – FR1

|  |  |  | Expanded uncertainty [dB] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 30MHz<f≤6 GHz |  | 6GHz<f ≤19GHz |  |  |  | 19GHz<f ≤26GHz |  |  |  |
| General Directional Chamber |  |  | 2.29 |  |  |  |  | 4.14 | 4.14 |  |  |
| Reverberation Chamber |  |  | 3.29 |  | 3.38 |  |  |  | 3.38 |  |
| Common maximum accepted test system uncertainty |  |  | 2.30 |  |  |  |  | 4.20 | 4.20 |  |  |

--- Next change ---

#### 12.3.2.3	MU value derivation, FR1

Table 12.3.2.3-1 captures derivation of the expanded measurement uncertainty values for OTA RX spurious emissions measurements in general chamber (Normal test conditions, FR1).

Table 12.3.2.3-1: General chamber MU value derivation for RX spurious emissions, FR1

| UID | Uncertainty source | Uncertainty value |  |  | Distribution of the probability | Divisor based on distribution shape |  | ci | Standard uncertainty ui [dB] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 30MHz<f≤6 GHz | 6GHz<f ≤19GHz | 19GHz<f ≤26GHz |  |  |  |  | 30MHz<f≤6 GHz |  |  | 6GHz<f ≤19GHz |  |  | 19GHz<f ≤26GHz |  |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A5-1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | 0.03 | Rectangular | 1.73 |  | 1 | 0.02 |  |  | 0.02 |  |  | 0.02 |  |
| A5-2 | Pointing misalignment between the AAS BS and the receiving antenna | 0.00 | 0.00 | 0.00 | Rectangular | 1.73 |  | 1 | 0.00 |  |  | 0.00 |  |  | 0.00 |  |
| A5-3 | Quality of quiet zone | 0.10 | 0.10 | 0.10 | Gaussian | 1.00 |  | 1 | 0.10 |  |  | 0.10 |  |  | 0.10 |  |
| A5-4a | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | 0.01 | Rectangular | 1.73 |  | 1 | 0.01 |  |  | 0.01 |  |  | 0.01 |  |
| A5-5a | Mutual coupling between the AAS BS and the receiving antenna | 0.00 | 0.00 | 0.00 | Rectangular | 1.73 |  | 1 | 0.00 |  |  | 0.00 |  |  | 0.00 |  |
| A5-6a | Phase curvature | 0.05 | 0.05 | 0.05 | Gaussian | 1.00 |  | 1 | 0.05 |  |  | 0.05 |  |  | 0.05 |  |
| C3-4 | Receiver spurious emissions - Conducted Uncertainty (minus missmatch) | 1.00 | 2.00 | 2.00 | Gaussian | 1.00 |  | 1 | 1.00 |  |  | 2.00 |  |  | 2.00 |  |
| A5-7 | Impedance mismatch in the receiving chain | 0.20 | 0.45 | 0.45 | U-shaped | 1.41 |  | 1 | 0.14 |  |  | 0.32 |  |  | 0.32 |  |
| A5-8 | Random uncertainty | 0.10 | 0.10 | 0.10 | Rectangular | 1.73 |  | 1 | 0.06 |  |  | 0.06 |  |  | 0.06 |  |
| A5-17 | Measurement antenna frequency variation | 0.10 | 0.10 | 0.10 | Rectangular | 1.73 |  | 1 | 0.06 |  |  | 0.06 |  |  | 0.06 |  |
| A5-18 | FSPL estimation error | 0.00 | 0.00 | 0.00 | Gaussian | 1.00 |  | 1 | 0.00 |  |  | 0.00 |  |  | 0.00 |  |
| A5-16 | Test system frequency flatness | 0.25 | 0.25 | 0.25 | Gaussian | 1.00 |  | 1 | 0.25 |  |  | 0.25 |  |  | 0.25 |  |
| A5-19 | Measurement system dynamic range uncertainty | 0.51 | 0.51 | 0.51 | Gaussian | 1.00 |  | 1 | 0.51 |  |  | 0.51 |  |  | 0.51 |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A5-9 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | 0.05 | U-shaped | 1.41 |  | 1 | 0.04 |  |  | 0.04 |  |  | 0.04 |  |
| A5-10 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | 0.01 | Rectangular | 1.73 |  | 1 | 0.01 |  |  | 0.01 |  |  | 0.01 |  |
| A5-11 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | 0.05 | U-shaped | 1.41 |  | 1 | 0.04 |  |  | 0.04 |  |  | 0.04 |  |
| A5-3 | Quality of quiet zone | 0.10 | 0.10 | 0.10 | Gaussian | 1.00 |  | 1 | 0.10 |  |  | 0.10 |  |  | 0.10 |  |
| A5-4b | Polarization mismatch for reference antenna | 0.01 | 0.01 | 0.01 | Rectangular | 1.73 |  | 1 | 0.01 |  |  | 0.01 |  |  | 0.01 |  |
| A5-5b | Mutual coupling between the reference antenna and the receiving antenna | 0.00 | 0.00 | 0.00 | Rectangular | 1.73 |  | 1 | 0.00 |  |  | 0.00 |  |  | 0.00 |  |
| A5-6b | Phase curvature | 0.05 | 0.05 | 0.05 | Gaussian | 1.00 |  | 1 | 0.05 |  |  | 0.05 |  |  | 0.05 |  |
| C1-3 | Uncertainty of the network analyzer | 0.13 | 0.20 | 0.20 | Gaussian | 1.00 |  | 1 | 0.13 |  |  | 0.20 |  |  | 0.20 |  |
| A5-12 | Influence of the reference antenna feed cable | 0.05 | 0.05 | 0.05 | Rectangular | 1.73 |  | 1 | 0.03 |  |  | 0.03 |  |  | 0.03 |  |
| A5-13 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | 0.06 | Gaussian | 1.00 |  | 1 | 0.06 |  |  | 0.06 |  |  | 0.06 |  |
| A5-14 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | 0.05 | Rectangular | 1.73 |  | 1 | 0.03 |  |  | 0.03 |  |  | 0.03 |  |
| C1-4 | Uncertainty of the absolute gain of the reference antenna | 0.50 | 0.43 | 0.43 | Rectangular | 1.73 |  | 1 | 0.29 |  |  | 0.25 |  |  | 0.25 |  |
| A5-15 | Uncertainty of the absolute gain of the receiving antenna | 0.00 | 0.00 | 0.00 | Rectangular | 1.73 |  | 1 | 0.00 |  |  | 0.00 |  |  | 0.00 |  |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 1.22 |  |  | 2.14 |  |  | 2.14 |  |  |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 2.39 |  |  | 4.19 |  |  | 4.19 |  |  |
| TRP Summation error |  |  |  |  |  |  | 0.75 |  |  | 0.75 |  |  | 0.75 |  |  |
| Total MU |  |  |  |  |  |  | 2.50 |  |  | 4.25 |  |  | 4.25 |  |  |

--- Next change ---

### 12.3.3	Maximum accepted test system uncertainty

The TRP MU is very similar to that for the transmitter mandatory spurious emissions. However, the receiver requirements are at a much lower power level so TRP calculation may be affected by the noise floor of the measurement system.

For FR2 the following MU are calculated.

Table 12.7.2.2-1: Receiver spurious emissions MU values – FR1

|  | Expanded uncertainty [dB] |  |  |
| --- | --- | --- | --- |
|  | 30MHz<f≤6 GHz | 6GHz<f ≤19GHz | 19GHz<f ≤26GHz |
| General Directional Chamber | 2.50 | 4.25 | 4.25 |
| Reverberation Chamber |  |  |  |
| Common maximum accepted test system uncertainty | 2.50 | 4.20 | 4.20 |

For FR2 the out of band frequency range is much larger, the range can be split into a number of regions:

30 MHz < f ≤ 6 GHz

The same value is assumed for receiver spurious emissions as for transmitter spurious emissions. This is the same as the in band FR2 MU value.

6 GHz < f ≤ 18 GHz

The same value is assumed for receiver spurious emissions as for transmitter spurious emissions. This is the same as the in band FR2 MU value.

18 GHz < f ≤ 40 GHz

The same value is assumed for receiver spurious emissions as for transmitter spurious emissions. This is calculated in clause 12.7.1.2.

40 GHz < f ≤ 60 GHz

The same value is assumed for receiver spurious emissions as for transmitter spurious emissions. This is calculated in clause 12.7.1.2.

Table 12.7.2.2-2: Receiver spurious emissions MU values

| Frequency range | MU (dB) |  |
| --- | --- | --- |
| 30 MHz < f ≦ 6 GHz | 2.5 |  |
| 6 GHz < f ≦ 18 GHz | 2.7 |  |
| 18 GHz < f ≦ 40 GHz | 2.7 |  |
| 40 GHz < f ≦ 60 GHz | 5 |  |

--- Next change ---

#### 12.4.2.3	MU value derivation, FR1

Table 12.4.2.3-1 captures derivation of the expanded measurement uncertainty values for additional (co-existence) OTA TX spurious emissions measurements in CATR (Normal test conditions, FR1).

Table 12.4.2.3-1: CATR MU value derivation for additional (co-existence) OTA TX spurious emissions

| UID | Uncertainty source | Uncertainty value |  |  | Distribution of the probability |  |  | Divisor based on distribution shape |  | ci |  | Standard uncertainty ui [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | f≤3 GHz | 3<f≤4.2 GHz | 4.2<f≤6 GHz |  |  |  |  |  |  |  | f≤3 GHz |  |  | 3<f≤4.2 GHz | 4.2<f≤6 GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A2-18 | Misalignment DUT & pointing error for TRP | 0.30 |  |  |  | 0.30 | 0.30 |  | Rectangular |  | 1.73 |  | 1 | 0.17 |  |  |  | 0.17 | 0.17 |
| C3-5 | Additional (COEX) emissions - Conducted Uncertainty (minus mismatch) | 1.02 |  |  |  | 1.28 | 1.53 |  | Gaussian |  | 1.00 |  | 1 | 1.02 |  |  |  | 1.28 | 1.53 |
| A2-2a | Standing wave between DUT and test range antenna | 0.21 |  |  |  | 0.21 | 0.21 |  | U-shaped |  | 1.41 |  | 1 | 0.15 |  |  |  | 0.15 | 0.15 |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | 0.00 |  |  |  | 0.00 | 0.00 |  | Gaussian |  | 1.00 |  | 1 | 0.00 |  |  |  | 0.00 | 0.00 |
| A2-4a | QZ ripple DUT | 0.09 |  |  |  | 0.09 | 0.09 |  | Gaussian |  | 1.00 |  | 1 | 0.09 |  |  |  | 0.09 | 0.09 |
| A2-17 | Measurement system dynamic range uncertainty | 0.51 |  |  |  | 0.51 | 0.51 |  | Gaussian |  | 1.00 |  | 1 | 0.51 |  |  |  | 0.51 | 0.51 |
| A2-12 | Frequency flatness | 0.25 |  |  |  | 0.25 | 0.25 |  | Gaussian |  | 1.00 |  | 1 | 0.25 |  |  |  | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C1-3 | Uncertainty of the network analyzer | 0.13 |  |  |  | 0.20 | 0.20 |  | Gaussian |  | 1.00 |  | 1 | 0.13 |  |  |  | 0.20 | 0.20 |
| A2-5 | Mismatch of receiver chain | 0.13 |  |  |  | 0.33 | 0.33 |  | U-shaped |  | 1.41 |  | 1 | 0.09 |  |  |  | 0.23 | 0.23 |
| A2-6 | Insertion loss variation of receiver chain | 0.18 |  |  |  | 0.18 | 0.18 |  | Rectangular |  | 1.73 |  | 1 | 0.10 |  |  |  | 0.10 | 0.10 |
| A2-3 | RF leakage, (SGH connector terminated & test range antenna connector cable terminated) | 0.00 |  |  |  | 0.00 | 0.00 |  | Gaussian |  | 1.00 |  | 1 | 0.00 |  |  |  | 0.00 | 0.00 |
| A2-7 | Influence of the calibration antenna feed cable: | 0.02 |  |  |  | 0.02 | 0.02 |  | U-shaped |  | 1.41 |  | 1 | 0.02 |  |  |  | 0.02 | 0.02 |
| C1-4 | Uncertainty of the absolute gain of the reference antenna | 0.50 |  |  |  | 0.43 | 0.43 |  | Rectangular |  | 1.73 |  | 1 | 0.29 |  |  |  | 0.25 | 0.25 |
| A2-8 | Misalignment positioning system | 0.00 |  |  |  | 0.00 | 0.00 |  | Exp. normal |  | 2.00 |  | 1 | 0.00 |  |  |  | 0.00 | 0.00 |
| A2-1b | Misalignment of calibration antenna and test range antenna | 0.50 |  |  |  | 0.50 | 0.50 |  | Exp. normal |  | 2.00 |  | 1 | 0.25 |  |  |  | 0.25 | 0.25 |
| A2-9 | Rotary Joints | 0.05 |  |  |  | 0.05 | 0.05 |  | U-shaped |  | 1.41 |  | 1 | 0.03 |  |  |  | 0.03 | 0.03 |
| A2-2b | Standing wave between calibration antenna and test range antenna | 0.09 |  |  |  | 0.09 | 0.09 |  | U-shaped |  | 1.41 |  | 1 | 0.06 |  |  |  | 0.06 | 0.06 |
| A2-4b | QZ ripple calibration antenna | 0.01 |  |  |  | 0.01 | 0.01 |  | Gaussian |  | 1.00 |  | 1 | 0.01 |  |  |  | 0.01 | 0.01 |
| A2-11 | Switching uncertainty | 0.26 |  |  |  | 0.26 | 0.26 |  | Rectangular |  | 1.73 |  | 1 | 0.15 |  |  |  | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.28 | 1.51 | 1.73 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.51 | 2.96 | 3.38 |
| TRP Summation error |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.75 | 0.75 | 0.75 |
| Total MU |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.62 | 3.05 | 3.47 |

--- Next change ---

### 12.4.3	Maximum accepted test system uncertainty

The additional spurious emission requirements including the co-existence with other BS in the same geographical area are based on existing co-existence with other 3GPP bands so the frequency ranges for the uncertainty assessment are different from the mandatory spurious emissions. In this case the uncertainty budgets for the in-band chambers are considered in the MU analysis. As the TRP level is at a low power level the measurement system dynamic range uncertainty is also considered as with the receiver spurious emissions.

Table 12.4.3-1: Test system specific MU values for the additional (co-existence) spurious emissions measurement

|  | Expanded uncertainty [dB] |  |  |
| --- | --- | --- | --- |
|  | 30MHz<f≤3 GHz | 3GHz<f ≤4.2GHz | 4.2<f<6 GHz |
| CATR | 2.62 | 3.05 | 3.47 |
| Common maximum accepted test system uncertainty | 2.60 | 3.00 | 3.50 |

NOTE: There are currently no additional spurious emissions requirements or co-existence requirements for FR2.

--- End of changes ---