3GPP TSG-RAN WG4 Meeting #94bis-e	R4-2004591

Online, 20th April – 30th April 2020

Source:	Ericsson, T-Mobile US, MediaTek

Title:	TP for TR 37.716-11-11: DC_66_n46

Agenda item:			6.1.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.716-11-11 to include DC_66_n46 as defined in WID [1].

# 2. Text Proposal

# ---Start of changes---

### 6.1.x	DC_66A_n46A

#### 6.1.x.1	Operating bands for DC

Table 6.1.x.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_66_n46 | 66 | n46 | No |

#### 6.1.x.2	Configuration for DC

Table 6.1.x.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_66A_n46A | DC_66A_n46A | 66A | n46A |

#### 6.1.x.3	Maximum output power for DC

Table 6.1.x.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_66A_n46A | 23 | +2/-3 |

#### 6.1.x.4	Spurious emission band UE co-existence for DC

Does not need to be defined for this combination.

#### 6.1.x.5	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.x.5-1

Table 6.1.x.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 5150 | 5925 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 10300 | 11850 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 15450 | 17775 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 | 20600 | 23700 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8900 | 25750 | 29625 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 10260 | 10680 | 30900 | 35550 |
| 7th harmonics frequency limits | 7* fy_low | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 11970 | 12460 | 36050 | 41475 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4215 | 3370 | 6860 | 7705 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2505 | 1590 | 8520 | 10140 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8570 | 9485 | 12010 | 13630 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 8430 | 6740 | 13720 | 15410 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 795 | 190 | 13670 | 16065 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10280 | 11265 | 17160 | 19555 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 21990 | 18820 | 1970 | 915 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 22310 | 25480 | 11990 | 13045 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 14355 | 11890 | 4960 | 6720 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 18870 | 21335 | 15430 | 17190 |

Based on Table 6.1.X.5-1, it can be seen that

- 2rd order harmonic may fall into Rx frequencies of band 22, 42, 48, 49, n77 and n78.

- 3rd order harmonic may fall into own Rx frequencies of band n46.

- 2nd order IMD products may fall into Rx frequencies of bands 22, 42, 43, 48, 49, n77 and n78.

- 3rd order IMD products may fall into Rx frequencies of bands 1, 2, 3, 4, 9, 10, 23, 25, 30, 33, 34, 35, 36, 37, 39, 40, 41, 65 and 70 and also own Rx frequencies of band 66.

- 4th order IMD products may fall into Rx frequencies of bands 12, 13, 14, 17, 20, 28, 29, 31, 44, 67, 68, 71, 72 and 85.

- 5th order IMD products may fall into Rx frequencies of bands 2, 3, 8, 9, 11, 21, 24, 25, 32, 33, 35, 36, 37, 39, 45, 47, 50, 51, 74, 75, 76, n79 and n258 and own Rx frequencies of band n46.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.1.X.5-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

6.1.x.5-2: Band 2 and Band n46 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | 3rd, 5th IMD |
| Galileo | 1559 | - | 1591 | Yes |  | 3rd, 5th IMD |
| GLONASS | 1591 | - | 1610 | Yes |  | 3rd, 5th IMD |
| GPS | 1563 | - | 1587 | Yes |  | 5th IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes |  | 3rd IMD |
|  | 2400 | - | 2494 | Yes |  | 3rd IMD |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 5th IMD 3rd harmonics |
|  | 5150 | - | 5350 | Yes | Europe | 5th IMD 3rd harmonics |
|  | 5470 | - | 5725 | Yes |  | 5th IMD |
|  | 5150 | - | 5825 | Yes | Asia | 5th IMD 3rd harmonics |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

### 6.1.x.6	∆TIB and ∆RIB values

For DC_66_n46, the TIB,c and RIB,c values are derived from LTE combination CA_46-66 and are given in the tables below.

# Table 6.1.x.6-1: ΔTIB,c

| EN-DC Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n46 | 66 | 0 |
|  | n46 | 0 |

# Table 6. 1.x.6-2: ΔRIB,c

| NR CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_66_n46 | 66 | 0 |
|  | n46 | 0 |

### 6.1.x.7	MSD

MSD for DC_66_n46 is proposed to be defined like in table below, where Note 1 in Table 6.x.1.7-1 is derived from Note 5 in TS36.101 Table 7.3.1A-0eA. The Table 6.x.1.7-2 is derived from TS36.101 Table 7.3.1A-0eC.

Table 6.x.1.7-1: Reference sensitivity QPSK PREFSENS (EN-DC with n46)

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 66 | n46 |  |  |  | 0 |  | 0 |  | 0 | 0 |  | 0 |
| NOTE 1: These requirements do not apply when there is at least one individual RE within the downlink (victim) transmission bandwidth which falls into the reference sensitivity exclusion region as specified in Table 6.x.1.7-2 and Table 6.x.1.7-3. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.x.1.7-2: n46 Reference sensitivity measurement exclusion region in MHz

| Licensed Component Carriers / E-UTRA Band / Harmonic order / Channel BW in UL |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Band | Harmonic order | 5MHz | 10MHz | 15MHz | 20MHz |
| 66 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| NOTE 1: Even though UL harmonic does not fall directly into n46 the exclusion region still applies. NOTE 2: The center of the exclusion region is obtained by multiplying the uplink channel center frequency by the harmonic order. |  |  |  |  |  |

---End of changes---

# Reference

[1] 		RP-192926, “Revised WID on NR-based Access to Unlicensed Spectrum”, Qualcomm