3GPP TSG-RAN WG4 Meeting #94bis-e	R4-2004599

Online, 20th April – 30th April 2020

Source:	Ericsson, Telstra

Title:	TP to TR 37.716.21-21: Addition of DC configurations for DC_1-7_n7-n78

Agenda item:	9.7.2

Document for:	Approval

# Background

This text proposal for TR 37.716-21-21 to add DC_1-7_n7-n78 configurations as defined in WID [1].

# Text Proposal

##### ---Start of changes---

## 7.x	DC_1-7_n7-n78

### 7.x.1	Operating bands for DC

Table 7.x.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-7_n7-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.x.2	Channel bandwidths per operating band for DC

Table 7.x.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-7A_n7A-n78A | DC_1A_n7A DC_7A_n7A4 DC_1A_n78A DC_7A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| NOTE 4: Only single switched UL is supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.x.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

### 7.x.4	∆TIB and ∆RIB values

For DC_1-7_n7-n78 the TIB,c and RIB,c values are same as for CA_n1-n7-n78 and are given in the tables below.

Table 7.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7_n7-n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |

Table 7.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7_n7-n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |

### 7.x.5	MSD

Since DC_1_n7 UL gives IMD4 towards n78 DL, DC_1_n78 UL gives IMD4 towards n7 DL and DC_7_n78 UL gives IMD4 towards 1 DL, MSD needs to be defined as it is for DC_1A-7A_n78A and DC_1A-7A_n78A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-7A_n7A-n78A | 1 | 1950 | 5 | 25 | 2140 | 8.7 | IMD4 |
|  | 7, n7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |
|  | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 7, n7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | IMD4 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 7, n7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 10.1 | IMD4 |

##### ---End of changes---

# Reference

[1]	RP-200080, “Revised WID on EN-DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)”, LG Electronics