3GPP TSG-RAN WG4 Meeting #94-e-Bis 	R4-2004672

# Electronic Meeting, April 20-April 30, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | xxxx | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Measurement report mapping for PRS RSRP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_POS-Core |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Measurement report mapping is missing for PRS RSRP |  |  |  |  |  |  |  |  |
| Summary of change: |  | Measurement report mapping is introduced for PRS RSRP |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Measurement report mapping is missing for PRS RSRP |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sections: 10.1.23, 10.1.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 10.1.23	DL PRS-RSRP measurement accuracy requirements

### 10.1.24	DL PRS-RSRP measurement report mapping

The reporting range of PRS-RSRP is defined from -156 dBm to -31 dBm with 1 dB resolution.

The mapping of measured quantity is defined in Table 10.1.24-1. The range in the signalling may be larger than the guaranteed accuracy range.

Editor’s note: report mapping for differential PRS-RSRP reporting is FFS.

Table 10.1.24-1: Measurement report mapping for PRS-RSRP

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RSRP_0 | SS-RSRP<-156 | dBm |
| RSRP_1 | -156≤ SS-RSRP<-155 | dBm |
| RSRP_2 | -155≤ SS-RSRP<-154 | dBm |
| RSRP_3 | -154≤ SS-RSRP<-153 | dBm |
| RSRP_4 | -153≤ SS-RSRP<-152 | dBm |
| RSRP_5 | -152≤ SS-RSRP<-151 | dBm |
| RSRP_6 | -151≤ SS-RSRP<-150 | dBm |
| RSRP_7 | -150≤ SS-RSRP<-149 | dBm |
| RSRP_8 | -149≤ SS-RSRP<-148 | dBm |
| RSRP_9 | -148≤ SS-RSRP<-147 | dBm |
| RSRP_10 | -147≤ SS-RSRP<-146 | dBm |
| RSRP_11 | -146≤ SS-RSRP<-145 | dBm |
| RSRP_12 | -145≤ SS-RSRP<-144 | dBm |
| RSRP_13 | -144≤ SS-RSRP<-143 | dBm |
| RSRP_14 | -143≤ SS-RSRP<-142 | dBm |
| RSRP_15 | -142≤ SS-RSRP<-141 | dBm |
| RSRP_16 | -141≤ SS-RSRP<-140 | dBm |
| RSRP_17 | -140≤ SS-RSRP<-139 | dBm |
| RSRP_18 | -139≤ SS-RSRP<-138 | dBm |
| … | … | … |
| RSRP_111 | -46≤ SS-RSRP<-45 | dBm |
| RSRP_112 | -45≤ SS-RSRP<-44 | dBm |
| RSRP_113 | -44≤ SS-RSRP<-43 | dBm |
| RSRP_114 | -43≤ SS-RSRP<-42 | dBm |
| RSRP_115 | -42≤ SS-RSRP<-41 | dBm |
| RSRP_116 | -41≤ SS-RSRP<-40 | dBm |
| RSRP_117 | -40≤ SS-RSRP<-39 | dBm |
| RSRP_118 | -39≤ SS-RSRP<-38 | dBm |
| RSRP_119 | -38≤ SS-RSRP<-37 | dBm |
| RSRP_120 | -37≤ SS-RSRP<-36 | dBm |
| RSRP_121 | -36≤ SS-RSRP<-35 | dBm |
| RSRP_122 | -35≤ SS-RSRP<-34 | dBm |
| RSRP_123 | -34≤ SS-RSRP<-33 | dBm |
| RSRP_124 | -33≤ SS-RSRP<-32 | dBm |
| RSRP_125 | -32≤ SS-RSRP<-31 | dBm |
| RSRP_126 | -31≤ SS-RSRP | dBm |
| RSRP_127 (Note) | Infinity | dBm |