3GPP TSG-RAN4 Meeting #94bis-e		R4-2004698

Electronic meeting, 20 April – 30 April, 2020

Title:	[DRAFT] LS on Frequency separation class for DL-only spectrum for FR2

Response to:	-

Release:	Rel-16

Work Item:	NR_RF_FR2_req_enh

Source:	Apple Inc. [to be changed to TSG RAN WG4]

To:	TSG RAN WG2

Cc:	-

Contact Person:

#### Name:	Camila Priale

Tel. Number:

E-mail Address:	cpriale@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN4 has enhanced the frequency separation (Fs) to up to 2400 MHz as shown in Table 1. It is important to emphasize that the frequency separation (Fs) larger than 1400 MHz apply only to downlink frequency separation.

Table 1: Frequency separation classes for non-contiguous intra-band operation

| Frequency separation class | Frequency separation (Fs) |
| --- | --- |
| I | Fs ≤ 800 MHz |
| II | Fs ≤ 1200 MHz |
| III | Fs ≤ 1400 MHz |
| IV | Fs ≤ 1000 MHz |
| V | Fs ≤ 1600 MHz |
| VI | Fs ≤ 1800 MHz |
| VII | Fs ≤ 2000 MHz |
| VIII | Fs ≤ 2200 MHz |
| IX | Fs ≤ 2400 MHz |
| NOTE 1: Fs values larger than 1400 MHz apply only to downlink frequency separation. |  |

Additionally, RAN has agreed on the introduction of frequency separation class for DL-only spectrum (Fsd), which has been defined as the maximum DL-only frequency separation span of DL CCs that UE can support per band.

For the implementation of the signalling, we kindly ask to take into account:

- DL-only spectrum configures only DL CCs and not UL CCs.

- The spectrum covered by the DL-only frequency separation extends on one-side relative to the bidirectional spectrum.

- The bidirectional spectrum is defined as the UL/DL common spectrum in which the UE supports the configuration of uplink or downlink CCs.

- The sum value of the frequency separation (Fs) and frequency separation class for DL-only spectrum (Fsd) cannot exceed 2400 MHz.

- The signalling shall be implemented in addition to the signalling of frequency separation (Fs) from Rel-15.

RAN4 considers the frequency separation classes for DL-only spectrum as provided in the Table 2 below.

Table 2: Frequency separation classes for DL-only spectrum

| Frequency separation class for DL-only spectrum | Frequency separation (Fsd) |
| --- | --- |
| I | Fsd ≤ 200 MHz |
| II | Fsd ≤ 400 MHz |
| III | Fsd ≤ 600 MHz |
| IV | Fsd ≤ 800 MHz |
| V | Fsd ≤ 1000 MHz |

2. Actions:

RAN4 kindly asks RAN2 to take into account the parameters above for the signalling implementation.

3. Date of Next TSG-RAN WG5 Meetings:

RAN4 #95-e	 25th May – 06th June 2020	     Online

RAN4 #96 	 24th August – 28th August 2020	     Toulouse, France