3GPP TSG-RAN WG4 Meeting #94bis-e            	R4-2004925

Electrical Meeting, 20th April– 30th April, 2020

Agenda item:	6.11.3.3

Source:	Samsung

Title:	Initial simulation results for PDSCH test cases with multi-TRP/panel transmission

Document for:	Discussion

# 1	Introduction

In the last RAN4 meeting, the WF was approved regarding to the demodulation requirement based on multi-TRP/Panel transmission for NR eMIMO [1]. The agreements related to Multi-TRP (eMBB basis) requirements were reached in the WF as listed below:

- Requirements for possible scheduling schemes besides non-overlapping

  - Option 1: Partial overlapping

  - Option 2: Full-overlapping

  - Option 3: Both full overlapping and partial overlapping

  - Option 4: No other schemes

- Test configurations for Non-overlapping

  - Layer combination

    - Option 1: 2+2

    - Option 2: 1+1

    - Option 3: 1+2 or 2+1

    - Option 4: both options 1, 2 and 3

  - Timing offset

    - Option 1: [2us]

    - Option 2: other options are not precluded

  - Frequency offset

    - Option 1: [0Hz]

    - Other options are not precluded

- Test configurations for Full-overlapping (if agreed)

  - Layer combination

    - Option 1: 1+1

    - Option 2: 1+2 or 2+1)

    - Option 3: (2+2)

    - Option 4: all

  - Timing offset

    - Option 1: [0 us]

    - Option 2: other options are not precluded

  - Frequency offset

    - Option 1: [300]Hz

    - Other options are not precluded

In this contribution, the initial simulation results are provided for alignment purpose.

# 2	Simulation results

We provide initial simulation results of Multi-TRP tests with two TRPs under Non-overlapping and Full-overlapping scenario respectively.  Besides, we also take the TA offset and Frequency offset into consideration to observe the related performance influence.

Fig2-1: SNR--Throughput of Non-overlapping with TA offset=2us

Fig2-2: SNR--Throughput of Full-overlapping with Frequency offset=300Hz

Table2-1: SNR point at 70% of peak throughput

| nTx ports | 2Tx*2TRP |  |
| --- | --- | --- |
| Scenario | Non-overlapping (TA offset=2us) | Full-overlapping (Frequency offset=300Hz) |
| SNR point for PDSCH without TRS | -0.78dB | -1.6dB |
| SNR point for PDSCH with TRS | -3.06dB | -2.4dB |

As indicated in the figures, the performance have some degradation with impact of timing offset and frequency offset. With TRS configured, the performance can be close with ideal case.

The performance with non-overlap is slightly better than full-overlapping with 1+1 layer combination.

Observation 1:  With TRS configured, the performance can be close with ideal case.

Observation 2: The performance with non-overlap is slightly better than full-overlapping with 1+1 layer combination

# 3	Conclusion

In this contribution, the initial simulation results are provided with Multi-TRP of eMBB under non-overlapping and full-overlapping with TA offset and Frequency offset

Observation 1:  With TRS configured, the performance can be close with ideal case.

Observation 2: The performance with non-overlap is slightly better than full-overlapping with 1+1 layer combination

# References

R4-2002420,”Way forward on PDSCH demodulation requirement based on multi-TRP/Panel transmission for NR eMIMO” Huawei, HiSilicon. 3GPP TSG-RAN WG4 Meeting #94e

# Appendix

Simulation assumption:

| Parameter |  | Unit | TRP1 | TRP2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| Subcarrier spacing |  | kHz | 15 |  |
| Duplex Mode |  |  | FDD |  |
| Propagation channel |  |  | TDLC300-100 |  |
| Antenna configuration |  |  | LOW 2x2 | LOW 2x2 |
| Cell ID |  |  | 0 | 2 |
| DMRS configuration | CDM group index |  | 0 | 1 |
|  | Additional number |  | 1 |  |
|  | ratio of PDSCH EPRE to DM-RS EPRE |  | -3dB |  |
| CSI-RS configuration | ZP CSI-RS |  | Not configured |  |
|  | NZP CSIRS for measurement |  | Not configured |  |
|  | CSI IM |  | Not configured |  |
| TRS configuration | Resource number |  | 4 |  |
|  | Time domain location |  | l0=6;l1=10 |  |
|  | Frequency domain location |  | k0=0; k1=4; k2=8 | k0=2; k1=6; k2=10 |
|  | Time Config interval and offset | slot | 20/10 |  |
| TA Offset |  | us | 0 | 2 |
| Frequency Offset |  | Hz | 0 | 300 |
| CSI Feedback |  |  | Not configured |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | MCS 4 FDD (see FRC) |  |

PDSCH Reference Channel

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | R.PDSCH.1-1.1 FDD |
| Channel bandwidth | MHz | 10 |
| Subcarrier spacing | kHz | 15 |
| Number of allocated resource blocks | PRBs | 52 |
| Number of consecutive PDSCH symbols |  | 12 |
| Allocated slots per 2 frames | Slots | 19 |
| MCS table |  | 64QAM |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 0.30 |
| Number of MIMO layers |  | 1 |
| Number of DMRS rEs |  | 24 |
| Overhead for TBS determination |  | 0 |
| Information Bit Payload per Slot |  |  |
| For Slot i = 0 | Bits | N/A |
| For Slots i = 1,…, 19 | Bits | 3752 |
| Transport block CRC per Slot |  |  |
| For Slot i = 0 | Bits | N/A |
| For Slots i = 1,…, 19 | Bits | 24 |
| Number of Code Blocks per Slot |  |  |
| For Slot i = 0 | CBs | N/A |
| For Slots i = 1,…, 19 | CBs | 1 |
| Binary Channel Bits Per Slot |  |  |
| For Slot i = 0 | Bits | N/A |
| For Slots i = 10, 11 | Bits | 11856 |
| For Slots i =1,…, 9, 12, …, 19 | Bits | 12480 |
| Max. Throughput averaged over 2 frames | Mbps | 3.564 |