3GPP TSG-RAN WG4 (Radio) Meeting #94-e-Bis	R4-2004951

Electronic Meeting, 20 – 30 April 2020

Agenda Item:	11.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	System level simulation methodology and assumptions for study on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz

Document for:	Approval

# 1.	Introduction

The study item on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz was approved at TSG RAN#87-e [1]. The purpose of this study item is to study the IMT parameters relevant for sharing and compatibility for the following frequencies:

- 6425-7025MHz and 7025-7125MHz.

- 10000-10500MHz.

This study item aims as answering requests from ITU-R WP5D regarding NR in these frequencies for IMT [2]. Two set of parameters requested by ITU-R WP5D are the ACLR and ACS of BS and UE in these frequencies. Currently, different sets of ACLR and ACS are specified in RAN4 specifications [3, 4, 5] for NR BS and UE in FR1 and FR2 based on coexistence studies as recorded in TR 38.803 [6]. Therefore, coexistence studies will need to be carried out to provide answers to ITU-R WP5D on these parameters with sound technical justifications from coexistence perspective.

This contribution proposes the system level simulation methodology and assumptions for coexistence study on ACLR and ACS of NR BS and UE for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz.

# 2.	Discussion

Unless stated otherwise, the simulation methodology and assumptions shall mainly be based on those RAN4 used for NR coexistence simulation for NR coexistence study as recorded in TR 38.803, the channel model for frequencies from 0.5 to 100 GHz in TR 38.901 [7], as well as those in RAN4 reply to WP5D for ITU-R sharing and compatibility studies in the frequency band 3.3-3.4GHz in RP-182783 [8].

### 2.1		Network layout model

#### 2.1.1	Urban macro

Details on urban macro network layout model are listed in Tables 2.1.1-1 and 2.1.1-2.

Table 2.1.1-1: Single operator layout for urban macro

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 0.45 km (urban) 0.9 km (suburban) | Based on cell radius: 0.3 km (urban) 0.6 km (suburban) |
| BS antenna height |  | 20 m (urban) 25 m (suburban) |  |
| UE location [6] | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 20% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Channel model |  | UMa |  |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |

Table 2.1.1-2: Multi operators layout for urban macro

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operators layout | coordinated operation (0% Grid Shift) and un-coordinated operation (100% Grid Shift) | RAN4 has long been using un-coordinated operation in below 6GHz coexistence simulation |

| Coordinated Operation: each network with co-location of sites |  |
| --- | --- |

Figure 2.1.1-1: Coordinated operation

| Uncoordinated Operation: second network’s sites are located at the first network’s cell edge |  |
| --- | --- |

Figure 2.1.1-2: Uncoordinated operation

#### 2.1.2	Dense urban

Details on dense urban network layout model are listed in Tables 2.1.2-1 and 2.1.2-2.

Table 2.1.2-1: Single operator layout for dense urban

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | Fixed cluster circle within a macro cell. | note1 |
| Number of micro BSs per macro cell |  | 3 | 3 cluster circles are in a macro cell. 1 cluster circle has 1 micro BS. |
| Radius of UE dropping within a micro cell |  | < 65 m (urban) < 130 m (suburban) | Based on cell radius: 0.3 km (urban) 0.6 km (suburban) |
| BS antenna height |  | 6 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 80 % |  |
|  | 50% low loss, 50% high loss | Low/high Penetration loss ratio |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMi in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 3m |  |
| Channel model |  | UMi |  |
| Shadowing correlation |  | Between cite: 0.5 |  |
| Note 1: Micro BS is randomly dropped on an edge of the cluster circle. All UEs communicate with micro BS, i.e. macro cell is only used for determining position of micro BS. As a layout of macro cell, hexagonal grid, 19 macro sites, 3 sectors per site model with wrap around with ISD = 200m is assumed. |  |  |  |

Figure 2.1.2-1: Network layout for dense urban

Table 2.1.2-2: Multi operators layout for dense urban

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operator layout | Cluster circle is coordinated | Note 1 |
| Minimum distance between micro BSs in different operator | 10 m |  |
| Note 1: Macro cell is collocated. Micro BS itself is randomly dropped. |  |  |

#### 2.1.3	Indoor

Details on indoor network layout model are listed in Tables 2.1.3-1 and 2.1.3-2.

Table 2.1.3-1: Single operator layout for indoor

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | 50m x 120m, 12BSs |  |
| Inter-site distance |  | 20m |  |
| BS antenna height |  | 3 m | ceiling |
| UE location | Outdoor/indoor | Indoor |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | 1 m |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 0 m |  |
| Channel model |  | Indoor Office |  |
| Shadowing correlation |  | NA |  |

Figure 2.1.3-1: Network layout for indoor

Table 2.1.3-2: Multi operators layout for indoor

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operator layout | Coordinated operation (0% Grid Shift) |  |

### 2.2		Propagation model

#### 2.2.1	Path loss

The pathloss models are summarized in Table 2.2.1-1 and the distance definitions are indicated in Figures 2.2.1-1 and 2.2.1-2. Note that the distribution of the shadow fading is log-normal, and its standard deviation for each scenario is given in Table 2.2.1-1.

| Figure 2.2.1-1: Definition of d2D and d3D for outdoor UTs | Figure 2.2.1-2: Definition of d2D-out, d2D-in and d3D-out, d3D-in for indoor UTs. |
| --- | --- |

Note that

d3D-out+d3D-in=d2D-out+d2D-in2+hBS-hUT2	(2.2-1)

Table 2.2.1-1: Pathloss models

| Scenario | LOS/NLOS | Pathloss [dB], fc is in GHz and d is in meters, see note 6 | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- | --- |
| RMa | LOS | PLRMa-LOS=PL110m≤d2D≤dBPPL2dBP≤d2D≤10km, see note 5 PL1=20log10(40πd3Dfc/3)+min(0.03h1.72,10)log10(d3D) -min(0.044h1.72,14.77)+0.002log10(h)d3D PL2=PL1(dBP)+40log10(d3D/dBP) | σSF=4 σSF=6 | hBS=35m hUT=1.5m W=20m h=5m h = avg. building height W = avg. street width The applicability ranges: 5m≤h≤50m 5m≤W≤50m 10m≤hBS≤150m 1m≤hUT≤10m |
|  | NLOS | PLRMa-NLOS=max(PLRMa-LOS,PLRMa-NLOS') for 10m≤d2D≤5km PLRMa-NLOS'=161.04-7.1log10(W)+7.5log10(h) -(24.37-3.7(h/hBS)2)log10(hBS) +(43.42-3.1log10(hBS))(log10(d3D)-3) +20log10(fc)-(3.2(log10(11.75hUT))2-4.97) | σSF=8 |  |
| UMa | LOS | PLUMa-LOS=PL110m≤d2D≤dBP'PL2dBP'≤d2D≤5km, see note 1 PL1=28.0+22log10(d3D)+20log10(fc) PL2=28.0+40log10(d3D)+20log10(fc) -9log10((dBP')2+(hBS-hUT)2) | σSF=4 | 1.5m≤hUT≤22.5m hBS=25m |
|  | NLOS | PLUMa-NLOS=max(PLUMa-LOS,PLUMa-NLOS') for 10m≤d2D≤5km | σSF=6 | 1.5m≤hUT≤22.5m hBS=25m Explanations: see note 3 |
|  |  | Optional PL=32.4+20log10fc+30log10d3D | σSF=7.8 |  |
| UMi - Street Canyon | LOS | PLUMi-LOS=PL110m≤d2D≤dBP'PL2dBP'≤d2D≤5km, see note 1 PL1=32.4+21log10(d3D)+20log10(fc) PL2=32.4+40log10(d3D)+20log10(fc) -9.5log10((dBP')2+(hBS-hUT)2) | σSF=4 | 1.5m≤hUT≤22.5m hBS=10m |
|  | NLOS | PLUMi-NLOS=max(PLUMi-LOS,PLUMi-NLOS') for 10m≤d2D≤5km | σSF=7.82 | 1.5m≤hUT≤22.5m hBS=10m Explanations: see note 4 |
|  |  | Optional PL=32.4+20log10fc+31.9log10d3D | σSF=8.2 |  |
| InH - Office | LOS | PLInH-LOS=32.4+17.3log10(d3D)+20log10(fc) | σSF=3 | 1m≤d3D≤150m |
|  | NLOS | PLInH-NLOS=max(PLInH-LOS,PLInH-NLOS') PLInH-NLOS'=38.3log10d3D+17.30+24.9log10fc | σSF=8.03 | 1m≤d3D≤150m |
|  |  | Optional PLInH-NLOS'=32.4+20log10fc+31.9log10d3D | σSF=8.29 | 1m≤d3D≤150m |
| InF | LOS | PLLOS=31.84+21.50log10d3D+19.00log10fc | σSF=4.3 | 1≤d3D≤600 m |
|  | NLOS | InF-SL: PL=33+25.5log10d3D+20log10fc PLNLOS=max⁡(PL,PLLOS) | σSF=5.7 |  |
|  |  | InF-DL: PL=18.6+35.7log10d3D+20log10fc PLNLOS=max⁡(PL,PLLOS,PLInF-SL) | σSF=7.2 |  |
|  |  | InF-SH: PL=32.4+23.0log10d3D+20log10fc PLNLOS=max⁡(PL,PLLOS) | σSF=5.9 |  |
|  |  | InF-DH: PL=33.63+21.9log10d3D+20log10fc PLNLOS=max⁡(PL,PLLOS) | σSF=4.0 |  |
| Note 1: Breakpoint distance d'BP = 4 h'BS h'UT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. The effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and hE is the effective environment height. For UMi hE = 1.0m. For UMa hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. With C(d2D, hUT) given by Cd2D,hUT=0,hUT<13mhUT-13101.5gd2D,13m≤hUT≤23m, where gd2D=0,d2D≤18m54d2D1003exp-d2D150,18m<d2D. Note that hE depends on d2D and hUT and thus needs to be independently determined for every link between BS sites and UTs. A BS site may be a single BS or multiple co-located BSs. Note 2: The applicable frequency range of the PL formula in this table is 0.5 < fc < fH GHz, where fH = 30 GHz for RMa and fH = 100 GHz for all the other scenarios. It is noted that RMa pathloss model for >7 GHz is validated based on a single measurement campaign conducted at 24 GHz. Note 3: UMa NLOS pathloss is from TR36.873 with simplified format and PLUMa-LOS = Pathloss of UMa LOS outdoor scenario. Note 4: PLUMi-LOS = Pathloss of UMi-Street Canyon LOS outdoor scenario. Note 5: Break point distance dBP = 2π hBS hUT fc/c, where fc is the centre frequency in Hz, c = 3.0 108 m/s is the propagation velocity in free space, and hBS and hUT are the antenna heights at the BS and the UT, respectively. Note 6: fc denotes the center frequency normalized by 1GHz, all distance related values are normalized by 1m, unless it is stated otherwise. |  |  |  |  |

#### 2.2.2	LOS probability

The Line-Of-Sight (LOS) probabilities are given in Table 2.2.2-1.

Table 2.2.2-1 LOS probability

| Scenario | LOS probability (distance is in meters) |
| --- | --- |
| RMa | PrLOS=1,d2D-out≤10mexp-d2D-out-101000,10m<d2D-out |
| UMi - Street canyon | PrLOS=1,d2D-out≤18m18d2D-out+exp-d2D-out361-18d2D-out,18m<d2D-out |
| UMa | PrLOS=1,d2D-out≤18m18d2D-out+exp-d2D-out631-18d2D-out1+C'hUT54d2D-out1003exp-d2D-out150,18m<d2D-outwhere C'(hUT)=0,hUT≤13mhUT-13101.5,13m<hUT≤23m |
| Indoor - Mixed office | PrLOS=1,d2D-in≤1.2mexp-d2D-in-1.24.7,1.2m<d2D-in<6.5mexp-d2D-in-6.532.6⋅0.32,6.5m≤d2D-in |
| Indoor - Open office | PrLOS=1,d2D-in≤5mexp-d2D-in-570.8,5m<d2D-in≤49mexp-d2D-in-49211.7⋅0.54,49m<d2D-in |
| InF-SL InF-SH InF-DL InF-DH | PrLOS,subsced2D=exp-d2Dksubsce where ksubsce=-dclutterln1-rfor InF-SL and InF-DL-dclutterln1-r∙hBS-hUThc-hUTfor InF-SH and InF-DH The parameters dclutter, r, and hc are defined in Table 7.2-4 |
| InF-HH | PrLOS=1 |
| Note: The LOS probability is derived with assuming antenna heights of 3m for indoor, 10m for UMi, and 25m for UMa |  |

    - O-to-I penetration loss

#### 2.2.3.1	O-to-I building penetration loss

The pathloss incorporating O2I building penetration loss is modelled as in the following:

PL=PLb+PLtw+PLin+N0,σP2	(2.2-2)

where PLb is the basic outdoor path loss given in Section 2.2.1, where d3D is replaced by d3D-out+d3D-in . PLtw is the building penetration loss through the external wall, PLin is the inside loss dependent on the depth into the building, and σP is the standard deviation for the penetration loss.

PLtw is characterized as:

PLtw=PLnpi-10log10i=1Npi×10Lmaterial_i-10	(2.2-3)

PLnpi is an additional loss is added to the external wall loss to account for non-perpendicular incidence; Lmaterial_i=amaterial_i+bmaterial_i⋅f, is the penetration loss of material i, example values of which can be found in Table 2.2.3-1; pi is proportion of i-th materials, where i=1Npi=1; and N is the number of materials.

Table 2.2.3-1: Material penetration losses

| Material | Penetration loss [dB] |
| --- | --- |
| Standard multi-pane glass | Lglass=2+0.2f |
| IRR glass | LIIRglass=23+0.3f |
| Concrete | Lconcrete=5+4f |
| Wood | Lwood=4.85+0.12f |
| Note: f is in GHz |  |

Table 2.2.3-2 gives PLtw, PLin and σP for two O2I penetration loss models. The O2I penetration is UT-specifically generated and is added to the SF realization in the log domain.

Table 2.2.3-2: O2I building penetration loss model

|  | Path loss through external wall: PLtw in [dB] | Indoor loss: PLin in [dB] | Standard deviation: σP in [dB] |
| --- | --- | --- | --- |
| Low-loss model | 5-10log100.3⋅10-Lglass10+0.7⋅10-Lconcrete10 | 0.5 d2D-in | 4.4 |
| High-loss model | 5-10log100.7⋅10-LIIRglass10+0.3⋅10-Lconcrete10 | 0.5 d2D-in | 6.5 |

d2D-in is minimum of two independently generated uniformly distributed variables between 0 and 25 m for UMa and UMi-Street Canyon, and between 0 and 10 m for RMa. d2D-in shall be UT-specifically generated.

Both low-loss and high-loss models are applicable to UMa and UMi-Street Canyon.

Only the low-loss model is applicable to RMa.

Only the high-loss model is applicable to InF.

#### 2.2.3.2	O-to-I car penetration loss

The pathloss incorporating O2I car penetration loss is modelled as in the following:

PL=PLb+Νμ,σP2	(2.2-4)

where PLb is the basic outdoor path loss given in Clause 2.2.1. μ = 9, and σP = 5. The car penetration loss shall be UT-specifically generated. Optionally, for metallized car windows, μ = 20 can be used. The O2I car penetration loss models are applicable for at least 0.6-60 GHz.

### 2.3		Antenna and beam forming pattern modelling

#### 2.3.1	General

A general antenna model is a uniform rectangular panel array, comprising MgNg panels, as illustrated in Figure 2.3.1-1.

-	Mg is number of panels in a column

-	Ng is number of panels in a row

-	Antenna panels are uniformly spaced in the horizontal direction with a spacing of dg,H and in the vertical direction with a spacing of dg,V.

-	On each antenna panel, antenna elements are placed in the vertical and horizontal direction, where N is the number of columns, M is the number of antenna elements with the same polarization in each column.

-	Antenna numbering on the panel illustrated in Figure 2.3.1-1 assumes observation of the antenna array from the front (with x-axis pointing towards broad-side and increasing y-coordinate for increasing column number).

-	The antenna elements are uniformly spaced in the horizontal direction with a spacing of dH and in the vertical direction with a spacing of dV.

-	The antenna panel is either single polarized (P=1) or dual polarized (P=2).

The rectangular panel array antenna can be described by the following tuple Mg,Ng,M,N,P.

Figure 2.3.1-1: General antenna model

For a uniformly distributed array (ULA) antenna, as shown in Figure 2.3.1-2, the radiation elements are placed uniformly along the vertical z-axis in the Cartesian coordinate system. The x-y plane constructs the horizontal plane. A signal acting at the array elements is in the direction of u. The elevation angle of the signal direction is denoted as θ (defined between 0° and 180°, 90° represents perpendicular angle to the array antenna aperture) and the azimuth angle is denoted asϕ (defined between -180° and 180°).

Figure 2.3.1-2: Antenna Array Geometry

The linear phase progression-based beamforming is assumed, as described in Table 2.3.1-1.

Table 2.3.1-1: Composite antenna pattern

| Parameter | Values |
| --- | --- |
| Composite Array radiation pattern in dB AAθ,ϕ | For beam i: AA,Beamiθ,ϕ=AEθ,ϕ+10log10m=1NHn=1NVwi,n,m⋅vn,m2 the super position vector is given by: vn,m=expi⋅2πn-1⋅dVλ⋅cosθ+(m-1)⋅dHλ⋅sin(θ)⋅sin(ϕ), n=1,2,…NV;m=1,2,...NH; the weighting is given by: wi,n,m=1NHNVexpi⋅2πn-1⋅dVλ⋅sinθi,etilt-(m-1)⋅dHλ⋅cos(θi,etilt)⋅sin(ϕi,escan) |

In this simulation, there is one beam formed using all the antenna elements. Each beam is directed to one scheduled UE.

Note the above gives the correct antenna array radiation pattern, however the correct gain is only achieved if the element pattern AAθ,ϕ is selected for the exact element spacing. For other element spacings, the element pattern AAθ,ϕ must be separately calculated such that it is correct for the element spacing (dg,H and dg,V). If AAθ,ϕ is not linked to the element spacing, then the calculated absolute gain may diverge from the correct value in a manner that varies as the beam is steered.

The correct composite array radiation pattern directivity(D) is given by:

DAθ,ϕ=10⋅log4πAAθ,ϕ2-ππ0πPθ,ϕ2sinθdθdϕ,

The composite array radiation pattern gain can then be calculated as:

GAθ,ϕ=DAθ,ϕ-L

Where L is the Loss associated with the antenna. This is currently included in the estimate for element gain AEθ,ϕ, and is 1.8dB.

#### 2.3.2	BS Antenna modelling

##### 2.3.2.1	Urban macro scenario

Table 2.3.2.1-1: BS antenna modelling for Urban macro scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=65°,SLAV=30dB |
| Antenna element horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=80°,Am=30dB |
| Combining method for 3D antenna element pattern (dB) | A″(θ″,ϕ″)=-min-AE,Vθ″+AE,Hϕ″,Am |
| Maximum directional gain of an antenna element GE,max | 8 dBi |
| (Mg, Ng, M, N, P) note | (1, 1, 4, 8, 2) |
| (dv, dh) | (0.9λ, 0.6λ) |
| Note: An additional 3dB gain is added to the total beamforming gain to account for the two polarization directions. Boresight direction is horizontal. |  |

##### 2.3.2.2	Dense urban scenario

Table 2.3.2.2-1: BS antenna element pattern for Dense urban scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=65°,SLAV=30dB |
| Antenna element horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=80°,Am=30dB |
| Combining method for 3D antenna element pattern (dB) | A″(θ″,ϕ″)=-min-AE,Vθ″+AE,Hϕ″,Am |
| Maximum directional gain of an antenna element GE,max | 8 dBi |
| (Mg, Ng, M, N, P) note | (1, 1, 4, 8, 2) |
| (dv, dh) | (0.9λ, 0.6λ) |
| Note: An additional 3dB gain is added to the total beamforming gain to account for the two polarization directions. Boresight direction is horizontal. |  |

##### 2.3.2.3	Indoor scenario

Table 2.3.2.3-1: BS antenna element pattern for Indoor scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=65°,SLAV=25dB |
| Antenna element horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=80°,Am=25dB |
| Combining method for 3D antenna element pattern (dB) | A″(θ″,ϕ″)=-min-AE,Vθ″+AE,Hϕ″,Am |
| Maximum directional gain of an antenna element GE,max | 8 dBi |
| (Mg, Ng, M, N, P) note | (1, 1, 4, 8, 2) |
| (dv, dh) | (0.9λ, 0.6λ) |
| Note: An additional 3dB gain is added to the total beamforming gain to account for the two polarization directions. Boresight direction is perpendicular to the ceiling. |  |

#### 2.3.3	UE antenna element pattern

Some sort of UE beamforming is required to compensate for the higher propagation loss in this frequency range compared to that in FR1, but the required beamforming gain is less than that in FR2. Therefore, both omnidirectional antenna with 0dBi gain and the antenna element pattern given in Table 2.3.3-1 are used.

Table 2.3.3-1: UE antenna element pattern

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=90°,SLAV=25dB |
| Antenna element horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=90°,Am=25dB |
| Combining method for 3D antenna element pattern (dB) | A″(θ″,ϕ″)=-min-AE,Vθ″+AE,Hϕ″,Am |
| Maximum directional gain of an antenna element GE,max | 5 dBi |
| (Mg, Ng, M, N, P) | (1, 1, 2, 2, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| UE orientation | Random orientation in the azimuth domain: uniformly distributed between -90 and 90 degrees* Fixed elevation: 90 degrees |
| NOTE: This is done to emulate two panels: the configuration is equivalent to 2 panels with 180 shift in horizontal orientation and UE orientation uniformly distributed in the azimuth domain between -180 and 180 degrees. |  |

### 2.4		Transmission power control model

For downlink scenario, no power control scheme is applied.

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 is applied with following parameters.

-	CLx-ile = 88 + 10*log10(200/X), where X is UL transmission BW (MHz)

-	γ = 1

### 2.5		Received power model

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

where:

RX_PWR is the received power

TX_PWR is the transmitted power

G_TX is the transmitter antenna gain (directional array gain)

G_RX is the receiver antenna gain (directional array gain).

### 2.6		ACLR and ACS modelling

For DL it seems reasonable from the perspective of simulating worst case scenarios that we assume BS ACLR is modelled as flat in space, and the UE ACS can be modelled flat in space.

If this assumption is for DL, then the similar assumption could be made for the UL because:

-	UE has a much small number of antennas, thus the effect of directivity should be smaller for ACLR (or the adjacent channel interference). It can also be reasonably assumed that the UE ACLR will play a dominant role than the BS ACS in the adjacent channel interference.

-	Again, BS ACS flat in space might mean worse coexistence performance than actual performance because BS has better capability of steering its receive antennas to suppress interference.

If a UE occupies a smaller bandwidth than the channel bandwidth for transmission, a two stop ACLR model could be considered in frequency to avoid overly estimating interference, as done in E-UTRA coexistence study (as recorded in TR 36.942).

Therefore, it is assumed that both ACLR (or the adjacent channel interference) and ACS are flat in both space and frequency. The ACIR model can be express as

(assuming ACLR, ACS and ACIR to be linear).

### 2.7		Link level performance for 5G NR coexistence

The throughput of a modem with link adaptation can be approximated by an attenuated and truncated form of the Shannon bound. (The Shannon bound represents the maximum theoretical throughput than can be achieved over an AWGN channel for a given SNIR). The following equations approximate the throughput over a channel with a given SNIR, when using link adaptation:

Throughput SNIR, bps/Hz =0                                  for SNIR< SNIRMIN                                        ∝∙SSNIR                     for SNIRMIN≤SNIR<SNIRMAX ∝∙SSNIRMAX               for SNIR ≥ SNIRMAX

Where:

S(SNIR)		Shannon bound, S(SNIR) =log2(1+SNIR) bps/Hz
			Attenuation factor, representing implementation losses
SNIRMIN		Minimum SNIR of the code set, dB
SNIRMAX		Maximum SNIR of the code set, dB

The parameters α, SNIRMIN and SNIRMAX can be chosen to represent different modem implementations and link conditions. The parameters proposed in Table 2.7-1 represent a baseline case, which assumes:

-	1:1 antenna configuration

-	AWGN channel model

-	Link Adaptation (see Table 2.7-1 for details of the highest and lowest rate codes)

-	No HARQ

Table 2.7-1: Parameters describing baseline Link Level performance for 5G NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation | 0.6 | 0.4 | Represents implementation losses |
| SNIRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SNIRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

Note that the parameters proposed in Table 2.7-1 are targeted for eMBB coexistence scenario.

### 2.8		Other simulation parameters

Table 2.8-1: Other simulation parameters

| Parameters | Indoor | Urban macro | Dense urban |
| --- | --- | --- | --- |
| Carrier frequency | 7GHz, 10GHz | 7GHz, 10GHz | 7GHz, 10GHz |
| Channel bandwidth | 100MHz | 100MHz | 100MHz |
| Scheduled channel bandwidth per UE (DL) | 100MHz | 100MHz | 100MHz |
| Scheduled channel bandwidth per UE (UL) | 100MHz | 100MHz | 100MHz |
| The number of active UE (DL) | Same as the number of BS beam | Same as the number of BS beam | Same as the number of BS beam |
| The number of active UE (UL) | Same as the number of BS beam | Same as the number of BS beam | Same as the number of BS beam |
| Traffic model | Full buffer | Full buffer | Full buffer |
| DL power control | NO | NO | NO |
| UL power control | YES | YES | YES |
| BS max TX power in dBm | 23dBm | 43dBm | 33dBm |
| UE max TX power in dBm | 23dBm | 23dBm | 23dBm |
| UE min TX power in dBm | -40dBm | -40dBm | -40dBm |
| BS Noise figure in dB | 7 (Note 1) | 7 (Note 1) | 7 (Note 1) |
| UE Noise figure in dB | 9 (Note 1) | 9 (Note 1) | 9 (Note 1) |
| Handover margin | 3dB | 3dB | 3dB |
| Note 1: BS and UE noise figures for the 10GHz sample frequency in TR 38.820 |  |  |  |

## 2.9		Co-existence simulation methodology

Adopt following simulation steps.

1.	Aggressor and victim network are generated.

-	UEs are distributed randomly across the network.

2.	UE associations: UEs are associated to base station based on coupling loss.

-	Associations are made assuming a single element at both UE and BS.

3.	Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS-UE. This is done for both victim and aggressor networks.

4.	Throughput is computed in the victim systems without considering ACI as below:

-	ThputNO ACIbpshz=fSINRICI=fSN+IICI, where IICI is the inter-cell interference.

5.	Throughput is computed considering ACI as below:

-	ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where IACI is the adjacent channel interference.

6.	RF parameters are determined based on the degradation cause by ACI as below:

-	LossACI=1-ThputACIThputSINGLE.

# 3.	Conclusion

This contribution has proposed the system level simulation methodology and assumptions for coexistence study on the ACLR and ACS of NR BS and UE for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz. It is proposed to approve the system level simulation methodology and assumptions in section 2 to perform the coexistence studies to provide answers to ITU-R WP5D on these parameters in these frequencies with sound technical justifications from coexistence perspective.

# References

[1]	RP-200513, “New SI proposal: Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz”, Ericsson, Huawei, HiSilicon.

[2]	RP-200042, “LS on parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23”, ITU-R WP5D.

[3]	3GPP TS 38.101-1 v16.3.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[4]	3GPP TS 38.101-2 v16.3.0, “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

[5]	3GPP TS 38.104 v16.3.0, “NR; Base Station (BS) radio transmission and reception”.

[6]	3GPP TR 38.803 v14.2.0, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”.

[7]	3GPP TR 38.901 v16.1.0, “Study on channel model for frequencies from 0.5 to 100 GHz”.

[8]	RP-182783, “LS on Characteristics of IMT-Advanced, IMT-2020 and Advanced Antenna Systems for ITU-r sharing and compatibility studies in the frequency band 3 300-3 400 MHz”, TSG RAN.