3GPP TSG-RAN4#94bis-e	R4-2004999

20th Apr – 30th Apr 2020

Source: 	Nokia, Nokia Shanghai Bell

Title: 					FR2 new MPR and modifiedmpr-signaling

Agenda Item: 			6.14.1.6

Document for:	Discussion

# 1	Introduction

In RAN4#93 FR MPR requirement was enhanced [1] but unfortunately modifiedMPR table was not updated. In this contribution we discuss potential flaw in NR modifiedMPR signalling structure.

# 2	Discussion

In LTE there were 32-bit bitmap available for UE to indicate that it supports new MPR or A-MPR definition. This bit map is common for all bands. See below.

Table H.1-1: Definitions of the bits in the field modifiedMPRbehavior

| Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- |
| 0 (leftmost bit) | - The MPR for intra-band contiguous carrier aggregation bandwidth class C with non-contiguous resource allocation specified in Clause 6.2.3A in version 12.5.0 of this specification | - This bit shall be set to 1 by a UE supporting intra-band contiguous CA bandwidth class C |
| 1 | - The A-MPR associated with NS_05 for Band 1 in Clause 6.2.4 in version 12.10.0 of this specification. | - This bit shall be set to 1 by a UE supporting A-MPR associated to NS_05 for Band 1. |
| 2 | The A-MPR associated with NS_04 for Band 41 in Table 6.2.4-4 in version 14.1.0 of this specification. | - This bit shall be set to 1 by a power class 3 UE supporting A-MPR associated to NS_04 for Band 41. |
| 3 | The A-MPR associated with NS_31 for Band 41 in Table 6.2.4-26 in version 15.3.0 of this specification. | - This bit shall be set to 1 by a UE supporting A-MPR associated to NS_31 for Band 46. |

In NR however the bitmap which is 8-bits is per band. Below example from -3 as -1 or -2 does not have such table yet.

Table H.1-1: Definitions of the bits in the field modifiedMPRbehavior

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n41 | 0 (leftmost bit) | - EN-DC contiguous intraband MPR as defined in clausue 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit may be set to 1 by a UE supporting DC_(n)41AA UE EN-DC |
|  | 1 | - EN-DC non-contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 v15.5.0 | - This bit may be set to 1 by a UE supporting DC_41A_n41A EN-DC |
| n71 | 0 (leftmost bit) | - EN-DC contiguous intraband MPR as defined in clausue 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit may be set to 1 by a UE supporting DC_(n)71AA UE EN-DC |

Now because [1] FR2 MPR was enhanced we need to add modifiedMPRbehavior-table to -2 but the problem is that as MPR is common to all bands we need to reserve one bit for enhanced MPR for all FR2 bands. This is bit unfortunate, perhaps there would be better way?

Observation: RAN4 needs to discuss how to indicate enhanced FR 2 MPR in modifiedMPRbehavior-table.

# 4	References

[1] R4-1915755	CR to 38.101-2-g10 Corrections to maximum output power reduction for power class 3	Skyworks Solutions Inc., RAN4#93