3GPP TSG-RAN4#94bis-e	R4-2005014

20th Apr – 30th Apr 2020

Source: 	Nokia, US Cellular

Title: 					TP to TR 37.716-31-11: Addition of DC_5A-48A_(n)12AA

Agenda Item: 			8.5.2

Document for:	Approval

# Introduction

This is a TP to introduce EN-DC configuration C_5A-48A_(n)12AA into TR 37.716-31-11.

***************************** Start of the TP **********************************

## 5.1.X		DC_5-48_(n)12

### 5.1.X.1	Operating bands for EN-DC

Table 5.2B.4.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_5-48_(n)12 | CA_5-12-48 | n12 |  |

### 5.1.X.2	Configuration for EN-DC

Table 5.5B.4.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| C_5A-48A_(n)12AA | DC_5A_n12A DC_48A_n12A DC_(n)12AA1 | CA_5A-12A-48A | n12 |
| NOTE1: Only single switched UL is supported |  |  |  |

### 5.1.X.3	∆TIB and ∆RIB values

Based on CA_5-12-48following is proposed

Table 6.2B.4.2.3.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-48_(n)12 | 5 | 0.8 |
|  | 12 | 0.4 |
|  | 48 | 0.3 |
|  | n12 | 0.8 |

Table 7.3B.3.3.3-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-48_(n)12 | 5 | 0.5 |
|  | 12 | 0.3 |
|  | 48 | 0 |
|  | n12 | 0.5 |

***************************** End of the TP **********************************