3GPP TSG-RAN WG4 Meeting #94e Bis	R4-2005382

Electronic Meeting, April 20 ~30, 2020

Agenda item:			4.9

Source:	Moderator (Huawei, HiSilicon)

Title:	Email discussion summary for [94e Bis][101] NR_NewRAT_RRM_Core

Document for:	Information

# Introduction

According to RAN4 Chairmen arrangement, this contribution provides the summary of topics of Rel-15 NR RRM Core maintenance under agenda 4.9.

List of candidate target of email discussion for 1st round and 2nd round.

- 1st round: Invite companies to provide the comments for the discussion paper and CRs. According to comments, the possible way forward will be suggested. Based on the possible way forward, the Chair can allocate the Tdoc numbers for way forward or CRs to the responsible companies by the deadline of the first round.

- 2nd round: The responsible companies are expected to provide the way forward or revised/new CRs as soon as possible by capturing the comments in the first round, and companies are encouraged to review them again. The comments in the 2nd round will be captured in this summary. If no further comment for the way forward or CRs, the moderator will report that those documents are agreeable to the Chair. If there is still controversial issues, the moderator will capture the issues and opinions from the companies for the further discussion in the next meetings.

# Topic #1: UE measurement capability

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2004100 | ZTE | Discussion on measurement reporting criteria for EN-DC In this contribution, we provide our views on the questions in the RAN2 LS on measurement reporting criteria for EN-DC operation. Based on the observations following proposals are present. Proposal 1. Reporting criteria for NR serving cell frequencies, i.e. component 9×n in Ecat,EN-DC,NR, needs to be coordinated between the MN and the SN in EN-DC operation. Proposal 2. RAN4 should answer questions in the LS directly without additional information. A companion reply LS [3] is provided. |
| R4-2004431 | Nokia | Reporting Criteria discussion In this paper we discuss the RAN4 aspect of the reporting criteria based on the RAN2 LS in [1]. Based on the discussion we observe: Observation 1: There is a need to exchange information impacting the reporting criteria configuration, between MN and SN. Additionally, we provide a TP for an LS reply and a draft LS in [2] |
| R4-2004652 | Ericsson | On measurement reporting criteria with EN-DC - Observation 1: From the latest TS 38.133, it follows that Ecat,EN-DC,NR is the total number of NR reporting criteria configured by PSCell and E-UTRA PCell, which means that if PCell and PSCell are configuring on the same serving NR carrier frequency, the PCell and PSCell may need to be aware of the still available number of reporting criteria to not exceed the limit specified in TS 38.133. - Observation 2: The above observation does not come from the clarifying CR in R4-1907862, rather this approach had been already in both TS 38.133 and TS 36.133. Based on the above observations, a draft response LS is provided in [3]. |
| R4-2004102 | ZTE | Reply LS on measurement reporting criteria for EN-DC |
| R4-2004432 | Nokia | LS on UE reporting criteria |
| R4-2004653 | Ericsson | Response LS on measurement reporting criteria for EN-DC |
| R4-2004101 | ZTE | Remaining issues on NR reporting criteria In this contribution, we provide our views on remaining issues on reporting criteria for NR. Based on the observations following proposals are present. Proposal 1. The reporting criteria for EN-DC and NE-DC when E-UTRA SCell(s) are configured is to be specified. Proposal 2. The requirements structure for reporting criteria in TS36.133 is not changed by introducing requirements for CA at E-UTRA side. Proposal 3. Reporting criteria for EN-DC is 36+9*n when the UE is configured with E-UTRA SCell(s), and n is the number of E-UTRA SCell carrier frequencies. Proposal 4. For NE-DC, the total number of E-UTRA reporting criteria is Ecat,NE-DC,E-UTRA=10+9×n, and n is the number of configured E-UTRA serving frequencies, including PSCell and SCells carrier frequencies. |
| R4-2004433 | Nokia | Reporting Criteria in 36.133 In this paper we discuss the need to clarify the number of reporting criteria in 36.133 covering when a UE configured with EN-DC is configured with more LTE SCell’s or NR SCell’s. It was discussed that currently, the requirements in 36.133 does not cover these options, and provide text proposals how this could be captured. Observation 1: it needs to be clarified what the reporting criteria is for an EN-DC capable UE configured with additional SCells. We propose: Proposal 1: For each configured SCell the UE shall support additionally 9 reporting criteria. Proposal 2: UE requirement for reporting criteria when UE is configured with SCells and NR SCells need to be clarified. Proposal 3: RAN4 to select one of the text proposals for clarifying the UE reporting criteria requirement when configured with SCells and NR SCells. |
| R4-2004651 | Ericsson | On reporting criteria with NR - Observation: Reporting criteria for all NR deployments are calculated based on equations in section 9.1.4.2, TS 38.133. Such equations did not exist in LTE, so listing explicitly the numbers outside the tables in Section 8.2.2, TS 36.133, was simply convenient. - Proposal 1: RAN4 to discuss and choose between two approaches: - Approach 1: only specify explicitly the number of reporting criteria for the scenario without SCells and PSCell, but refer to equations in TS 38.133 for deriving the number of additional reporting criteria for any other scenario with any number of SCell and/or PSCell. - Approach 2: specify total numbers of reporting criteria for different configurations, including NR and LTE carriers. EN-DC: - Proposal 3 (Approach 1): For EN-DC, the total number of mandatory reporting criteria is captured in TS 36.133 as follows: - 36 reporting criteria if the UE is not configured with any (LTE) SCell or NR SCell or NR PSCell carrier frequencies, - When the UE is configured with at least one of NR PSCell or at least one of NR SCells, the number of additional reporting criteria is Ecat,EN-DC,NR specified in Section 9.1.4.2 of TS 38.133, - When the UE is configured with at least one (LTE) SCell, the number of additional reporting criteria is derived based on Table 8.2.2-1 in TS 36.133. - Proposal 4 (Approach 2): For EN-DC, the total number of mandatory reporting criteria is captured in TS 36.133 as follows: - 36 reporting criteria if the UE is not configured with any (LTE) SCell or NR SCell or NR PSCell carrier frequencies, - 36+(10+9*1) reporting criteria if the UE is not configured with any (LTE) SCell or NR SCell but configured with one NR PSCell carrier frequency, - 36+9*k reporting criteria if the UE is configured with k carrier frequencies with (LTE) SCells, but no carrier frequencies with NR PSCell or NR SCells, - 36+9*k+(10+9*n) reporting criteria if the UE is configured with k carrier frequencies with (LTE) SCells, one NR PSCell carrier frequency, and (n-1) carrier frequencies with NR SCells. NE-DC: - Proposal 5 (Approach 1): For NE-DC, the total number of mandatory reporting criteria is captured in TS 36.133 as follows: - 29 reporting criteria if the UE is not configured with any (LTE) SCell or NR SCell or NR PSCell carrier frequencies, - When the UE is configured with at least one of (LTE) PSCell or at least one of (LTE) SCells, the number of additional reporting criteria is derived based on Table 8.2.2-1 in TS 36.133, - When the UE is configured with at least one NR SCell, the number of additional reporting criteria is (Ecat,EN-DC,NR-10), where Ecat,EN-DC,NR is specified in Section 9.1.4.2 of TS 38.133. - Proposal 6 (Approach 2): For NE-DC, the total number of mandatory reporting criteria is captured in TS 36.133 as follows: - 29 reporting criteria if the UE is not configured with any LTE SCell or LTE PSCell or NR SCell, but configured with NR PCell, - 29+(10+9*1) reporting criteria if the UE is not configured with any LTE SCell or NR SCell, but configured with LTE PSCell and NR PCell, - 29+9*n reporting criteria if the UE is configured with n carrier frequencies with NR SCells, but no carrier frequencies with LTE PSCell or LTE SCells, - 29+(10+9*k)+9*n reporting criteria if the UE is configured with (k-1) LTE SCells, LTE PSCell, and n NR SCells carrier frequencies. Based on the above, a draft CR is provided in [1], based on Approach 1. |
| R4-2004103 | ZTE | draft CR to 36.133 on NR reporting criteria - Specified reporting criteria for EN-DC when E-UTRA SCell carrier frequencies are configured. - Specified reporting criteria for NE-DC when E-UTRA SCell carrier frequencies are configured. - Change ‘excluding’ to ‘in addition to’ - Change the property of Table 8.2.2-1 so it can be on the same page with the title. - Editorial changes |
| R4-2004434 | Nokia | Draft CR to 36.133 for NR reporting criteria Clarification and defining UE requirements for reporting criteria for a UE supporting EN-DC and NE-DC. |
| R4-2004650 | Ericsson | Reporting criteria with NR Removed editor’s note and updated the reporting criteria for EN-DC and NE-DC accordingly |
| R4-2004812 | Ericsson | Reporting criteria with NR Clarifying the reference to reporting criteria in TS 36.133 |
| R4-2003599 | Mediatek | CR on TS38.133 for modification on number of cells and number of SSB to be measured for FR2 intra-freq. measurement (Section 9.2.3 Clarify that UE is only required to conduct the neigboring cell measurement on 1 serving carrier in a FR2 band. |

## Open issues summary

### Sub-topic 1-1

Issue 1-1: LS reply for need of coordination between MN and SN for 9×n in reporting criteria

- Proposals for reply LS

  - Option 1 (ZTE R4-2004100):

    - Therefore RAN4 confirms that component 9×n in E_(cat,EN-DC,NR) needs to be coordinated between the MN and the SN in EN-DC operation.

  - Option 2 (Nokia R4-2004432): There is a need to exchange information impacting the reporting criteria configuration, between MN and SN.

    - RAN4 has been discussing the question raised in the LS and has concluded that regarding question 1:

      - There is a need to exchange of information between MN and SN related to configurations impacting the component 9×n in E_(cat,EN-DC,NR).

      - This follows from that TS 38.133 E_(cat,EN-DC,NR) includes the total number of NR reporting criteria configured by the NR PSCell and E-UTRA PCell,

      - Note: the UE requirements defined by the TS 38.133 and TS 36.133 specifications is unaffected and remain as before the CR in R4-1907862 was agreed.

  - Option 3 (Ericsson R4-2004653):

    - From the latest TS 38.133, it follows that is the total number of NR reporting criteria configured by PSCell and E-UTRA PCell, which means that if PCell and PSCell are configuring on the same serving NR carrier frequency, the PCell and PSCell may need to be aware of the still available number of reporting criteria to not exceed the limit specified in TS 38.133.

    - The above observation does not come from the clarifying text in the CR in R4-1907862 (mentioned in the RAN2 LS), rather this approach had been already in both TS 38.133 and TS 36.133:

      - Ecat,EN-DC,NRhad been already specified to include the PSCell and SCells carrier frequencies in TS 38.133, and

      - Inter-RAT NR carrier frequency carrier reporting criteria in TS 36.133 had been specified to be only applicable for UE with this capability and measurements on any of the NR carrier frequencies other than the carrier frequency of the NR PSCell or NR SCell.

- Recommended WF

  - To answer RAN2 LS, it is proposed to agree on

    - There is a need to exchange information impacting the reporting criteria configuration, between MN and SN.

    - The UE requirements defined by the TS 38.133 and TS 36.133 specifications is unaffected and remain as before the CR in R4-1907862 was agreed.

  - Further discussion on the text of draft LS

### Sub-topic 1-2

Issue 1-2: Reporting criteria for EN-DC/NE-DC with more than one LTE and/or NR SCells configured

- Proposals for CR for EN-DC

  - Option 1 (ZTE R4-2004103) :

…the UE shall always support in parallel), the UE need not support more than the number of reporting criteria, excluding in addition to reporting criteria specified in TS 38.133 [50] that are applicable for the UE configured with EN-DC operation, as follows:

-	[36] reporting criteria if the UE is not configured with any SCell or PSCell carrier frequency or NR SCell or NR PSCell carrier frequency,

-	[36] reporting criteria if the UE is not configured with any SCell or NR SCell but configured with one NR PSCell carrier frequency.

-	[36+9×n] reporting criteria if the UE is configured with SCells and one NR PSCell carrier frequencies, and n is the number of configured SCells carrier frequencies.

  - Option 1a (Nokia R4-2004434):

…the UE shall always support in parallel), the UE need not support more than the number of reporting criteria, excluding in addition to reporting criteria specified in TS 38.133 [50] that are applicable for the UE configured with EN-DC operation, as follows:

-	[36] reporting criteria if the UE is not configured with any SCell, or PSCell, carrier frequency or NR SCell or NR PSCell carrier frequency,

-	[36] reporting criteria if the UE is not configured with any SCell or NR SCell but configured with one NR PSCell carrier frequency.

-	[36+9×n] reporting criteria if the UE is configured with n SCells and with one NR PSCell carrier frequency.

-	[36+9×n] reporting criteria if the UE is configured with n SCells and with one NR PSCell carrier frequency and with one or more NR SCell carrier frequencies.

  - Option 2 (Ericsson R4-2004650):

The UE, which is capable of supporting EN-DC operation with NR PSCell and one or more NR carrier frequencies in total, shall be able to support in parallel per category up to Ecat reporting criteria according to table 8.2.2-1 and according to TS 38.133 [50, Section 9.1.4]. For the measurement categories belonging to measurements on: E-UTRA intra-frequency cells, E-UTRA inter-frequency cells, inter-RAT per supported RAT, and NR cells on serving and non-serving carrier frequencies (i.e. without counting other categories that the UE shall always support in parallel), the UE need not support more than the number of reporting criteria in total, excluding reporting criteria as specified in TS 38.133 [50] that are applicable for the UE configured with EN-DC operation, as follows:

-	[36] reporting criteria if the UE is not configured with any SCell or PSCell carrier frequency or NR SCell or NR PSCell carrier frequencies,

-	[36] reporting criteria if the UE is not configured with any SCell or NR SCell but configured with one NR PSCell carrier frequency

-	When the UE is configured with at least one of NR PSCell or at least one of NR SCells, the number of additional reporting criteria is Ecat,EN-DC,NR specified in Section 9.1.4.2 of TS 38.133 [50],

-	When the UE is configured with at least one SCell, the number of additional reporting criteria is derived based on Table 8.2.2-1.

- Proposals for CR for NE-DC

  - Option 1 (ZTE R4-2004103) :

Option 1a (Nokia R4-2004434):

  - Option 2 (Ericsson R4-2004650):

- Recommended WF

  - Agreement: UE requirement for reporting criteria for EN-DC/NE-DC when UE is configured with SCells and NR SCells need to be clarified.

  - Further discussion on how to modify the criteria:

    - Alternative 1: specify explicitly the number of reporting criteria with SCells in addition to the criteria specified in TS38.133.

    - Alternative 2: only specify explicitly the number of reporting criteria for the scenario without SCells and PSCell, but refer to equations in TS 38.133 for deriving the number of additional reporting criteria for any other scenario with any number of SCell and/or PSCell.

    - Alternative 3: specify total numbers of reporting criteria for different configurations, including NR and LTE carriers.

  - Decide which CR can be used as baseline.

### Sub-topic 1-3

For CR R4-2003599 and R4-2004812, please provide the comments directly in Section 1.3.2.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Erisson | Sub topic 1-1: We agree to reply to RAN2 on the need for coordination. The statement that “ The UE requirements defined by the TS 38.133 and TS 36.133 specifications is unaffected and remain as before the CR in R4-1907862 was agreed.” Is a step in the right direction but the wording “unaffected” is confusing. The CR changed the requirements, but didn’t introduce the need for coordination. So a better wording would be something like “ The need for coordination was already implied by RAN4 requirements before the CR in R4-1907862 was agreed. “ Sub topic 1-2: Regardless if the wording is “excluding” or “in addition to” it is easily misinterpreted. So our view is that it would be better to an explicit reference to “Ecat,EN-DC,NR as defined in clause 9.1.4.2 of TS 38.133[50]” in this CR for 36.133. …. Others: |
| ZTE | Sub topic 1-1: The wording below as in option 2 would be fine with us as a compromise to move forward. - There is a need to exchange of information between MN and SN related to configurations impacting the component 9×n in Ecat,EN-DC,NR. - This follows from that TS 38.133 Ecat,EN-DC,NR includes the total number of NR reporting criteria configured by the NR PSCell and E-UTRA PCell, However to adding additional note as in option 2 is not necessary in our view. We are still discussing RAN4 requirements. Sub topic 1-2: EN-DC: We are aligned with Nokia generally. The difference is about NR SCell. The reporting criteria for NR SCell are specified in 38.133 as component 9xn. It will not impact requirements in 36.133. As can be seen in option 1a, the requirements with/without NR SCell are the same. To simplify requirements we prefer option 1, which is similar to Nokia Text proposal 3 in 4433. For the description part regarding the total number, option 2 can be further revised to …as specified in TS 38.133 [50] for UE configured with EN-DC. Then we can accept the revised description part. For the requirements below the description part we don’t agree option 2. When the UE is configured with at least one of NR PSCell or at least one of NR SCells, the number of additional reporting criteria is Ecat,EN-DC,NR specified in Section 9.1.4.2 of TS 38.133 [50], This requirements have already been specified in 38.133 - When the UE is configured with at least one SCell, the number of additional reporting criteria is derived based on Table 8.2.2-1. The requirements should be clearly specified as in LTE SA, NR SA. Otherwise everything can be derived based on Table for Ecat. NE-DC: For the structure of requirements we are aligned with Nokia. Option 1a and 1b should be used. Same comments as for EN-DC above regarding description part. Revision of description part of option 2 would be acceptable to us. …as specified in TS 38.133 [50] for UE configured with NE-DC. For the number of reporting criteria, it is not clear how 24 and 29 are derived. Our proposal 19 comes from: For single carrier case, the number of reporting criteria should be 19 by taking inter frequency and intra frequency E-UTRA measurements into consideration as E-UTRA PSCell can only configure E-UTRA measurements, which is intra-frequency E-UTRA measurements and inter-frequency E-UTRA measurements. When CA is configured the number of reporting criteria is scaled by number of serving frequencies, similar to EN-DC case. |
| Nokia, Nokia Shanghai Bell | Sub-topic 1-1: Issue 1-1: LS reply for need of coordination between MN and SN for 9×n in reporting criteria If agreeable to both Ericsson and ZTE, we can volunteer to try work on the LS text once again in this meeting. Maybe we can find a compromise. Sub-topic 1-2: Issue 1-2: Reporting criteria for EN-DC/NE-DC with more than one LTE and/or NR SCells configured If one of the proposed TPs in the CRs is acceptable to all this is of course good. Otherwise we can agree to the proposal in WF to look at a better way to express the requirements. One option is as in option 3 where we use a specific expression as used in 38.133. As for the number we basically added all minimum reporting criteria together (intra-f, inter-f etc.) which are applicable in table 8.2.2-1. NE-DC: 10 intra-f + 1 intra-f RSTD1 + intra-f E-CID + 10 inter-f + 1 inter-f RSTD which then only equals 23 (needs to be updated). Detailed numbers needs to checked (can have them in []). |

### CRs/TPs comments collection

The CRs included in the above sub-topics are not listed here.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2003599 | Nokia: section 9.1.5.1 is about Monitoring of multiple layers outside gaps. the requirements related number of cells and SSBs are in 9.2.3. RAN4 has not agreed to this limitation. In fct RAN4 spend quite a long time discussing and agreeing on the current requirements. We cannot agree to this CR and there is no reason to come back in next meeting. |
|  | Company B |
| R4-2004812 | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | R4-2004432 can be used as baseline. Companies had different views on whether the note is needed in R4-2004432 and more discussion is needed. Tentative agreements: Option 2 and R4-2004432 can be used as the baseline for the second round discussion. Nokia will lead the LS drafting. Candidate options: Recommendations for 2nd round: Further discussion on whether the note in R4-2004432 as part of option 2 is needed and/or how to improve it. |
| Sub-topic #1-2 | The situation is similar as that in the previous meeting. There are two solutions: one is to refer to total number while the other is to separately specify the number for LTE. One company suggested considering the other option, which is to use the same expression as 38.133 in 36.133. Tentative agreements: N/A Candidate options: In addition to the existing options, more option is added Option 3: Use a specific expression as used in 38.133. Recommendations for 2nd round: Prepare the way forward to drive compromise. According to moderator understanding, the total number of criteria for EN-DC/NE-DC is specified and number of criteria for a single SCell is also specified, but they are distributed in different sections or specifications. So although the readability is not good without any change, the number of criteria can be calculated. One possible candidate solution is to follow the direction provided by Nokia. Maybe we can explicitly specify the number for LTE with n SCell and at the same time cite or refer to 38.133 or other section. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | Revise R4-2004432 Draft LS on UE reporting criteria | Nokia |
| #2 | New WF WF on remaining issue on NR reporting criteria | ZTE |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs/LS-s Status update

| CR/TP/LS number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2004102 | Noted |
| R4-2004432 | Revised |
| R4-2004653 | Noted |
| R4-2004103 | Return to If there was an agreement, this CR will be revised to capture the agreement. |
| R4-2004434 | Return to |
| R4-2004650 | Return to |
| R4-2003599 | Return to One companies disagreed with the CR. The response from proponent is needed. |
| R4-2004812 | Endorsed. There is no comment until the deadline of 1st round. |

## Discussion on 2nd round (if applicable)

Moderator tries to summarize discussions for the 2nd round.

|  | Status summary |
| --- | --- |
| R4-2005266 | WF on the remaining issues on Rel-15 NR reporting criteria The draft was uploaded. No comment received until April 29. It can be approved. |
| R4-2005265 | Revised from R4-2004432. Draft LS on UE reporting criteria The draft was uploaded very late. But no comment was received. |
| R4-2004103 | draft CR to 36.133 on NR reporting criteria The revised version was uploaded. No comment received until April 29. It can be endorsed. It needs revised number. |
| R4-2004434 | Draft CR to 36.133 for NR reporting criteria No discussion is organized for this document. It is suggested to be noted. |
| R4-2004650 | Reporting criteria with NR No discussion is organized for this document. It is suggested to be noted. |
| R4-2005411 | Revised from R4-2003599. CR on TS38.133 for modification on number of cells and number of SSB to be measured for FR2 intra-freq. measurement (Section 9.2.3) The revised version was uploaded. ------------------------------------------------------------------------------------------- [Nokia] As discussed offline and also mentioned in round 1 we cannot agree to this CR in 5411. |
| R4-2004812 | Reporting criteria with NR The document was endorsed after the 1st round, but changed to “return to” afterwards due to comment from company. --------------------------------------------------------------------------------------------- [ZTE] I provided comments to email thread [101] on the CR R4-2004812. This CR is for 38.133 and releated to ongoing discussion on reporting criteria in 36.133. |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2005266 | Approved. |
| R4-2005265 | Approved (?) The draft was uploaded very late. But no comment was received. |
| R4-2005429 | Endorsed. Revised from R4-2004103. |
| R4-2004434 | Not pursued |
| R4-2004650 | Not pursued |
| R4-2005411 | Not pursued. Revised from R4-2003599 |
| R4-2004812 | Not pursued |

# Topic #2: RRM measurement and measurement gap

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2003088 | Ericsson | Requirements on measurements outside gaps for FR2 Proposal 1: The requirements in this clause for FR2 measurement objects apply provided that the following conditions are met Either: - There are only SCells configured for FR2 Or: - The same SMTC offset is used for different CC on FR2 and: - If smtc2 is configured on any FR2 CC, all CCs have the same periodicity for smtc1, and all CCs configured with smtc2 have the same periodicity for smtc2 - If smtc2 is not configured on any FR2 CC, the total number of different SMTC periodicities on all CCs does not exceed 2 |
| R4-2004332 | Huawei, HiSilicon | Discussion on FR2 measurement outside gap In this paper we provided our views on the SMTC alignment for FR2 intra-frequency measurement. Proposal: Adopt option 1 as the compromised solution for FR2 SMTC alignment issue. - Case 1: If there is PCC or PSCC in FR2 - The same SMTC offset is used for different CC on FR2 and: - If smtc2 is configured on any FR2 CC, all CCs have the same periodicity for smtc1, and all CCs configured with smtc2 have the same periodicity for smtc2 - If smtc2 is not configured on any FR2 CC, the total number of different SMTC periodicities on all CCs does not exceed 2 - Case 2: If there is only SCC in FR2 - No restriction on SMTC offset or periodicity on each CC. |
| R4-2004430 | Nokia | Discussion on SMTC configuration in FR2 In this paper we analysed the UE complexity when performing intra-frequency measurements in FR2 and the complexity in ensuring the UE minimum requirements. Based on the detailed analysis we conclude: Proposal 1: No limitations are introduced on the use of SMTC periodicities for intra-frequency carriers. Proposal 2: No limitations are introduced on the use of Offset. Proposal 3: Limit the use of SMTC2 for intra-frequency measurements in Rel-15. |
| R4-2003089 | Ericsson | (CR) Requirements on measurements outside gaps for FR2 Capture the restriction that non gap based measurement requirements apply, provided that the following conditions are met: Either: There are only SCells configured for FR2 Or: - The same SMTC offset is used for different CC on FR2 and: -If smtc2 is configured on any FR2 CC, all CCs have the same periodicity for smtc1, and all CCs configured with smtc2 have the same periodicity for smtc2 -If smtc2 is not configured on any FR2 CC, the total number of different SMTC periodicities on all CCs does not exceed 2 |
| R4-2004333 | Huawei, HiSilicon, Mediatek | CR on FR2 measurement requriements outside gaps R15 Define applicability for FR2 intra-freqeuncy measurement requirements. |
| R4-2003598 | Mediatek | CR on TS38.133 for modification of the layer 3 and layer 1 measurement sharing factor when both SSB and RSSI symbol to be measured are considered (Section 9.2.5.1) Revise the conditions for Klayer1_measurement =1, - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped by with any of the SSB symbols and the RSSI symbols, indicated by SSB-ToMeasure and 1 symbol before each consecutive SSB symbols and RSSI symbols indicated by SSB-ToMeasure and 1 symbol after each consecutive SSB symbols and RSSI symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure and SS-RSSI-Measurement isare configured, where SSB symbols are indicated by SSB-ToMeasure and RSSI symbols are indicated by SS-RSSI-Measurement; |
| R4-2004286 | Huawei, HiSilicon | Correction on gap pattern applicability in TS 36.133 Clarify that the appliability restriction of gap pattern #0, 1, 2, 3, 4, 6, 7, 8, 10 apply for ENDC, NEDC and LTE standalone operation. |
| R4-2004334 | Huawei, HiSilicon | CR to remove RSTD requirements for NE-DC in 36.133 R15 Remove intra-frequency RSTD measurement requirements for NE-DC from 36.133. Note that a companion CR R4-20xxxx to 38.133 is submitted to update inter-RAT RSTD measurement requirements in 38.133. |
| R4-2004335 | Huawei, HiSilicon | CR on inter-RAT RSTD requirements for NE-DC in 38.133 R15 Update the applicable LTE intra-frequency requriements (those for LTE SA) that apply for inter-RAT RSTD measurement configured by NR PCell on LTE serving frequency. |

## Open issues summary

### Sub-topic 2-1

Issue 2-1: SMTC alignment for FR2 intra-frequency measurement

The situation does not change too much compared to previous meeting. For the following proposal, Option 1 and Option 1a are similar.

- Proposals for conditions under which FR2 intra-frequency measurement requirements (Clause 9.1.5) apply

  - Option 1 (Ericsson R4-2003088, R4-2003089)

  - Option 1a (Huawei, HiSilicon, Mediatek, R4-2004332, R4-2004333)

    - Case 1: If there is PCC or PSCC in FR2

  - The same SMTC offset is used for different CC on FR2 and:

    - If smtc2 is configured on any FR2 CC, all CCs have the same periodicity for smtc1, and all CCs configured with smtc2 have the same periodicity for smtc2

    - If smtc2 is not configured on any FR2 CC, the total number of different SMTC periodicities on all CCs does not exceed 2

    - Case 2: If there is only SCC in FR2

  - No restriction on SMTC offset or periodicity on each CC

  - Option 2 (Nokia R4-2004430)

    - No limitations are introduced on the use of SMTC periodicities for intra-frequency carriers.

    - No limitations are introduced on the use of Offset.

    - Limit the use of SMTC2 for intra-frequency measurements in Rel-15.

- Recommended WF

  - It is proposed to agree on Option 1a.

### Sub-topic 2-2

For CR R4-2003598, R4-2004286, R4-2004334, and R4-2004335, please provide the comments directly in Section 2.3.2.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Sub topic 2-1: Option 1 and 1a appear totally identical with different wording to describe the same SMTC restrictions. Only real difference is the order that the two top level cases are presented in, and the change of description between “SCells only” or “PCC or PSCC in FR2”. As a network vendor in general we do not wish to introduce restrictions on configurations where RAN4 requierments apply, unless there is strong justification. In this instance, we think that the UE implementation aspects do justify the restrictions in option 1/1a, so we would like one of these options to be agreed rather than option 2, so that this long standing R15 issue can be closed. …. Others: |
| MTK | Issue 2-1: SMTC alignment for FR2 intra-frequency measurement Support Option 1 or 1a. Our very original proposal is to have one single SMTC configuration for all FR2 CCs in the same band. After the discussions in several meeting, we now can compromise to Option 1 or 1a. |
| Apple | Scheduling restriction is always assumed in FR2 for intra-frequency measurement. That means there is no motivation to configure different SMTC parameters, including offset and periodicity, for different CC, which can be PSCell or SCell. With this, a modified Option 1a is suggested - Case 1: If there is PCC or PSCC in FR2 - The same SMTC offset is used for different CC on FR2 and: - If smtc2 is configured on any FR2 CC, all CCs have the same periodicity for smtc1, and all CCs configured with smtc2 have the same periodicity for smtc2 - If smtc2 is not configured on any FR2 CC, all CC have the same periodicity for smtc1. the total number of different SMTC periodicities on all CCs should be the same - Case 2: If there is only SCC in FR2 - No restriction on SMTC offset or periodicity on each CC |
| Huawei, HiSilicon | Issue 2-1: SMTC alignment for FR2 intra-frequency measurement Similar comment as MTK above, we support option 1 or 1a as compromised solution. We also agree with Ericsson comment that option 1 and option 1a are technically same. We also support modified option 1a above from Apple. |
| Intel | Either option 1 or 1a can solve the problem. We are fine with recommended WF. |
| Qualcomm | We also support option 1 or 1a. |
| Nokia, Nokia Shanghai Bell | Sub-topic 2-1: Issue 2-1: SMTC alignment for FR2 intra-frequency measurement We are still not acknowledging the problem described and presented. Fundamentally nothing is broken or even claimed to be broken in the specification. Hence, updating a closed release with such fundamental UE limitation should not be done. Hence, we cannot agree to either options 1 or 1a. And we cannot support the WF. |
| NEC | Support option 1or 1a |

### CRs/TPs comments collection

The CRs included in the above sub-topics are not listed here.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2003598 | Nokia: We understand the issue raised but RLM, BFD and BCD is not performed on all serving cells in Rel-15. This needs to be addressed. |
|  | Company B |
| R4-2004286 | Nokia: Specification clarification could help. However, the new text can be misunderstood in term when which GPs apply. Additionally, LTE SA requirements should not be in 38.133. |
|  | Company B |
| R4-2004334 | MTK: We agreed with this CR that LTE RSTD measurement configured by LPP via NR PCell should be named as inter-RAT. |
|  | Company B |
| R4-2004335 | MTK: We agreed with this CR that LTE RSTD measurement configured by LPP via NR PCell should be named as inter-RAT. But we wonder if section 9.4.4 is a better place to put these clarification? |
|  | Huawei, HiSilicon: To MTK’s comment, we are also fine to move the change (on applicable requirements for RSTD measurement configured by LPP via NR PCell) to 9.4.4.1.1 and 9.4.4.2.1. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #2-1 | Totally 8 companies provided the comments. 7 companies supported Option 1 and Option 1a, which were viewed as almost identical. 1 company proposed the modification based on Option 1a. 1 company still have concern on putting any limitation. Tentative agreements: NA Candidate options: The following options can be further discussed: - Option 1a with Ericsson comments and modification suggestion from Apple. - Option 2 Recommendations for 2nd round: In moderator’s understanding, it seems common understanding among the chipset vendors on the conditions under which the requirements apply. It would be better to make it clear to provide guidance for deployment for operators and infra-vendors. The potential suggestion to address the concern on Option 1/1a would be to make the wording clearer that it is the condition under which the requirement works, and further discuss the requirements for which the proposed condition cannot be fulfilled in TEI-16 or Rel-17 WI. Hope that the company could give the compromise. Since there is a concrete comment for modification of Option 1a, it is suggested to revise the joint CR R4-2004333 to capture the comment. R4-2003089 is suggested to merge into R4-2004333. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2003089 | Merged. Merged into the revised version of R4-2004333. |
| R4-2004333 | Revised. Capture the comments from companies. |
| R4-2003598 | Return to Proponent needs respond to Nokia comment. |
| R4-2004286 | Return to Proponent needs respond to Nokia comments. |
| R4-2004334 | Endorsed 1 company supported it. No negative comment is received by deadline of 1st round. |
| R4-2004335 | Revised 1 company made comment, which is agreed to be captured. |

## Discussion on 2nd round (if applicable)

Moderator tries to summarize discussions for the 2nd round.

|  | Status summary |
| --- | --- |
| R4-2005267 | Revised from R4-2004333 Discussion is on-going. -------------------------------------------------------------------------------------- [Huawei] Based on the guidance from the moderator, please find the revised CR on FR2 SMTC alignment (R4-2004333) as linked below. It is for Issue 2-1 of the email discussion. The CR is still based on option 1/1a, and the revision is a simple wording update trying to describe cases more clearly. - To Ericsson: please kindly check if the wording is ok. - To Apple: I have not captured your suggested update to option 1a, because we understand it may not be acceptable to Ericsson based on earlier discussions. Hope this is ok for you. - To Nokia: we understand companies have different views, but we hope option 1/1a is an acceptable compromise for you. We are also open to add some note such as “no restriction to network configuration” or “longer measurement period is expected” if this can help to address your concern to some extent. [Apple] To move forward on this topic, we can compromise to your option 1a. [Ericsson] For Apple’s comments, I wanted to say that one use case of different SMTC period on different CC would be network power saving. The network is implicitly required to transmit SSBs at least the periodicity of the SMTC since UEs will be expecting an SSB to be available in each SMTC. So one motivation of two SSB periods would be to have an SMTC period for “higher performance/higher NW power usage” (eg for PCell) and another for “longer delays/lower NW power usage” (eg for all SCells). For the proposal from Huawei to address Nokia concerns below, we would be fine in principle with some addition such as a note. [Apple] Thanks for the discussion. We are fine with this CR as well. Just one follow-up question to Ericsson for clarification: regarding the motivation of the higher performance and lower performance usage from network perspective, our understanding is: in some scenarios, whether one specific CC is used for PCell or SCell is UE specific, that is, same CC may be used as different serving cell types for different UEs(e.g. CC1 may be PCC for UE#1 but it’s also SCC for UE#2). If that’s true, it looks like network cannot take advantage of power saving by avoiding transmit SSB in lower performance CCs because that lower performance CC might be high performance ones to the other UEs. Or will network in FR2 implement that the same serving cell type (PCell or SCell) applied for all the UEs in the coverage? Thanks! [Ericsson] Thanks for the follow up question. You are correct that the choice of PCell is UE specific as far as signalling is concerned, however if the network wants to move all UEs to use the same serving frequency as PCC (for example during times of light loading where there might only be a small number of active UE that does not create too much load on PCC) it may do that, for example with interfrequency handover. We need to avoid making limitations on how UEs can be configured, which might take away the possibility for power saving that hasn’t even been thought about yet, unless such limitations are needed to make UE implementation feasible. We think the CR is a good balance in this respect (which is also why we proposed the same in our CR). [Mediatek] Thanks for the good discussion. We are fine with the CR from Huawei. Regarding Nokia’s comment of 4 SMTC periodicities, it is also acceptable to us. If we try to merge Huawei’s change with Nokia’s comment, it will be like: The requirements in this clause apply provided that - There are only SCCs in FR2, or - The SMTC on all CCs in FR2 have the same offset, and one of following conditions is met - If smtc2 is configured on any FR2 CC, - All CCs have the same configuration for smtc1, and - All CCs configured with smtc2 have the same configuration for smtc2 - If smtc2 is not configured on any FR2 CC, - The total number of different SMTC periodicities on all CCs does not exceed 4 Note: Longer delays for cell identification and measurement periods derived based on CSSFoutside_gap,i in clauses 9.2.5.1, 9.2.5.2, can be expected, if the UE is configured with more than 4 different SMTC periodicities on serving carriers. This applies for the intra-frequency measurement objects with the longest SMTC periodicity/periodicities.. Is this an acceptable version for everyone? [Nokia] Thanks Ato for providing this proposal. It seems we may be converging towards an acceptable text proposal. I might have missed it during the long discussions, but why is there the condition ‘only SCC in FR2’? Please find some additional updates from our side: The requirements in this clause apply for when up to 4 different SMTC periodicities are configured on serving carriers, provided that [- There are only SCCs in FR2, or] - The SMTC on all CCs in FR2 have the same offset, and one of following conditions is met - If smtc2 is configured on any FR2 CC, - All CCs have the same configuration for smtc1, and - All CCs configured with smtc2 have the same configuration for smtc2 - If smtc2 is not configured on any FR2 CC, - The total number of different SMTC periodicities on all CCs does not exceed 4 Note: Longer delays for cell identification and measurement periods derived based on CSSFoutside_gap,i in clauses 9.2.5.1, 9.2.5.2, can be expected, if the UE is configured with more than 4 different SMTC periodicities on FR2 serving carriers. The longer delay applies for the FR2 intra-frequency measurement objects with the longest SMTC periodicity/periodicities. [Ericsson] To Nokia, The “only SCC case” came from an earlier discussion paper from Ericsson (R4-2001406 observation 1/proposal 1); since there is assumed in requirements to be one baseband searcher for PCC and one baseband searcher shared between all the SCC, it means that if there are only SCC on FR2 then the UE is only required to measure one SCC at a time anyway, due to the baseband searcher limitation. So then we don’t need to worry about how the RX beamforming is done between different measurements since they are not concurrent. [Mediatek] We share the same view as Ericsson. Without PCC or PSCC in the band, UE only needs to handle one CC at a time for this band. The UE complexity is somehow reduced. [Nokia] thanks Ato for the input. I think it would be good to get this clear before agreeing to the CR. Anyway – I believe the basic frame is here and the text in [] below is now just needs a clarification. [Huawei] Thanks for the discussion and good that we are converging. We are fine with the latest version from Lars, but just one wording issue that we want to check with Lars – do we need to have both sentences in blue and in yellow? To us they are technically same, and both mean UE needs to meet the requirements when <=4 SMTC periodicities are configured. @Lars, is it ok to remove the sentence in blue? The requirements in this clause apply for when up to 4 different SMTC periodicities are configured on serving carriers, provided that [- There are only SCCs in FR2, or] - The SMTC on all CCs in FR2 have the same offset, and one of following conditions is met - If smtc2 is configured on any FR2 CC, - All CCs have the same configuration for smtc1, and - All CCs configured with smtc2 have the same configuration for smtc2 - If smtc2 is not configured on any FR2 CC, - The total number of different SMTC periodicities on all CCs does not exceed 4 [Nokia] Good to see progress. I was thinking about the two lines before sending out the proposal. But we somehow need to capture ‘serving carriers’. Maybe this can be done quite simply by adding it in the last bullet: The requirements in this clause apply for when up to 4 different SMTC periodicities are configured on serving carriers, provided that [- There are only SCCs in FR2, or] - The SMTC on all CCs in FR2 have the same offset, and one of following conditions is met - If smtc2 is configured on any FR2 CC, - All CCs have the same configuration for smtc1, and - All CCs configured with smtc2 have the same configuration for smtc2 - If smtc2 is not configured on any FR2 CC, - The total number of different SMTC periodicities on all serving CCs does not exceed 4 Would this ok? Then the blue line can be removed. One question again to the [] – do I understand the replies such that SCC does not include PSCell? [Huawei] Thanks for the suggestion on the wording and yes, it looks good for us. Regarding the question on SCC, my understand is that it does not include PSCell. With this clarification and also comments from Chris and Ato, would it be ok to remove the square bracket? [Nokia] Thanks for confirming my understanding. But does this then lead to that we do not support PSCell or PCell in FR2 – i.e. no UE requirements? [Mediatek] I don’t think this CR precludes supporting PSCell or PCell in FR2. J In case PSCell or PCell in FR2 is supported, the SMTC be somehow constrained based on this CR. In case PSCell or PCell in FR2 is supported, there is no limitation. [Huawei] Please find the updated version for the CR uploaded in the link below. The wording is changed based on the discussion below, and Ericsson is added as co-sourcing company. draft R4-200xxxx revised CR on FR2 SMTC alignment_Nokia_HW.docx Please let me know if you have further comments or if you would like to co-source. The formal Tdoc will be uploaded in around 30min. [Nokia] As such the CR is fine and we can co-source as well. Just note that the wording about only SCC is still maybe unclear. |
| R4-2005419 | Revised from R4-2003598. CR on TS38.133 for modification of the layer 3 and layer 1 measurement sharing factor when both SSB and RSSI symbol to be measured are considered Draft revised version was uploaded. No comment received on the revised version until April. 29. It can be endorsed. |
| R4-2004286 | CR on CSI-RS based BFD Draft revised version was uploaded. CR is agreeable. ---------------------------------------------------------------------------------------- [Huawei] - The new text can be misunderstood in term when which GPs apply. [Huawei: The following sentence is captured in the CR: “all gap patterns #0~11 in Table 8.1.2.1-1 can be configured for measurement of NR carrier only, and gap pattern#0, 1, 2, 3, 4, 6, 7, 8, 10 can be configured for measurement of E-UTRA carrier so with the applicability as specified in Table 8.1.2.1-1.” So we think when which GPs apply is clear]. - LTE SA requirements should not be in 38.133. [Huawei: this CR is for 36.133, so the LTE SA is here] Thus I think no need to revise the CR. Hope I correctly understood your comments. If not, please correct me. [Nokia] Sorry about the wrong specification. That would be my error. For the initial comment concerning: all gap patterns #0~11 in Table 8.1.2.1-1 can be configured for measurement of NR carrier only, and gap pattern#0, 1, 2, 3, 4, 6, 7, 8, 10 can be configured for measurement of E-UTRA carrier with the applicability as specified in Table 8.1.2.1-1.” The word ‘only’ is misleading and should be removed. Hence: all gap patterns #0~11 in Table 8.1.2.1-1 can be configured for measurement of NR carriers only, and gap pattern#0, 1, 2, 3, 4, 6, 7, 8, 10 can be configured for measurements of E-UTRA carriers with the applicability as specified in Table 8.1.2.1-1.” With such minor update the CR would be agreeable. [Huawei] Thanks for your comments. You mentioned that The word ‘only’ is misleading and should be removed. We think NR carrier only shall be remained (which discussed for a long time in R15 ). For example, GP#5 can only be used for NR only measurement. [Nokia] Thank you for the revised version. Concerning the word ‘only’ – when reading the text it is a bit confusing if the word only means that this line is when only NR carriers are configured to be measured or can also LTE carrier be configured. I understand that GP#5, GP#9 and #11 can only be configured for NR measurement according to Note 3. However, the other gaps can be used also for LTE measurements. In essence it is only GP#5, 9 and 11 which are for NR only. In E-UTRA-NR dual connectivity mode, NR - E-UTRAdual connectivity mode and E-UTRA standalone mode, all gap patterns #0~11 in Table 8.1.2.1-1 can be configured for measurements of NR carriers only, and gap pattern#0, 1, 2, 3, 4, 6, 7, 8, 10 can be configured for measurements of E-UTRA carriers with the applicability as specified in Table 8.1.2.1-1. Anyway – not a big issue. It should be clear from the table – it was only to get a more clear text. Otherwise Cr is fine. [Huawei] Technically our understanding is aligned. You mentioned in essence it is only GP#5, 9 and 11 which are for NR only. However if we just say GP#5, 9 and 11 are for NR only, then GP #4/6/7/8/10 are only for LTE+NR，and can not be applied for NR only. This is also misunderstanding. So the best way is keep the original sentence unchanged. Anyway as you said, the applicable table is clear. I’d like to confirm with you that the CR is fine to you, right? [Nokia] First of – the CR is agreeable. I also agree with you on what you say below. It is very difficult to find better wording and current wording is understandable with the help of the table. Hence, we can keep the ‘only’ wording. |
| R4-2005268 | Revised from R4-2004335 Draft revised version was uploaded. The revised version can be endorsed. -------------------------------------------------------- [Mediatek] We are fine with the CR. |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2005267 | Endorsed. Revised from R4-2004333. |
| R4-2005419 | Endorsed. Revised from R4-2003598. |
| R4-2004286 | Endorsed. |
| R4-2005268 | Endorsed |

# Topic #3: Connected state mobility

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2004281 | Huawei, HiSilicon | Discussion on RRC processing delay for RRC release with redirection Proposal 1: The expression for the RRC procedure delay in the core requirements shall remain unchanged for both R15 and R16 spec. Proposal 2: The value is remained unchanged in R16 test cases and the same changes shall be made to remove the reference to TS 38.331 in the test cases. |
| R4-2004842 | ZTE | Discussion on RRC procedure delay in RRC release with redirection Proposal 1: TRRC_procedure_delay = 20 ms. Proposal 2: Specify TRRC_procedure_delay = X ms in core requirements and test cases. Proposal 3: Specify TRRC_procedure_delay = 110 ms in clause 6.3.2.4 in 36.133 since there is no test case for redirection to NR. Endorse CR [4]. |
| R4-2004834 | ZTE | [CR] RRC release with redirection test cases 38.133 R15 Remove references to 38.331 as agreed in WF R4-2002206. |
| R4-2004833 | ZTE | [CR] RRC release with redirection 38.133 R16 |
| R4-2004835 | ZTE | [CR] RRC release with redirection 36.133 R15 Specify the value of TRRC_procedure_delay = 110 ms. |
| R4-2004836 | ZTE | [CR] RRC release with redirection 36.133 R16 Cat A |
| R4-2004854 | ZTE | Discussion on RRC re-establishment requirement - The UE is not aware of whether the network contains UE context before sending RRCReestablishmentRequest. Thus, the UE has to fulfill the delay requirement defined in clause 6.2.1.2.1 in TS 38.133 always. Having a line saying “There is no requirement if the target cell does not contain the UE context” in the specification gives impression to readers that under some cases, the UE is certain that the network doesn’t have UE context, which can be misleading. Proposal 1: The UE shall meet the delay requirement always since it can’t be sure whether the network has UE context or not. Proposal 2: Agree on the CRs [3][4] to remove the statement “There is no requirement if the target cell does not contain the UE context” in the specification. |
| R4-2004855 | ZTE | CR on RRC re-establishment requirements R15 Remove the statement “There is no requirement if the target cell does not contain the UE context”. |
| R4-2004856 | ZTE | CR on RRC re-establishment requirements R16 (Cat A) |
| R4-2003395 | Nokia | Correction of CFRA RSRP threshold Replacing cfra-csirs-DedicatedRACH-Threshold by rsrp-ThresholdCSI-RS |

## Open issues summary

### Sub-topic 3-1

Issue 3-1: RRC processing delay for RRC release with redirection

- Proposals for how to handle RRC processing delay in core requirement

  - Option 1 (Huawei R4-2004281):

    - The expression for the RRC procedure delay in the core requirements shall remain unchanged for both R15 and R16 spec.

    - The value is remained unchanged in R16 test cases and the same changes shall be made to remove the reference to TS 38.331 in the test cases.

  - Option 2 (ZTE R4-2004842, R4-2004834, R4-2004835):

    - TRRC_procedure_delay = 20  ms.

    - Specify TRRC_procedure_delay = 20 ms in core requirements and test cases.

    - Specify TRRC_procedure_delay = 110 ms in clause 6.3.2.4 in 36.133 since there is no test case for redirection to NR. Endorse CR [4].

- Recommended WF

  - Further discussion is needed.

### Sub-topic 3-2

Issue 3-2: removal of the statement about no requirement if UE context not contained for RRC re-establishment requirement

- Status of outcome of discussion in last meeting see in R4-2002290

  - Sub-Topic 5-3. CR (ZTE). Ericsson is neutral. According to 1st round, three companies supported, while two/three were negative. So maybe it can be postponed.

[2020-03-05]

Ericsson: Agree with CR.

Huawei: Suggest no change.

- Proposals (ZTE, R4-2004854/R4-2004855) (the proposal is almost the same as last meeting)

  - Proposal 1: The UE shall meet the delay requirement always since it can’t be sure whether the network has UE context or not.

  - Proposal 2: Agree on the CRs [3][4] to remove the statement “There is no requirement if the target cell does not contain the UE context” in the specification.

- Recommended WF

  - Quick check with companies who had the concern if the positions on this topic are changed.

### Sub-topic 3-3

For CR R4-2003395, please provide the comments directly in Section 3.3.2.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Sub topic 3-1: For R16, we have a preference that the Xms RRC processing delay is captured as a core requirement in 38.133. Since it offers better performance, we prefer ZTE option 2 (X=20ms) however if there is strong concern on the value we have not identified 110ms as a major issue in release 15, so a middle approach where 110ms is specified as a core requirement and test case is unchanged from R15 would be OK for us as a compromise Eg option 2bis - TRRC_procedure_delay = 110 ms. - Specify TRRC_procedure_delay = 100 ms in core requirements. Already in test cases. - Specify TRRC_procedure_delay = 110 ms in clause 6.3.2.4 in 36.133 since there is no test case for redirection to NR. Sub topic 3-2: As per our view at the end of last meeting we are neutral. Since the UE won’t know whether the target cell has the UE context, we can understand that the UE requirement should be the same regardless. However, we also understand that without UE context in the target cell, the whole procedure will fail, so there is no real need to define a UE requirement for a case that the procedure will not work anyway (even if the UE is blind to this) …. Others: |
| Mediatek | Issue 3-1: RRC processing delay for RRC release with redirection Option 1. We have the same view with Huawei. Since this isn’t a critical issue in R15, we suggest to keep current value. Issue 3-2: removal of the statement about no requirement if UE context not contained for RRC re-establishment requirement Support Huawei, no change to current spec. Since this isn’t a critical issue in R15, we suggest to close this issue in this meeting. As what we commented in last meeting, this requirement is for RRC re-establishment. If the network doesn’t send the RRCreestablishment message to UE, it means the overall procedure is not a RRC re-establishment, but a RRC setup. We suggest to keep these sentences to align with legacy LTE spec. |
| ZTE | Sub topic 3-1: As reasoned in our paper R4-2004842, we propose to specify TRRC_procedure_delay = 20 ms in core requirements and test cases. Having the requirement in both core requirements and test cases would mean a thorough correction. Related CRs: R4-2004834: simply captures changes agreed in WF R4-2002206, should be endorsed. R4-2004835: interested companies can check this CR, here we spotted that TRRC_procedure_delay is unspecified in redirection to NR and we suggest to specify the value of TRRC_procedure_delay = 110 ms. Should be agreeable in our view. As companies views are a bit diversed, we can lead another separate WF discussion like last meeting and try to reach some compromise. If companies think TRRC_procedure_delay = 20 ms is a bit too strict we’re willing to discuss a more proper value. Sub topic 3-2: The discussion started last meeting and was postponed. For companies which don’t support to remove the statement of “There is no requirement if the target cell does not contain the UE context”, please explain under which condition the UE would know the target cell don’t contain UE context. Our intention is not to tighten requirements for UE, it’s just we believe the UE wouldn’t know if UE context is released or not anyways so the UE has to meet the requirements anyhow. The problem with this statement is that it suggests the UE would know, at least under some cases, whether the UE context is present at network, which is not true. We have concerns that this statement would give misleading impressions to readers and cause problems. |
| Apple | Sub topic 3-1: Option 1 is preferred |
| Huawei, HiSilicon | Sub topic 3-1: We support option 1. The issue results from the wrong reference to TS 38.331. We are fine to remove the reference in the related test cases. It can be observed that the value the RRC processing delay for Release with redirection in the test (110ms) is reused from LTE. We think it is not critical and necessary to enhance the performance of the delay for RRC processing at current stage for R16. We suggest to keep the core parts unchanged and remove the reference in the test cases for both R15 and R16. Enhancement of the processing delay could be considered in the further releases. |
| Intel | Issue 3-1: RRC processing delay for RRC release with redirection It is not encouraged to further update R15 core requirement unless it is critical. We support option 1. |
| Qualcomm | Sub-topic 3-1: we support option 1. |

### CRs/TPs comments collection

CRs included in the above sub-topics are not listed here.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2003395 | NEC: OK |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #3-1 | 7 companies made comments. 5 companies supported Option 1. 2 company supported to capture the RRC processing delay (Option 2). 1 company suggested the modification on Option 2. Tentative agreements: NA Candidate options: - Option 1 - Option 2 - Option 2bis - TRRC_procedure_delay = 110 ms. - Specify TRRC_procedure_delay = 100 ms in core requirements. Already in test cases. - Specify TRRC_procedure_delay = 110 ms in clause 6.3.2.4 in 36.133 since there is no test case for redirection to NR. Recommendations for 2nd round: Further discuss the options above. If there was an agreement, it is suggested to use R4-2004834 and/or R4-2004835 to capture the agreement. |
| Sub-topic #3-2 | 4 companies made comments. 2 companies proposed no change. 1 company is neutral. 1 company proposed the change. Tentative agreements: N/A Candidate options: Recommendations for 2nd round: Further discussion. Proponent needs convince companies. If companies still had strong view, it is suggested not to come back in the future meeting. Then the status could be “rejected”. In moderator’s understanding, there is no draft Cat A CR which needs be treated in this meeting. So both draft Cat A CRs are suggested to withdraw. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2004834 | Return to ZTE：R4-2004834: simply captures changes agreed in WF R4-2002206, should be endorsed. |
| R4-2004833 | Withdrawn Cat A CR to R4-2004834 |
| R4-2004835 | Return to |
| R4-2004836 | Withdrawn Cat A CR to R4-2004835 |
| R4-2004855 | Return to |
| R4-2004856 | Withdrawn Cat A CR to R4-2004855 |
| R4-2003395 | Endorsed |

## Discussion on 2nd round (if applicable)

Moderator tries to summarize discussions for the 2nd round.

|  | Status summary |
| --- | --- |
| R4-2004834 | [CR] RRC release with redirection test cases 38.133 R15 No further discussion is organized. Tdoc is suggested to be noted. |
| R4-2004833 | [CR] RRC release with redirection 38.133 R16 In the first round it was recommended to be withdrawn. But the proponent thought it is not a Cat A CR and should be agreeable. No further discussion is organized. Tdoc is suggested to be noted. |
| R4-2004835 | [CR] RRC release with redirection 36.133 R15 No discussion is organized. Tdoc is suggested to be noted. |
| R4-2004855 | CR on RRC re-establishment requirements R15 No discussion is organized. Tdoc is suggested to be noted. |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2004834 | Not pursued. |
| R4-2004833 | Not pursued. |
| R4-2004835 | Not pursued. |
| R4-2004855 | Not pursued. |

# Topic #4: Timing

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2004284 | Huawei, HiSilicon | CR on UE transmit timing Remove the one shot timing in the introduction part |

## Open issues summary

### CRs/TPs comments collection

CRs included in the above sub-topics are not listed here.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2004284 | Ericsson : OK |
|  | MTK: ok |
|  | ZTE: OK |
|  | Nokia: OK |
|  | NEC: OK |

## Summary for 1st round

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2004284 | Endorsed |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |

# Topic #5: Signaling characteristics

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2003494 | Mediatek | Discussion on remaining issues of TCI state switch In the contribution, we discuss the remaining issues for TCI state switch. We have the following proposals: Proposal 1: Delete the description of uplink transmission behaviour in TCI state switching requirement. Proposal 2: To keep consistency to scheduling restriction requirement in Section 8.5.7.2 and Section 9.2.5.3.2, scheduling restriction shall be also applied to CSI-RS for CQI in TCI state switching. |
| R4-2003493 | Mediatek | draftCR on TCI state switch - Change 3ms to 3Nslotsubframe,µ - Add Repition ON for CSI-RS used if the target TCI state is unknown - If the target TCI state is known, upon receiving PDSCH carrying MAC-CE activation command at slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ +(3 ms +TOk*(Tfirst-SSB + TSSB-proc))/NR slot length. (If TOk =0, then it would be n+THARQ +3ms) - Delete PUSCH wording in DCI based TCI state switch requirment. - Add CSI-RS for CQI in scheduling restrcition in RRC based TCI state switch requirement. Delete the PUCCH/PUSCH transmission in scheduling restrcition in RRC based TCI state switch requirement. |
| R4-2003574 | Qualcomm | Corrections to R15 MAC-CE based TCI state switching requirements Observation 1. Guaranteeing PDCCH performance on the new TCI state is independent of receive on the old TCI state. Observation 1. In case target TCI state is not in the active TCI state list for PDSCH, there will be at most ~2ms (TSSB-proc) when PDCCH performance cannot be guaranteed if NW schedules MAC-CE based TCI state switch exactly 3ms before an SSB. Proposal 1. Modify MAC-CE based TCI state switch delay requirements as: “The UE shall be able to receive on the old TCI state until slot n+ THARQ +3 ms +TOk*(Tfirst-SSB).” |
| R4-2003575 | Qualcomm | CR for correction to MAC-CE based TCI State switch timeline (Clause 8.10.3) UE to switch to new beam 3ms after HARQ. Demodulation performance guranteed after reception of SSB |
| R4-2004336 | Huawei, HiSilicon | Discussion on SCell activation requirements In this paper we provided our views on the SCell activation requirements. Proposal 1: Update the cell detection time for FR2 unknown SCell activation as TFirstSSB_MAX + 15* TSMTC_MAX + 8*Trs Proposal 2: The start point of the interruption shall not occur before n+1+THARQNR slot length, and shall not occur after n+1+THARQ+3+TSSB_usedNR slot length, where TSSB_used is - TFirstSSB for FR1 known SCell with measurement cycle 160ms, and FR2 SCell when there is active serving cell in the same band - TFirstSSB_MAX for FR1 known SCell with measurement cycle >160ms, FR1 unknown SCell, FR2 unknown SCell when there is no active serving cell in the same band - TFineTiming for FR2 known SCell when there is no active serving cell in the same band Proposal 3: The end point of the interruption shall not occur after n+1+THARQ+3+TSSB_usedNR slot length + Ninterruption, where is the interruption length defined in section 8.2. Proposal 4: For FR2 SCell activation where there is active serving cell in the band, the requirements apply provided that the SSB pattern is same for the active serving cell and the SCell being activated. Proposal 5: Definition of TFirstSSB and TFirstSSB_MAX should be updated such that it includes a complete SSB burst. Proposal 6: UE is not expected to meet the requirements for RRM measurement and in case of FR2 the requirements for SSB based RLM/BFD/CBD/L1-RSRP measurement during the cell detection time for unknown SCell activation. |
| R4-2004337 | Huawei, HiSilicon | CR on SCell activation requirements R15 - Add condition for FR2 SCell activation where there is active serving cell in the band that the SSB pattern is same for the active serving cell and the SCell being activated. - Update the definition of TFirstSSB and TFirstSSB_MAX such that it includes the complete SSB burst. - Add a statement that UE is not expected to meet the requirements for RRM measurement and in case of FR2 L1 measurement requirements during cell detection time for unknown SCell activation. |
| R4-2004418 | Ericsson | On Correction to SCell Activation Delay Requirements In this contribution we have addressed issues regarding SCell activation delay timelines and definitions of interruption windows. In order to align the activation timeline for activation of first unknown SCell in FR2 to other activation cases, the following mutually exclusive proposals are made, with the intention of having companies statting whether it is preferred to use TFirstSSB or TFirstSSB_MAX in the corrected timeline: Proposal 1a: The activation delay requirements for cases of first unknown SCell in FR2 are to be modified by replacing 24*Trs with TFirstSSB + 23*Trs. Proposal 1b: The activation delay requirements for cases of first unknown SCell in FR2 are to be modified by replacing 24*Trs with TFirstSSB_MAX + 23*Trs. We further propose a more compact description for activation of known SCell in FR2 with semi-persistent CSI-RS, as this simplifies the definition of the interruption window: Proposal 2: The activation delay requirements for cases of known SCell in FR2 with semi-persistent CSI-RS are to be modified into a single requirement: Tactivation_time = Tuncertainty_MAC +TFineTiming + 5ms, where Tuncertainty_MAC = 0 if UE receives SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time. Finally, we are proposing a modified description of the interruption window (see above for details): Proposal 3: The interruption window is to be modified such that the end-point specifies the starting point of an interruption, where this point depends on whichever of TFirstSSB, TFirstSSB_MAX, TFineTiming, etc, is applicable for the SCell activation delay requirement. The interruption window requirement is to make references to clause 8.2 regarding the length of the interruption, which depends on whether the interrupted cell is intra- or inter-band. A draft CR, comprising an updated version of the CR that was postponed at RAN4#94e, is provided in [4]. |
| R4-2004419 | Ericsson | Draft CR 38.133 (8.3.2) Corrections to SCell Activation Delay Requirements Introducing the following corrections: - Modifying activation timelines for activation of first unknown SCell in FR2 by replacing “24*Trs” by “TFirstSSB_MAX + 23*Trs” - Modifying activation timeline for activation of first known SCell in FR2 when SP CSI-RS is used, by merging the two cases into a single one and where the value of Tuncertainty_MAC is specified to 0 for one of the cases - Modifying the expression for the interruption window to now define the starting point of the interruption, which is related to whichever of TFirstSSB, TFirstSSB_MAX, TFineTiming, etc, that applies for the activation delay requirement. Referring to clause 8.2 for the length of the interruption, which depends on whether the impacted cell is intra- or inter-band to the SCell being activated. |
| R4-2003610 | Mediatek | CR for SCell activation delay in FR2 Currently, when semi-persistant CSI-RS is configured for CSI report, Tuncertainty_MAC will include the waiting time of MAC-CE for SP CSI-RS. However, performing fine timing tracking (TFineTiming ) is not necessarily to wait for the SP CSI-RS. Remove the waiting time of MAC-CE for SP CSI-RS from Tuncertainty_MAC, and make fine timing tracking not depeding on the SP CSI-RS. |
| R4-2004287 | Huawei, HiSilicon | (CR) Correction onTCI state switching The content related with one-shot timing adjustment is removed accordingly |
| R4-2004417 | Ericsson | Draft CR 38.133 (8.10.5) Corrections to RRC-based TCI state change Introducing the following correction: - Changing “slot n+ TRRC_processing +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length” to “slot n+ (TRRC_processing +TOk*(Tfirst-SSB + TSSB-proc)) / NR slot length” - Changing “slot n+ TRRC_processing +TL1-RSRP +TOuk*(Tfirst-SSB + TSSB-proc) / NR slot length” to “slot n+ (TRRC_processing +TL1-RSRP +TOuk*(Tfirst-SSB + TSSB-proc)) / NR slot length” Correcting some punctuation. |
| R4-2004238 | Huawei, Hisilicon, MediaTek | Correction on Psharingfactor Following changes are made: - For SSB based RLM/BFD/CBD/L1-RSRP, the first condition of Psharingfactor=1 is removed; - For SSB based RLM/BFD/CBD/L1-RSRP, the second condition of Psharingfactor=1 is changed. RSSI is considered. - Same rules apply to CSI-RS based RLM/BFD/CBD/L1-RSRP. - Editorial corrections. |
| R4-2004277 | Huawei, HiSilicon | CR on interruption due to Acitve BWP switch Remove the note in table Table 8.2.1.2.7-1 and Table 8.2.2.2.5-1. |

## Open issues summary

### Sub-topic 5-1

Issue 5-1: Correction about uplink transmission for TCI switching

- Proposals (Mediatek R4-2003494/R4-2003493)

  - Delete the description of uplink transmission behaviour in TCI state switching requirement.

- Recommended WF

  - TBA

### Sub-topic 5-2

Issue 5-2: Correction on restriction of CSI-RS reception for TCI switching

- Proposals (Mediatek R4-2003494/R4-2003493)

  - To keep consistency to scheduling restriction requirement in Section 8.5.7.2 and Section 9.2.5.3.2, scheduling restriction shall be also applied to CSI-RS for CQI in TCI state switching.

- Recommended WF

  - TBA

### Sub-topic 5-3

Issue 5-3: Correction on timeline of reception on old TCI for TCI switching

- Proposals (Qualcomm R4-2003574/ R4-2003575)

  - Modify MAC-CE based TCI state switch delay requirements as: “The UE shall be able to receive on the old TCI state until slot n+ THARQ +3 ms +TOk*(Tfirst-SSB).”

- Recommended WF

  - TBA

### Sub-topic 5-4

Issue 5-4: Correction of activation timeline in FR2

- Proposals  :

  - Option 1 (Ericsson R4-2004418/R4-2004419)

    - The activation delay requirements for first unknown SCell in FR2

      - Alternative 1: The activation delay requirements for cases of first unknown SCell in FR2 are to be modified by replacing 24*Trs with TFirstSSB + 23*Trs.

      - Alternative 2: The activation delay requirements for cases of first unknown SCell in FR2 are to be modified by replacing 24*Trs with TFirstSSB_MAX + 23*Trs.

    - The activation delay requirements for cases of known SCell in FR2 with semi-persistent CSI-RS are to be modified into a single requirement: Tactivation_time = Tuncertainty_MAC +TFineTiming + 5ms, where Tuncertainty_MAC = 0 if UE receives SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time.

  - Option 2 (Huawei R4-2004336)

    - Update the cell detection time for FR2 unknown SCell activation as TFirstSSB_MAX + 15* TSMTC_MAX + 8*Trs

- Recommended WF

  - Further discussion is needed

### Sub-topic 5-5

Issue 5-5: Interruption window definition for SCell activation

- Proposals

  - Option 1 (Huawei R4-2004336/R4-2004337) :

    - The start point of the interruption shall not occur before n+1+THARQNR slot length, and shall not occur after n+1+THARQ+3+TSSB_usedNR slot length, where TSSB_used is

      - TFirstSSB for FR1 known SCell with measurement cycle 160ms, and FR2 SCell when there is active serving cell in the same band

      - TFirstSSB_MAX for FR1 known SCell with measurement cycle >160ms, FR1 unknown SCell, FR2 unknown SCell when there is no active serving cell in the same band

      - TFineTiming for FR2 known SCell when there is no active serving cell in the same band

    - The end point of the interruption shall not occur after n+1+THARQ+3+TSSB_usedNR slot length + Ninterruption, where is the interruption length defined in section 8.2.

  - Option 2: (Ericsson R4-2004418/R4-2004419)

    - The interruption window is to be modified such that the end-point specifies the starting point of an interruption, where this point depends on whichever of TFirstSSB, TFirstSSB_MAX, TFineTiming, etc, is applicable for the SCell activation delay requirement. The interruption window requirement is to make references to clause 8.2 regarding the length of the interruption, which depends on whether the interrupted cell is intra- or inter-band.

- Recommended WF

  - TBA

### Sub-topic 5-6

Issue 5-6: SSB pattern for FR2 intra-band CA case for SCell activation

- Proposals (Huawei R4-2004336/R4-2004337): For FR2 SCell activation where there is active serving cell in the band, the requirements apply provided that the SSB pattern is same for the active serving cell and the SCell being activated.

- Recommended WF

  - TBA

### Sub-topic 5-7

Issue 5-7: Definition of TFirstSSB and TFirstSSB_MAX

- Proposals (Huawei R4-2004336/R4-2004337): Definition of TFirstSSB and TFirstSSB_MAX should be updated such that it includes a complete SSB burst.

- Recommended WF

  - TBA

### Sub-topic 5-8

Issue 5-8: Restriction of RRM measurements for unknown SCell activation

- Proposals (Huawei R4-2004336/R4-2004337): UE is not expected to meet the requirements for RRM measurement and in case of FR2 the requirements for SSB based RLM/BFD/CBD/L1-RSRP measurement during the cell detection time for unknown SCell activation.

- Recommended WF

  - TBA

### Sub-topic 5-9

For CR R4-2003610, R4-2004287, R4-2004238, R4-2004277 and R4-2004417, please provide the comments directly in Section 5.3.2.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Sub topic 5-1: Proposal seems OK Sub topic 5-2: Proposal seems OK Sub topic 5-3: This creates a gap during which the UE cannot be scheduled. This may put strict requirements on when the network applies the TCI state switch, in order to avoid loss of achievable end-user throughput. What is the assumed UE capability with respect to number of tracked TCI states? Is there any chance for the network to avoid such gap in the scheduling, other than getting tight constraints on when a TCI change command can be sent to the UE? If not, we would prefer to keep existing UE behaviour i.e. for switching to non-tracked TCI state stay on old state until SSB for target TCI state has been received and processed. Subtopic 5-4: We can further discuss Option 2. Agree that there is a justification for replacing 15 Trs by 15 TSMTC_max, but it is a bit pessimistic to assume that all sweeps would start with zero knowledge of the general received power level. Second bullet in Option 1, activation delay requirements for cases of known SCell in FR2 with semi-persistent CSI-RS, is not in conflict with Option 2, and hence can be agreed regardless of whether we go for Option 1 or Option 2 for activation of first unknown SCell in a FR2 band. Subtopic 5-5: Option 1: The proposed change does not solve the interruption window issue for all cases (e.g. for the case where periodic CSI-RS are used). Our proposal (Option 2) is as follows: The starting point of an interruption as specified in clause 8.2 on PCell or any activated SCell in MCG for NR standalone mode, or on PSCell or any activated SCell in SCG for EN-DC mode, shall not occur before slot n+1+THARQNR slot length and not occur after slot n+1+ THARQ+3+TXNR slot length, where TX is: - TFirstSSB, for any scenario where Tactivation_time includes TFirstSSB; - TFirstSSB_MAX, for any scenario where Tactivation_time includes TFirstSSB_MAX; - Tuncertainty_MAC +TFineTiming, for any other scenario where Tactivation_time includes TFineTiming and semi-persistent CSI-RS are configured; - max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ), for any other scenario where Tactivation_time includes TFineTiming and periodic CSI-RS are configured. The maximum duration of the interruption is specified in clauses 8.2.1.2.4, 8.2.2.2.2, 8.2.3.2.4, and 8.2.4.2.2 for EN-DC, SA, NE-DC, and NR-DC, respectively, where further the applicable value depends on whether the interrupted serving cell is in the same band (intra-band) as, or in a different band (inter-band) to, the SCell being activated. We do not see any significant difference between the proposals except that that our proposal is aiming at covering also the case with periodic CSI-RS, and that we are describing the end-point differently (endpoint versus duration of interruption). Both options are based on the same principle and we can work together on the wording. Subtopic 5-6: In principle the patterns do not have to be the same as long as the UE is provided a TCI state for the SCell being activated. It will then know how to relate timing on one carrier to the start of the target SSB on another carrier. Subtopic 5-7: According to our understanding e.g. TFirstSSB is the time to reception of the relevant SSB index. …. Others: |
| MTK | Issue 5-1: Correction about uplink transmission for TCI switching The intention of this proposal is that for UL transmissions, we should address them in the requirement of spatial relation switch rather than TCI-state switch. Further comments to Huawei: For PUSCH part, we delete the sentence because we should not to say ‘transmit the PUSCH with target TCI’. DL TCI state switch is irrelevant to UL spatial relation. Our intention here is not prohibited UE to transmit PUSCH, but prohibited UE to transmit PUSCH with target TCI state. For RRC part, the reason is the same. How to transmit the PUSCH is based on spatial info. If this spatial RS isn’t the DL RS in TCI switch, of course it’s fine to UE. If this spatial RS is the DL RS in TCI switch. The RRC reconfiguration only refresh the TCI state, UE still can transmit the UL based on the spatial relation info. with the DL RS Issue 5-2: Correction on restriction of CSI-RS reception for TCI switching The intention of this proposal is to keep consistency to scheduling restriction requirement with BM and Measurement. Issue 5-7: Definition of TFirstSSB and TFirstSSB_MAX |
| Apple | Sub topic 5-1: Agree with MediaTek proposal Sub topic 5-2: Agree with MediaTek proposal Sub topic 5-3: We have different understanding on this issue, if we agree on this change, then there is a time gap of TOk*(Tfirst-SSB), and within this gap it’s not clear which TCI could be used reliably for PDCCH to the UE, this time gap depends on the SSB periodicity which might be up to 160ms and the impact will be big. Our view is using old TCI in this time gap will be safer than use the new one without any timing/frequency information. We agree that there is a mismatch between Ran1 and Ran4 spec, and as we commented in last meeting, it’s better to send LS to RAN1 to ask them to refer TS38.214 to RAN4 delay requirement, and it’s same as what they did for SCell activation delay. Sub-topic 5-4: we support Option 1. We concerned on how likely network will make the SSBs on intra-band FR2 CCs not aligned with each other on time domain, and therefore to use TfirstSSB or use TfirstSSB_MAX may not have difference in the field. Sub-topic 5-5: we support option 1. Sub-topic 5-6: Huawei proposal is the most conservative assumption, in fact as long as at least one detectable SSB of to-be-activated SCell is colliding with the SSB of existing serving FR2 CC on time domain, the timing information from active serving cell in that band is still usable, which means the pattern does not have to be same in theory. But if all the companies are fine to have such assumption of same SSB patterns, we are also fine. Sub-topic 5-8: agree with Huawei |
| Huawei, HiSilicon | Issue 5-1: Correction about uplink transmission for TCI switching We have some concern on this proposal. Removing PUSCH part in DCI based TCI switch implies that UE is not required to transmit PUSCH after the TCI switch delay, which we understand is not the intention. Removing the PUSCH part in the scheduling restriction means UE is still able to transmit during RRC based TCI switch. The argument in the paper is that UE UL spatial info may not be impacted by TCI switch – we agree to this, but to conduct UL Tx, UE needs not only the spatial info but also the DL timing reference, and we understand the latter is not available during RRC based TCI switch. Issue 5-2: Correction on restriction of CSI-RS reception for TCI switching We are fine with the proposal. Issue 5-3: Correction on timeline of reception on old TCI for TCI switching We prefer to stick to the agreement from last meeting, i.e. the old TCI is used in TOk*(Tfirst-SSB). Issue 5-4: Correction of activation timeline in FR2 We support option 2 to capture the assumption of AGC for unknown SCell activation and get aligned with FR1 requirements. To Ericsson, for minimum requirements we are assuming the worst case and in our view it may be hard to agree on another assumption at this stage for Rel-15. Second bullet in Option 1 is another issue and we are fine with it. To Apple, as discussed in Issue 2-1 there can be at least 2 different SMTC periodicities for FR2 CC, so in this case option 2 is more precise. [Apple comment to Huawei reply]: yes, it depends on the conclusion of Issue 2-1. Issue 5-5: Interruption window definition for SCell activation We support option 1. To Ericsson, we agree that option 2 is more comprehensive covering more cases. However, the key difference between the options is that option 1 applies for the case when the RF re-tuning and AGC are non-consecutive in time, and in this case the “end point” is not identical to the “start point + duration”. Issue 5-6: SSB pattern for FR2 intra-band CA case for SCell activation To Ericsson and Apple, the current requirement is If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, then Tactivation_time is TFirstSSB+ 5ms UE is supposed to do fine time tracking based on the first SSB in to-be-activated SCell, but the question is which SSB in the burst UE should use. It cannot be any SSB but should be the one that corresponds to the Tx beam for scheduling the UE. We agree with Ericsson that if the TCI is provided UE would know which SSB to use, but in the current requirement there is no TCI provision, and UE is supposed to know which SSB to use from the active serving cell. [Apple comment to Huawei reply]: even though the TCI is provided and UE know which SSB to use for timing T/F tracking on to-be-activated SCell (e.g. TCI is QCLed to the SSB#0 on to-be-activated Scell), UE has no idea about the suitable Rx beam to receive this SSB#0 of target being-activated SCell in case the SSB#0 on the same time position is not transmitted on active serving CC. Issue 5-7: Definition of TFirstSSB and TFirstSSB_MAX To Ericsson, we will further check on the definition for TFirstSSB. On the other hand, we think TFirstSSB_MAX should be clear and should be updated such that it includes a complete SSB burst. Issue 5-8: Restriction of RRM measurements for unknown SCell activation We support the proposal. |
| Intel | Issue 5-1: Correction about uplink transmission for TCI switching Proposals from MTK make sense to us. We support the proposals. Issue 5-2: Correction on restriction of CSI-RS reception for TCI switching Proposals from MTK make sense to us. We support the proposals. Issue 5-3: Correction on timeline of reception on old TCI for TCI switching We were wondering if we can live with the mismatch. Removing TOk*(Tfirst-SSB) would create a time gap whereas UE is allowed not to have any data reception, which is not a preferred solution. However, although sending LS to ask RAN1 to update their spec may solve the problem, it is still not a highly preferred solution. we should be very careful if the solution involves change in release 15 specification in other working groups. Issue 5-8: Restriction of RRM measurements for unknown SCell activation Agree with Huawei’s proposal, given that during cell detection time for unknown target SCell activation procedure, no scaling factor is considered, which means UE should prioritize SCell activation procedure. |
| Qualcomm | Sub-topic 5-2: there is still a discrepancy between RAN1 and RAN4 requirements which is not addressed here. Also, we need more clarification why PUCCH/PUSCH is deleted from scheduling restriction Sub-topic 5-3: we do not agree with Ericsson or Apple that the time gap can be large. NW has complete control on when MAC-CE based TCI state switch is sent to UE so it can align it to be precisely 3ms before an SSB. In such case and when TOk = 1, there will be ~2ms (TSSB-proc) when PDCCH performance cannot be guaranteed which is negligible impact in our view. In our view, this is the best compromise in order to adhere to RAN1 requirements while maintaining RAN4 agreements. In other words, the switch starts at slot n + THARQ +3 ms. We don’t think ~2ms of scheduling restriction can have a big impact. Sub-topic 5-5: we support option 1. |
| Nokia, Nokia Shanghai Bell | 5-1: Issue 5-1: Correction about uplink transmission for TCI switching Mediatek R4-2003494/R4-2003493 Proposal 1 is ok. Proposal 2: TRS and CQI are both based on CSI-RS so can we just ay CSI-RS without being specific about the purpose of the CSI-RS? - section 8.10.6 correction is fine, but it was agreed in last meeting already? - ‘no later than’ change is not needed. - correction to 8.10.3 is not correct according to our understanding. 38.214 states 3ms - We don’t fully understand change 2. Needs more discussion. - As for adding CSI-RS for CQI: TRS and CQI are both based on CSI-RS so can we just ay CSI-RS without being specific about the purpose of the CSI-RS? 5-2: Issue 5-2: Correction on restriction of CSI-RS reception for TCI switching Proposals (Mediatek R4-2003494/R4-2003493) Proposal 1 is ok. Proposal 2: TRS and CQI are both based on CSI-RS so can we just ay CSI-RS without being specific about the purpose of the CSI-RS? - section 8.10.6 correction is fine, but it was agreed in last meeting already? - ‘no later than’ change is not needed. - correction to 8.10.3 is not correct according to our understanding. 38.214 states 3ms - We don’t fully understand change 2. Needs more discussion. - As for adding CSI-RS for CQI: TRS and CQI are both based on CSI-RS so can we just ay CSI-RS without being specific about the purpose of the CSI-RS? 5-3: Issue 5-3: Correction on timeline of reception on old TCI for TCI switching Proposals (Qualcomm R4-2003574/ R4-2003575) Can Qualcomm clarify if the UE is then not able to receive on the old TCI state (or new TCI state) for TOk*(Tfirst-SSB)? One reason for this additional TOk*(Tfirst-SSB) was that the UE would be able to receive on old TCI state until it receives SSB on the new TCI state. With this change there is always a gap in the PDCCH reception no matter if the target TCI state is known and in the active TCI state list. This needs more discussion (including the CR) 5-4: Issue 5-4: Correction of activation timeline in FR2 We support option 1(Ericsson R4-2004418/R4-2004419). 5-5: Issue 5-5: Interruption window definition for SCell activation Option 1 (Huawei R4-2004336/R4-2004337): the start point of interruption could be generally fine, Better naming for TSSB_used could be considered. Is it needed to define the end point of interruption? 5-6: Issue 5-6: SSB pattern for FR2 intra-band CA case for SCell activation (Huawei R4-2004336/R4-2004337): We do not see a need for such restriction. 5-7: Issue 5-7: Definition of TFirstSSB and TFirstSSB_MAX (Huawei R4-2004336/R4-2004337) Our understanding is that the 5ms in the delay is considering the up to 5ms burst length already. Do we now count it twice? 5-8: Issue 5-8: Restriction of RRM measurements for unknown SCell activation (Huawei R4-2004336/R4-2004337) Why is this needed? UE is already allowed time for detection during the delay. 5-9: CR R4-2003610, R4-2004287, R4-2004238, R4-2004277 and R4-2004417, please provide the comments directly in Section 5.3.2 |
| NEC | Issue 5-4: We support option 2. Issue 5-5: We support option 1. |

### CRs/TPs comments collection

CRs included in the above sub topics are not listed here.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2003610 | Ericsson: OK |
|  | Mediatek: This CR is proposed to reflect the agreement in RAN4#93 Agreement Fine time tracking will not depend on the SP CSI-RS activation used for CSI reporting Further comments to Apple: This CR is proposed to decouple the dependency between TCI of SP CSI-RS for CSI reporting and SSB for fine tine tracking, and it still keeps the dependency between PDCCH TCI and the SSB for fine time tracking, as the assumption you mentioned. It can be observed that the Tuncertainty_MAC includes the waiting time for PDCCH TCI, and the “max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP)” implies the SSB for fine time tracking will wait for the MAC-CE of PDCCH TCI. Hope this clarifies your concern. [Apple comment to MTK reply]: yes, thanks, that clarified my question. |
|  | Apple: small question: do we need to assume that the SSB of fine timing tracking shall be the SSB corresponding to the TCI state used for PDCCH? If this assumption is true, then UE needs to wait for MAC commend of PDCCH TCI activation before UE can know which SSB can be used for fine timing tracking. Even though we are also fine with it in RAN4 #93, but after that meeting we found this unclear point, we appreciate if companies can clarify it. |
|  | Nokia: Change is fine if needed and reason is clear. But current reason for change is not clarifying the need for this change. |
| R4-2004287 | Ericsson: OK |
|  | Mediatek: ok |
|  | Nokia: OK |
|  | NEC: OK |
| R4-2004238 | Ericsson:OK |
|  | Nokia: Not ok. Some change in wording (is-are). First line not needed. RSSI is not clearly defined in all conditions where this is measured. |
| R4-2004277 | Ericsson:OK |
|  | Mediatek: The note is correct in switch delay requirement, but should not be duplicated here for interruption requirement. In Rel-15, we do not assume the victim CC will change its SCS when the aggressor CC is changing its BWP. Further comments to Huawei: Sorry for the confusion. We are OK to this CR. |
|  | Huawei, HiSilicon: not sure if we get MTK comment correctly – we understand the comment is aligned with the proposed change in the CR, i.e. the note should be deleted in the interruption requirements. But please let us know if we get the comment incorrectly. |
|  | Nokia: OK |
|  | NEC: OK |
| R4-2004417 | Mediatek: ok |
|  | Nokia: OK |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic# 5-1 | 6 companies made the comments. 5 companies are OK with the proposal. 1 company had concern. The proponent provided the response. Need check if the response is OK to the company who had the concern. Tentative agreements: N/A Candidate options: Recommendations for 2nd round: Check if the response is OK to the company who has the concern. |
| Sub-topic# 5-2 | 7 companies made the comments. 5 companies are OK with the proposal. 2 company made the comments, and proposed some additional changes. 1 company had some questions. Tentative agreements: N/A Candidate options: Recommendations for 2nd round: Further discussion is needed to address companies’ comments and answer the questions. |
| Sub-topic# 5-3 | 6 companies provided the comment. 4 companies expressed the concern that with the change there will be a gap where the TCI state is unclear, and although the proponent made response more efforts are needed to convince companies. Tentative agreements: Candidate options: Recommendations for 2nd round: Further discussion is needed. |
| Sub-topic# 5-4 | 5 companies made the comments. 3 companies supported Option 1 while 2 companies supported Option 2. Tentative agreements: The following bullet seems agreeable to the group: - The activation delay requirements for cases of known SCell in FR2 with semi-persistent CSI-RS are to be modified into a single requirement: Tactivation_time = Tuncertainty_MAC +TFineTiming + 5ms, where Tuncertainty_MAC = 0 if UE receives SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time. Candidate options: - Option 1 (Ericsson, Apple, Nokia) - The activation delay requirements for first unknown SCell in FR2 - Alternative 1: The activation delay requirements for cases of first unknown SCell in FR2 are to be modified by replacing 24*Trs with TFirstSSB + 23*Trs. - Alternative 2: The activation delay requirements for cases of first unknown SCell in FR2 are to be modified by replacing 24*Trs with TFirstSSB_MAX + 23*Trs. - Option 2: (Huawei, NEC) - Update the cell detection time for FR2 unknown SCell activation as TFirstSSB_MAX + 15* TSMTC_MAX + 8*Trs Recommendations for 2nd round: More discussion is needed. |
| Sub-topic# 5-5 | 6 companies made comments. 4 companies supported Option 1. 1 company had question and 1 company thought that both options are on the same principle. Tentative agreements: The principle of both Option 1 and Option 2 is acceptable. Candidate options: Recommendations for 2nd round: Companies can work together to provide the way forward and based on the way forward to work on the CR R4-2004419 and R4-2004337. |
| Sub-topic# 5-6 | 4 companies made the comments. 2 companies were negative and 1 company is neutral. Tentative agreements: N/A Candidate options: Recommendations for 2nd round: The proponent needs convince the companies. More discussion is needed. |
| Sub-topic# 5-7 | 3 companies made comment. Companies are not convinced yet. Tentative agreements: N/A Candidate options: Recommendations for 2nd round: Further check the definition of TFirstSSB as commented by Ericsson. More discussion is needed for definition of TFirstSSB_MAX. Further discussion is needed. |
| Sub-topic# 5-8 | 4 companies made the comment. 3 companies supported the proposal. 1 company questioned the proposal. Tentative agreements: N/A Candidate options: Recommendations for 2nd round: More discussion is needed. The response to Nokia is needed. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on SCell activation requirements (Suggest to have a WF to organize the discussion for Sub-topic 5-4 ~ 5-8. Based on the WF the CRs can be prepared) | Huawei, HiSilicon |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2003493 | Return to Check if the response for sub-topic 5-1 is OK to Huawei, and need capture comments from companies. |
| R4-2003575 | Return to |
| R4-2004337 | Return to Need harmonize with 4419. |
| R4-2004419 | Return to Need harmonize with 4337. |
| R4-2003610 | Revised It is suggested to capture Nokia comment by changing the reason for change in the cover page. |
| R4-2004287 | Endorsed. |
| R4-2004238 | Return to More discussion with Nokia is needed. |
| R4-2004277 | Endorsed |
| R4-2004417 | Endorsed |

## Discussion on 2nd round (if applicable)

Moderator tries to summarize discussions for the 2nd round.

|  | Status summary |
| --- | --- |
| R4-2005269 | WF on SCell activation requirements The draft was submitted. The discussion is on-going. -------------------------------------------------------- [Huawei]: Here we would also like provide additional response to the comments received in the 1st round from Nokia, Ericsson and Apple. Hope it can help to clarify a bit. - Issue 5-5: Interruption window definition for SCell activation - To Nokia: We have no strong view on the naming. The end point is needed, as explained also in the WF, the RF retuning and AGC may or may not be adjacent in time, and in case not, the “end point” is not identical to the “start point + duration”. - To Ericsson: for FR2 known case with periodic CSI-RS, the first SSB will be used for fine time tracking. Since time tracking is not dependent on the CSI-RS for CQI, we understand there is no need to consider the RRC delay for configuring the CSI-RS. So in the WF there is no differentiation based on the CSI-RS (semi-persistent or periodic). - Issue 5-6: SSB pattern for FR2 intra-band CA case for SCell activation - To Apple: we understand from your further comments that the condition on same SSB pattern is needed, is it right? - Issue 5-7: Definition of TFirstSSB and TFirstSSB_MAX - To Nokia: we understand the 5ms is the sum of the 3ms for processing the MAC CE for activation command and the 2ms for processing the SSB to acquire the timing, while the duration of the SSB burst is currently missed. - Issue 5-8: Restriction of RRM measurements for unknown SCell activation - To Nokia: As also explained in the WF, it’s true that UE is allowed cell search time, but it is not shared with other L1 and L3 measurements (no scaling is applied). In case UE dedicates the searcher for the to-be-activated SCell, it cannot at the same time meet the requirements for other L3 and L1 measurement. [Nokia]: Page 3: We assume this option 2 addresses unknown SCell in FR2? [HW] Yes, it is for FR2 unknown case. [Nokia] ok Page 4: This is only background info, but it is important to distinguish between interrupt (interruption duration/length and when this may happen) and then SCell activation delay. This seems not to be clear on this page. Anyhow, this is only background. [HW] Yes, it is only background information. The intention is just try to explain when RF re-tuning and AGC may be performed, and it is all about where interruption may happen but not the activation delay. Hope this clarifies a bit. [Nokia] Thanks it does Page 5: We would like to understand why ‘+Ninterruption’ is needed. The interrupt duration happens during the interrupt window and does not increase the interrupt window. [HW] We have same understanding that interrupt duration happens during the interrupt window. If the start point of the interruption window is slot n+k, and the interruption duration is Ninterruption, then the end point of the interruption window would be n+k+Ninterruption, meaning no interruption is allowed after this time point. Not sure if I got something wrong here, but please let me know your views. [Nokia] ok, I might have misread the line a bit. But in first bullet we have start and end point. And here the window seems to be of length Tx+3ms. In second bullet we also list end point and here it is 3m+Tx+Ninterrupt. Maybe we can clarify this because to me the latter indicates that the interrupt window is extended by Ninterrupt? Page 6: The line ‘The requirement is assuming UE to fine time tracking based on the timing and beam information obtained from the active serving cell’ is also our understanding as this is intra-band CA. However, this is not the same as ‘The parameter ssb-PositionsInBurst is same for the serving cell(s) and the Scell’ which would need more discussion. [HW] We can leave this part as FFS in next meeting. I will update the WF and the CR accordingly. [Nokia] Fine to postpone Page 11: We think what can be expected is a longer delay on UE side related to thses requiements. Hence, we suggest ‘A longer delay for RRM requirements…..’ seem more appropriate and reflecting expected UE behaviour. [HW] This is also fine for us, and maybe we can re-use the wording from the discussion on FR2 SMTC alignment: “Longer delays for RRM measurement requirements and in case of FR2 SSB based RLM/BFD/CBD/L1-RSRP measurement requirements, can be expected during the cell detection time for unknown SCell activation”. Please let me know if this wording is ok or not. [Nokia]: Looks fine. One small edit ‘Longer delays for RRM measurement requirements, and in case of FR2 also SSB based RLM/BFD/CBD/L1-RSRP measurement requirements, can be expected during the cell detection time for unknown SCell activation’ Hence, these comment then of course also apply to the referenced CR’s in we do not manage to comment directly (it should be 4337 and 4419). [Ericsson] This is how I tried to capture it in a revision of 4419: The starting point of an interruption window on PCell or any activated SCell in MCG for NR standalone mode, or on PSCell or any activated SCell in SCG for EN-DC mode, shall not occur before slot n+1+THARQNR slot length and not occur after slot n+1+THARQ+3+TXNR slot length , where TX is: - TFirstSSB, for any scenario where Tactivation_time includes TFirstSSB; - TFirstSSB_MAX, for any scenario where Tactivation_time includes TFirstSSB_MAX; - Tuncertainty_MAC +TFineTiming, for any other scenario where Tactivation_time includes TFineTiming. The end-point of the interruption window shall not occur after slot n+1+THARQ+3+TXNR slot length + Ninterruption, where Ninterruption is the interruption window length as defined in section 8.2, where further the applicable interruption window length depends on whether the interrupted serving cell is in the same band (intra-band) as, or in a different band (inter-band) to, the SCell being activated. Please check whether it is agreeable. Otherwise maybe we should revert to the original version in 4419 (starting point + length). [Huawei] Thanks for the further comments. Please find some response for Page 5 below, and hopefully it can clarify. Page 5: We would like to understand why ‘+Ninterruption’ is needed. The interrupt duration happens during the interrupt window and does not increase the interrupt window. [HW] We have same understanding that interrupt duration happens during the interrupt window. If the start point of the interruption window is slot n+k, and the interruption duration is Ninterruption, then the end point of the interruption window would be n+k+Ninterruption, meaning no interruption is allowed after this time point. Not sure if I got something wrong here, but please let me know your views. [Nokia] ok, I might have misread the line a bit. But in first bullet we have start and end point. And here the window seems to be of length Tx+3ms. In second bullet we also list end point and here it is 3m+Tx+Ninterrupt. Maybe we can clarify this because to me the latter indicates that the interrupt window is extended by Ninterrupt? [HW2] The first bullet in Page 5 is all about the start point of the interruption window. It is not a fixed time point but there is a time window (this is not the interruption window) and the start point may be anywhere in this window. The window is defined by when UE does the RF re-tuning, and as mentioned in Page 4, it can be at - Earliest: UE finishes MAC CE processing for the activation command – in this case n+1+THARQNR slot length - Latest: the first SSB burst UE is going to use – in this case n+1+THARQ+3+TXNR slot length The second bullet in Page 5 is about the end point of the interruption window. It is defined by the latest possible starting point + Ninterruption. [Ericsson] I have concerns on Issue 5-5, and sorry for not spotting this earlier. When using a separate definition for the endpoint of the interruption window, one is in principle allowing unlimited interruptions during the time from the processing of the MAC command and until said endpoint. This would be a drastic and undesirable change since today only a single interruption is allowed. I also have some concern on why one would separate RF re-tuning and gain setting on intra-band. If doing the RF retuning at a different time from gain search, it would potentially mean that one cannot receive intra-band serving cell(s) from the point in time for RF retuning until a proper gain has been found. This time is desirable to keep as short as possible, hence the interruption requirement e.g. in 8.2.2.2.2 where for intra-band the interruption is allowed to be one SMTC window plus margin (RF retuning + gain search using the SSB burst). [Apple] For this issue 5-5, we are wondering if we could also include the deactivation interruption range in the WF as well. I think that case will be simple to add just the interruption duration into the equation. Thanks! [Huawei] @Jerry, the outcome of issue 5-5 will be captured in the revised CR from Joakim, so I guess your comments can be accounted by Joakim. @Joakim, just to clarify, the intention to define the end point of the interruption window is not to allow “unlimited interruptions during the time from the processing of the MAC command and until said endpoint”. The length of interruption is still limited by the requirements defined in section 8.2. To us it is similar as in LTE, where the interruption window is from n+5 to n+9 while the interruption length is 1ms within the window for inter-band case. With this clarification, would it be ok for Ericsson to have the separate end point? [Ericsson] @Li: Thank you for the clarification. Yes, then we are fine with it. Just need to work a little on the wording then for the related draft CR. @Jerry: I will have a look at it (the draft CR so far only covers the activation section in 8.3.2). If feasible, we can include it in this draft. Otherwise maybe better to come back on the deactivation next meeting? [Apple] Regarding the deactivation requirement, either including in the existing draft CR or come back next meeting is fine to us, since technically we are aligned. [Nokia] Sorry about late input due to compressed time schedules. Thank you for the discussion and elaboration. I see the issue but why isn’t it enough just to have earliest starting point and latest ending point for the overall window within which the UE may have an interruption. Right now I cannot see the benefit of this more detailed split – maybe you can clarify the gain? [Huawei] Thanks for the comments, I think it’s a good point. @Joakim, what do you think? Do you remember how we started to define the time window for the start point of the interruption window in last meeting? [Ericsson] @Li, the starting point was that we were missing interruption window definitions for Direct SCell activation, and while looking at it, I realized that while we had updated/streamlined the SCell activation timelines to be based on e.g. TFirstSSB, we had not done the same for the interruption window; that definition still said that a UE was allowed to cause an interruption anywhere between X and X+1 SMTC period. The interruption length and hence the end-point depends on whether we are observing the same activation of an SCell from an intra-band or inter-band CC. Hence the end-point needs to be “flexible” to accommodate that some CCs are intra-band and others are inter-band to the CC at which an SCell is activated. A short interruption on a inter-band CC due to RF retuning, but at the same time a longer interruption on an intra-band CC due to RF tuning + gain search/LNA setting which might require a full SMTC window duration. My earlier proposal was to just define the starting point, and then let duration – as specified in clause 8.2 – represent the flexible part since different durations are specified for inter- and intra-band. In the revised proposal, the only difference is that the end point is explicitly mentioned. I do not have a strong opinion as long as we still are discussing just a single interruption. I think maybe it is easier in the specification text to convey that it is a single interruption when just specifying the starting point, and then let the duration depend on whether it is intra-band or inter-band, but as I said – I have no strong opinion on whether to explicitly define the end-point or not. But I would rather see that we can close this issue before next meeting, so any suggestions on how to proceed would appreciated. Can we either endorse the current version, or endorse it and agree that we submit a revision at next meeting (I think that is possible, right)? Regardless how we describe it, it will describe the same thing. 😊 [Huawei] Many thanks for the background information. I agree that the end point should be “flexible” to account for the difference between intra-band CC and inter-band CC, and as such it depends on the interruption duration Ninterruption. On the other hand, I understand the point from Lars is about the start point: “isn’t it enough just to have earliest starting point and latest ending point”. As for now I have put two options for the start point in the WF, that start point of interruption window during SCell activation Option1: shall not occur before slot n+1+THARQNR slot length and not occur after slot n+1+ THARQ+3+TXNR slot length, Option 2: shall not occur before slot n+1+THARQNR slot length Currently we are thinking along option 1, but is option 2 also enough as commented by Lars? [Ericsson] We can change it to one starting point and one ending point. The former will be fixed, whereas the latter will depend on whether it is intra or inter-band. That should be fine. [Huawei] Thanks for your comments. Then it should be - Start point of interruption window during SCell activation shall not occur before slot n+1+THARQNR slot length - End point of interruption window during SCell activation shall not occur after slot n+1+ THARQ+3+TXNR slot length + Ninterruption, where Ninterruption is the length of interruption defined in section 8.2 and TX is: - TFirstSSB, for any scenario where Tactivation_time includes TFirstSSB; - TFirstSSB_MAX, for any scenario where Tactivation_time includes TFirstSSB_MAX; - Tuncertainty_MAC +TFineTiming, for any other scenario where Tactivation_time includes TFineTiming @Lars, could you please check if above is agreeable? If so, we will capture it in the WF and the CR. |
| R4-2003493 | draftCR on TCI state switch Draft revised version was uploaded. With the clarification, the revised version can be endorsed. ----------------------------------------------------------------------------------------------------- [Mediatek] To Nokia, Based on Nokia’s suggestion, we change the CSI-RS for CQI to CSI-RS only. For issue 1,2,3, they were already agreed in last meeting. Since some copy-paste errors, MCC suggest me to re-submit these changes in this meeting. For example, what your mentioned issue 3, actually, this is based on Nokia’s agreed CR in last meeting. - correction to 8.10.3 is not correct according to our understanding. 38.214 states 3ms. To QC, The reason why we delete the PUCCH/PUSCH scheduling restriction is as follow. How to transmit the PUSCH is based on spatial info. If this spatial RS isn’t the DL RS in TCI switch, of course it’s fine to UE to transmit the UL. If this spatial RS is the DL RS in TCI switch. The RRC reconfiguration only refresh the TCI state, UE still can transmit the UL based on the spatial relation info. with the DL RS. [Nokia] Thank you for the new revision. The CR is as such fine – one more clarification. One more question: ‘2. If the target TCI state is unknown, the UE shall execute L1-RSRP measurement with CSI-RS which shall be set with Repition ON’ would need more discussion. [Mediatek] I need a bit more clarification on this issue 2 below. This issue was copied from an already agreed CR in last meeting. You can double check with the chairman notes and the agreed CR R4-2002339. R4-2002339 CR on TCI state switch Type: CR For: Agreement 38.133 v15.8.0 CR-0475 Cat: F (Rel-15) Source: MediaTek inc. Discussion: Session chair: no Cat A CR. Needs to be requested Decision: Agreed.. But I had a copy-paste error when merging the contents from other companies in last meeting. Thus, MCC noticed that this CR was not captured in the spec. From: 3gpp_tsg_ran_wg4: tsg ran working group 4 [mailto:3GPP_TSG_RAN_WG4@LIST.ETSI.ORG] On Behalf Of Kai-Erik Sunell Sent: Monday, April 6, 2020 6:11 PM To: 3GPP_TSG_RAN_WG4@LIST.ETSI.ORG Subject: Re: RAN4 CR implementation, post-RAN#87-e (v6), a reminder to specification rapporteurs and CR authors Dear all, This is to inform you that draft 38.133 implementations are available for CR authors, specification rapporteur’s and everyone to review ftp://3gpp.org/tsg_ran/WG4_Radio/Draft%20Specs/After_RAN_87e/ Please see below comments on the implementation. CR0475r2 R4-2002339 & CR0585 R4-2002327 (MediaTek inc.) There is no revision mark of removing the original text in the spec. The original texts "at the first slot that is after" was replaced by the texts "no later than at slot" without any tracking mark. The CRs are not implemented.. So I just copied the agreed contents in CR R4-2002339 back into our new CR and added the note in CR change reason. The following columns were agreed in RAN4 #94-e meeting but not implemented in the TCI state switch requirements - The basic UE timing requirements needs to be aligned with the RAN1 requirements. - If the target TCI state is unknown, the UE shall execute L1-RSRP measurement with CSI-RS which shall be set with Repition ON - There is a mismatch between RAN1 spec and RAN4 spec on this TCI change delay. The CR capturing the following new update TCI state switch requirements - PUSCH QCL relation is based on spatial relation and will be defined in active spatial relation switching delay. - Scheduling restriction shall be also applied to CSI-RS for CQI.. Thus, in this meeting, only two new updates in TCI state CR. I’m very sorry for the misunderstanding on the CR change. [Nokia] Thanks for the clarification and sorry for the confusion. |
| R4-2003575 | CR for correction to MAC-CE based TCI State switch timeline (Clause 8.10.3) No further discussion was organized in 2nd round. It was suggested to be noted. |
| R4-2004337 | CR on SCell activation requirements R15 Draft revised version was uploaded. No comments received for the revised version until April 29. It can be endorsed. |
| R4-2004419 | Draft CR 38.133 (8.3.2) Corrections to SCell Activation Delay Requirements Draft revised version was uploaded. It can be endorsed. ----------------------------------------------------------------------------------------- [Ericsson] Thank you for the way forward. I have concerns on Issue 5-5, and sorry for not spotting this earlier. When using a separate definition for the endpoint of the interruption window, one is in principle allowing unlimited interruptions during the time from the processing of the MAC command and until said endpoint. This would be a drastic and undesirable change since today only a single interruption is allowed. I also have some concern on why one would separate RF re-tuning and gain setting on intra-band. If doing the RF retuning at a different time from gain search, it would potentially mean that one cannot receive intra-band serving cell(s) from the point in time for RF retuning until a proper gain has been found. This time is desirable to keep as short as possible, hence the interruption requirement e.g. in 8.2.2.2.2 where for intra-band the interruption is allowed to be one SMTC window plus margin (RF retuning + gain search using the SSB burst). [Apple] Many thanks for the draft WF. For this issue 5-5, we are wondering if we could also include the deactivation interruption range in the WF as well. I think that case will be simple to add just the interruption duration into the equation. Thanks! [Huawei] In this revision the scope has been reduced not to be in conflict with CR R4-2004337. Moreover, starting point and ending point of the interruption window are now explicitly defined, as suggested by Huawei. Comments are welcome. @Jerry: I have not updated the interruption for deactivation of SCell (clause 8.3.3). I think that would need a further discussion - hence I propose it to be handled separately. Best regards Joakim ----------------------- Hi Jerry, Joakim, Thanks for the discussion. @Jerry, the outcome of issue 5-5 will be captured in the revised CR from Joakim, so I guess your comments can be accounted by Joakim. @Joakim, just to clarify, the intention to define the end point of the interruption window is not to allow “unlimited interruptions during the time from the processing of the MAC command and until said endpoint”. The length of interruption is still limited by the requirements defined in section 8.2. To us it is similar as in LTE, where the interruption window is from n+5 to n+9 while the interruption length is 1ms within the window for inter-band case. With this clarification, would it be ok for Ericsson to have the separate end point? |
| R4-2005270 | Revised from R4-2003610. CR for SCell activation delay in FR2 Draft revised version was uploaded. It can be endorsed. ----------------------------------------------------------------------------------------- [Mediatek] The revision is to capture Nokia’s comment received in the 1st round: “Nokia: Change is fine if needed and reason is clear. But current reason for change is not clarifying the need for this change.” the cover page has been updated as Reason for change:; This CR is proposed to reflect the agreement in RAN4#93 Agreement Fine time tracking will not depend on the SP CSI-RS activation used for CSI reporting Currently, when semi-persistant CSI-RS is configured for CSI report, Tuncertainty_MAC will include the waiting time of MAC-CE for SP CSI-RS. However, performing fine timing tracking (TFineTiming ) is not necessarily to wait for the SP CSI-RS.. Please let me know if you have any comment to the CR. [Nokia] This clarification helps – thanks. We can agree to the CR. |
| R4-2004238 | Correction on Psharingfactor Draft revised version was uploaded. No comment received until April 29. Seems it can be endorsed. |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2005269 | On-going |
| R4-2005430 | Endorsed. Revised from R4-2003493. |
| R4-2003575 | Not pursued. |
| R4-2005834 | Endorsed. Revised from R4-2004337. |
| R4-2005426 | Endorsed. Revised from R4-2004419. |
| R4-2005270 | Endorsed. Revised from R4-2003610. |
| R4-2005835 | Endorsed. Revised from R4-2004238. |

# Topic #7: Beam management based on SSB and/or CSI-RS

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2003611 | Mediatek | Scheduling restriction due to SSB configured for aperiodic L1-RSRP reporting on FR2 In this paper, our view on SSB configured for aperiodic L1-RSRP reporting on FR2 is provided. We have the following observations and proposals: Observation 1: When the reporting type is aperiodic and the scheduling restriction is based on the reporting triggering, measurement time will be insufficient for the SSB based L1-RSRP in FR2, and it will not meet the corresponding accuracy requirement. Observation 2: Scheduling restriction should apply on the SSB configured for L1-RSRP measurement, no matter if the reporting type is periodic, semi-persistent or aperiodic, in order to provide the specified measurement time for SSB based L1-RSRP in FR2. Proposal 1: Down-select one of the following options, - Option 1: L1-RSRP accuracy requirement doesn’t apply for SSB based L1-RSRP in FR2, when the reporting type is aperiodic. - Option 2: Scheduling restriction should apply once the SSB is configured for L1-RSRP measurement, no matter the reporting type is periodic, semi-persistent or aperiodic. |
| R4-2004001 | ZTE | On definition of TCI chain Observation 1: With the current definition, it’s not reasonable to allow multiple QCL types in one TCI chain. Proposal 1: Clarify in the specification that “A TCI chain comprises of a single QCL Type.” |
| R4-2004809 | Qualcomm | QCL chain depth restriction in Rel-15 RRM requirements Observation 1: Adoption of the proposal in [2] may lead to multiple reference signals of different types to co-exist in the same chain. - With this proposal, reference signal 1 and 5 in scenario 3 of figure 1 will exist in the same chain and that is undesirable. Proposal 1: RAN4 introduces the following text in the spec, that was proposed in [3], for clarification (change shown in red color): ------------------ TP begins --------------------------------------------- 3.6.7 Applicability of QCL “For the requirements specified in this version of the specification, a reference signal is considered to be QCLed to another reference signal if it is in the same TCI chain as the other reference signal, provided that the number of Reference Signals in the chain is no more than 4. Even though a TCI state may have up to two QCL types, there is single QCL type per TCI chain.” -------------------- TP end ------------------------------- Observation 2: Proposal 1 allows one reference signal to be located in two different QCL chains and QCLed to two different reference signal w.r.t. different types. This is allowed according to RAN1 and RAN2 specs. |
| R4-2003612 | Mediatek, Huawei | CR on SMTC period for beam management requirements - Add clarification on TSMTCperiod for multiple FR2 CCs. - Add clarification on smtc1 and smtc2 for TSMTCperiod in candidate beam detection. |
| R4-2003407 | Apple | draft CR on SMTC2 configuration in SSB based CBD for R15 Add the SMTC2 configuration and exception condition into SSB based CBD requirement, which is same as in BFD requirement. |
| R4-2004282 | Huawei, HiSilicon | CR on CSI-RS based BFD Add the condition that the bandwidth of CSI-RS shall larger than 24 PRBs |
| R4-2004283 | Huawei, HiSilicon | CR on CSI-RS based CBD Add the condition that the bandwidth of CSI-RS shall larger than 24 PRBs |
| R4-2003613 | Mediatek | CR for CSI-RS based L1-RSRP measurement period Add side condition that QCL-Type D should be provided in FR2 for CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON. |

## Open issues summary

### Sub-topic 6-1

Issue 6-1: Scheduling restriction due to SSB configured for aperiodic L1-RSRP reporting on FR2

- Proposals (Mediatek R4-2003611) Down-select one of the following options,

  - Option 1: L1-RSRP accuracy requirement doesn’t apply for SSB based L1-RSRP in FR2, when the reporting type is aperiodic.

  - Option 2: Scheduling restriction should apply once the SSB is configured for L1-RSRP measurement, no matter the reporting type is periodic, semi-persistent or aperiodic.

- Recommended WF

  - Further discussion is needed.

### Sub-topic 6-2

Issue 6-2: TCI Chain and QCL

- Proposals for TCI chain (ZTE R4-2004001)

  - Clarify in the specification that “A TCI chain comprises of a single QCL Type.”

- Proposal for QCL chain depth restriction (Qualcomm, R4-2004809)

  - RAN4 introduces the following text in the spec, that was proposed in [3], for clarification (change shown in red color):

------------------ TP begins ---------------------------------------------

3.6.7	Applicability of QCL

“For the requirements specified in this version of the specification, a reference signal is considered to be QCLed to another reference signal if it is in the same TCI chain as the other reference signal, provided that the number of Reference Signals  in the chain is no more than 4. Even though a TCI state may have up to two QCL types, there is single QCL type per TCI chain.”

-------------------- TP end  -------------------------------

- Recommended WF

  - Agree on “ There is a single QCL type per TCI chain”.

### Sub-topic 6-3

For CR R4-2003612, R4-2003407, R4-2003613, R4-2004282, and R4-2004283, please provide the comments directly in Section 6.3.2.

There is partial overlapping part between R4-2003612 and R4-2003407.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Sub topic 6-1: Our preference is option 1, Sub topic 6-2: Basically the WF is ok, but we would prefer the wording “it is assumed there is single QCL type per TCI chain” Sub topic 6-3: …. Others: |
| MediaTek | Issue 6-1: Scheduling restriction due to SSB configured for aperiodic L1-RSRP reporting on FR2 The intention of this proposal is quickly recapped below. From subclause 5.2.1.4 of TS 38.214, slot offset between aperiodic reporting request and PUSCH is provided by reportSlotOffsetList. For a semi-persistent or aperiodic CSI report on PUSCH, the allowed slot offsets are configured by the higher layer parameter reportSlotOffsetList. The offset is selected in the activating/triggering DCI.. And the max offset is 32 slots (max 8ms in 60KHz), as specified in TS 38.331 aperiodic SEQUENCE { reportSlotOffsetList SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF INTEGER(0..32) However, in L1-RSRP measurement time specified in subclause 9.5.4.1 of TS 38.133 in FR2 could take an even longer time than 32 slots, e.g., 24 x SMTC periods. (480ms when SSB period is 20ms) Thus, we propose 2 options to resolve this problem. Either way is fine to us. Issue 6-2: TCI Chain and QCL Agree with the Recommended WF. |
| Apple | Sub topic 6-1: Option 2 Sub topic 6-2: Okay with moderator’s recommended WF |
| Huawei, HiSilicon | Issue 6-1: Scheduling restriction due to SSB configured for aperiodic L1-RSRP reporting on FR2 We prefer option 2. For option 1, the measurement will be meaningless and the report will be a waste of resource for both network and UE. We understand SSB can only be configured for L1-RSRP measurement as periodic resource (no matter if the reporting is periodic, semi-persistent or aperiodic). Issue 6-2: TCI Chain and QCL We support the recommended WF. |
| Intel | Issue 6-1: Scheduling restriction due to SSB configured for aperiodic L1-RSRP reporting on FR2 We support option 1. Option 2 may lead to throughput degradation when L1-RSRP is not triggered. Issue 6-2: TCI Chain and QCL We are OK with recommended WF. |
| Qualcomm | Sub topic 6-1: We propose a third option as below: For SSB based aperiodic L1-RSRP reporting, The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/CSI-RS for tracking/CSI-RS for CQI on configured SSB for L1-RSRP measurement. The UE is expected to receive PDSCH on configured SSB for L1-RSRP measurement if the configured SSB is QCLed with the DMRS of PDSCH w.r.t. type D. |
| NTT DOCOMO, INC | Issue 6-1: Scheduling restriction due to SSB configured for aperiodic L1-RSRP reporting on FR2 We support Option 2. If option 1 is applied, measurement result of L1-RSRP would be meaningless. Even if L1-RSRP measurement is not triggered, scheduling restriction would occur due to SSB based L3 measurement or RLM and so on, so keeping accuracy requirement is more important. QC’s proposal is also fine for us. |
| Nokia, Nokia Shanghai Bell | Sub-topic 6-1: Issue 6-1: Scheduling restriction due to SSB configured for aperiodic L1-RSRP reporting on FR2 We understand the issue raised but our understanding is that the current text referred to does not limit when the UE measures. In fact, it seems very clear that e.g. for triggered reporting UE would have to measure continuously as otherwise it would not be able to evaluate the trigger condition. Same for aperiodic trigger – UE is assumed to measure continuously once configured as otherwise the UE would not be able to report. We do not support either options. We have commented on option 2 multiple times that there is difference between configured and measured. We are wondering what change is needed? Sub-topic 6-2: Issue 6-2: TCI Chain and QCL The TP in Qualcomm paper starts to be aligned with our view. We understand the intention but we need to see how to capture this ‘direct TCI chain’ while not excluding the other TCI types. Recommended WF is to work on the detailed TP. |

### CRs/TPs comments collection

CRs included in the above sub topics are not listed here.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2003612 | Ericsson : Propose to merge 3612 and 3407 |
|  | Apple: Agree with updates. Some additional changes are captured in R4-2003407 for CBD section. Could we merge |
|  | Nokia: It is not clear to us what the intention with the change is? E.g. the line ‘TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, given the SMTC offset of all CCs are the same on the same band’ UE performs RLM on PCell and PSCell only. And for L1_RSRP specification say: ‘Upon request the UE shall be able to evaluate whether the L1-RSRP measured on the configured SSB resource’ And now this new text is potentially saying something else. Can lead to inconsistency? More discussion needed (and also how this relates to 3407) |
| R4-2003407 | Ericsson : Propose to merge 3612 and 3407 |
|  | MediaTek: It is OK for us. |
|  | Apple: Okay to merge with R4-2003612 |
|  | Qualcomm: Following typo needs to be clarified in the CR (black denotes the text of the CR; typo shown and corrected in red color): "The overlap between SSB CSI-RS for CBD and SMTC means that SSB CSI-RS for CBD is within the SMTC window duration.". |
|  | Nokia: Our understanding is that smtc2 is per cell: ‘If smtc2 is present, for cells indicated in the pci-List parameter in smtc2 in the same MeasObjectNR’ Which needs to be considered. It is not clear if this new text in related to Psharing? The text change related the new ‘longer evaluation..’ condition is not clear. It is unclear what it refers to. More discussion is needed. |
| R4-2003613 | Ericsson : OK |
|  | MediaTek: We realized the track changes on CR R4-2003613 are missing. We will request a new Tdoc and the version with track chages has been shared in the email reflector with titile ” [RAN4 R15 RRM] Revision for R4-2003613 CR for CSI-RS based L1-RSRP measurement period”. In the current SPEC, QCL-Type D is missing in some descriptions of N factor, while N factor intends to capture UE behavior related to QCL-Type D, i.e. RX beam behavior. Without this revision, the requirements will also be applicable to QCL-Type A/B/C. |
|  | Apple: No track changes. Okay with the change. |
|  | Nokia: no change marks. |
| R4-2004282 | Ericsson : OK |
|  | MediaTek: It is OK for us. |
|  | Apple: Is the change necessary? The minimum BW for CSI-RS is 24 PRB. |
|  | Huawei, HiSilicon: we will further check on the necessity, and the CR can be postponed. |
|  | Nokia: OK |
|  | NEC: Minimum CSI-RS BW is 24 RBs. OK for us as clarification CR |
| R4-2004283 | Ericson : OK |
|  | MediaTek: It is OK for us. |
|  | Apple: Is the change necessary? The minimum BW for CSI-RS is 24 PRB. |
|  | Huawei, HiSilicon: we will further check on the necessity, and the CR can be postponed. |
|  | Nokia: OK |
|  | NEC: Minimum CSI-RS BW is 24 RBs. OK for us as clarification CR |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic# 6-1 | 8 companies made the comments. 3 companies supported Option 1. 3 companies supported Option 2. 1 company provided the additional option. 1 company disagreed with two option and was not convinced by the necessity of the change. The views are quite diverse. Tentative agreements: N/A Candidate options: - Option 1 (Ericsson, Mediatek, Intel): L1-RSRP accuracy requirement doesn’t apply for SSB based L1-RSRP in FR2, when the reporting type is aperiodic. - Option 2 (Apple, Huawei, NTT DOCOMO) : Scheduling restriction should apply once the SSB is configured for L1-RSRP measurement, no matter the reporting type is periodic, semi-persistent or aperiodic. - Option 3 (Qualcomm): - For SSB based aperiodic L1-RSRP reporting, The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/CSI-RS for tracking/CSI-RS for CQI on configured SSB for L1-RSRP measurement. The UE is expected to receive PDSCH on configured SSB for L1-RSRP measurement if the configured SSB is QCLed with the DMRS of PDSCH w.r.t. type D. - Option 4 (Nokia): no change Recommendations for 2nd round: Further discussion is needed. |
| Sub-topic# 6-2 | 6 companies made comments and can accept the recommended WF in principle. 2 companies had comments on how to capture the agreement. Tentative agreements: It is assumed there is single QCL type per TCI chain. Candidate options: Recommendations for 2nd round: It is suggested to allocate a draft CR and companies can work together on it. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

Suggestion on new draft CR assignment

|  | CR t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | Draft CR on applicability of QCL | Qualcomm |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| New CR | Draft CR on applicability of QCL As proposed in the Table above in Section 6.4.1. |
| R4-2003612 | Revised Response to Nokia is needed. |
| R4-2003407 | Merged into R4-2003612 Response to Nokia is needed. |
| R4-2005231 | Revised from R4-2003613 due to no change mark. More discussion is needed on the revised version. |
| R4-2004282 | Return to |
| R4-2004283 | Return to |

## Discussion on 2nd round (if applicable)

Moderator tries to summarize discussions for the 2nd round.

|  | Status summary |
| --- | --- |
| R4-2005271 | (New) Draft CR on applicability of QCL Draft CR was uploaded. The draft CR is agreeable and Rel-16 Cat F CR is needed. Both Rel-15 and Rel-16 draft CRs can be endorsed. -------------------------------------------------------------------------------------------------- [Qualcomm] Please note that version 15.9.0 of 38.133 does not seem to contain section 3.6.7, i.e., “Applicability of QCL” section, and I am not sure why. The section was in version 15.8.0. Hence, I copied this section from version 15.8.0 and put it in the CR. Besides, I followed the moderator’s guideline from the 1st round summary and added the following sentence to this section “It is assumed there is single QCL type per TCI chain.” [Huawei] Thanks for the draft. We are fine with the added sentence. Just to clarify – as far as I can see section 3.6.7 is there in 15.9.0, could you please double check? On the other hand, the section is indeed missing in 16.3.0, so could you please also make a Rel-16 CR to add the section? [Nokia] Thank you for the draft. Nokia is fine with this CR. [Qualcomm] Thanks a lot for all of your feedback. 1. I have updated the Rel-15 CR based on your comments and uploaded in https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_94_eBis/Inbox/Drafts/%5B94e%20Bis%5D%5B101%5D%20NR_NewRAT_RRM_Core/Draft%20R4-2005271%20CR%20on%20applicability%20of%20QCL%20v2.docx. Zhangli is correct. Rel-15 indeed had “applicability of QCL” section. So, I updated the CR to add one sentence that reflects the 1st round summary. If there is no other feedback regarding this CR, I will upload the official CR in inbox before 5 pm UTC. 2. As suggested by Zhangli, I have uploaded a draft mirror CR for Rel-16 in https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_94_eBis/Inbox/Drafts/%5B94e%20Bis%5D%5B101%5D%20NR_NewRAT_RRM_Core/Draft%20R4-200xxxx%20CR%20on%20Applicability%20of%20QCL%20for%20Rel-16.docx. @Daixizeng: Can I get a Tdoc number for this mirror CR now? If not, I will submit this CR in the next meeting. Please let me know if you have any additional comment. [NEC] Provide the update. [Nokia] CR looks fine from our side. |
| R4-2005272 | Revised from R4-2003612. CR on SMTC period for beam management requirements The revised version was uploaded. Discussion is on-going. According to feedback until the submission of this summary, it is suggested to continue the discussion. -------------------------------------------------------------------------------- [Mediatek] The CR has been revised to merge with 3407 - on 8.5.5.2 & 8.5.6.2, - TSMTCperiod in candidate beam detection (8.5.5.2 & 8.5.6.2), the clarification on smtc1 and smtc2 is missing. - The exception condition for longer evaluation delay is missing in SSB based CBD requirement. - on 9.5.3.1, 9.5.3.2, 9.5.3.3 - correction on the wrong section IDs In response of Nokia’s comment, - Regarding “the line ‘TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, given the SMTC offset of all CCs are the same on the same band”, the justification is provided as follows - Although UE performs RLM/BFD on PCell/ PSCell, the P factor of RLM/BFD are also impacted by the SMTCs of other SCells. - E.g., SMTC of 40 ms periodic on on PCell/PSCell, and SMTC of 10 ms periodic on on SCells. - In this case the P factor of RLM should be based on the SMTC of 10 ms (i.e. the shortest SMTC period among all CCs), because the UE RX beam during SMTC will not be fixed to serving cells, due to L3 measurement. - The overlapping part with 3707 is on 8.5.5.2 & 8.5.6.2, as listed above. [Nokia] I still have a clarifying question. Looking at the change in 8.1.2.2: In the example below if the PCell has SMTC of 40ms and SCell has 10ms SMTC, then for FR2 the UE would need to be allowed time sweep for SSB measurements to fulfil the RRM measurements. UE will only be able to measure serving cell once per 40ms. Then if: - In this case the P factor of RLM should be based on the SMTC of 10 ms (i.e. the shortest SMTC period among all CCs), because the UE RX beam during SMTC will not be fixed to serving cells, due to L3 measurement. But why does the SMTC of CC impact the RLM-RS except when they overlap? I might be missing something. Hope you can help explaining. Secondly, if this is agreed, the change should be ‘under’ the FR2 Psharing level. [Mediatek] Thanks for your comment. For the 1st clarification, regarding “why does the SMTC of CC impact the RLM-RS except when they overlap?”, in this case, it will be categorized as “partially overlapped” scenario, because it is not fully overlapped with the SMTC, and the P will be relatively tight, comparing the “fully overlapped” case. E.g. - P=11-TSSBTSMTCperiod, when RLM-RS resource is not overlapped with measurement gap and the RLM-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod). On the 2nd question, I add “where” and indent to clarify it is under FR2 for “P”. E.g. in the 8.1.2.2 Where, If the high layer in TS 38.331 [2] signaling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, given the SMTC offset of all CCs are the same on the same band. [Nokia] Thank you for your explanation. I am not sure it fully answers my question and I wonder if this case is covered well. In the example we have Tssb = 40ms and Tsmtc is 40ms on PCell and 10ms on SCell. I wonder why the SMTC periodicity of the SCell impacts the RLM measurements. As I understood the change the intension is to have the Tsmtcperiod in the P equations to always depend on the shortest SMTC of a serving cell. I can understand that the UE need to sweep in FR2 and therefore cannot measure. But of the sweeping for SCell measurements (like in this example) only occasionally impacts RLM measurement it does not seem correct to scale according to shortest SMTC period. [Mediatek] Thank you for the clarification. The current P in FR2 doen’t work if the RLM-RS are additionally punctured by other CC’s SMTC. I might give a bad example, because PCC is unlike to have longer SMTC than other CCs. Could we assume the SMTC of PCC has the shortest SMTC periodicity among all CCs in the same FR2 band? Then, the RLM-RS will not additionally punctured by other CC’s SMTC, and the existing requirement can work. E.g. to update the text as ” For the determination of TSMTCperiod, PCC with the shortest SMTC period among all CCs in the same FR2 band is assumed, given the SMTC offset of all CCs are the same on the same band.” The revision is also available here. [Nokia] I agree that the example is just an example. However, I think this needs more consideration at least from our side before agreeing. To us the ‘shortest’ word at least in some cases might introduce unnecessary long relaxations. Maybe we can continue the discussion tomorrow or before next meeting? |
| R4-2005231 | Revised from R4-2003613 due to no change mark. CR for CSI-RS based L1-RSRP measurement period Draft revised version was uploaded. No comment was received for the revised version until April 29. It can be endorsed. |
| R4-2004282 | CR on CSI-RS based BFD No further discussion was organized. The document is suggested to be noted. |
| R4-2004283 | CR on CSI-RS based CBD No further discussion was organized. The document is suggested to be noted. |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2005271 | Endorsed. Revised from R4-2003612. |
| R4-2005836 | Endorsed. New Rel-16 Cat F CR corresponding to R4-2005271. |
| R4-2005272 | On-going |
| R4-2005231 | Endorsed. |
| R4-2004282 | Not pursued. |
| R4-2004283 | Not pursued. |