3GPP TSG-RAN WG4 Meeting #94-e-Bis		R4-200536

Electronic meeting, 20 – 30 April, 2020

Agenda item:	8.18.2

Source:	CableLabs

Title:	Summary of channel raster analysis in DSS (band 48/n48)

Document for:	Approval

# Introduction

RAN#86 meeting approved WID RP-193213 targeting “Channel raster: Confirm that NR channel raster can be aligned with LTE center frequencies.” Two NR channel raster options, 100 and 300 kHz, were discussed in RAN4 #94-e meeting but no agreement was achieved. A third and fourth raster option of 15 and 30 kHz are in the current draft of the standard for band n48. A WF for further study was proposed. In this contribution, we provide a brief summary of the of DSS in band 48/n48 channel raster challenges and makes appropriate observations and proposals.

Based on the analysis presented in companion document R4-2005035, we find the following high level outcomes for each channel raster under analysis:

1: 15 kHz Channel Raster

- It requires no changes to the actual standard.

- It supports PRB alignment with a coexistence LTE channel

- Requires further study to see if any of the issues found with other raster options are mitigated by using 15 kHz.

2: 30 kHz Channel Raster

- It requires no changes to the actual standard.

- It supports PRB alignment with a coexistence LTE channel

3: 100 kHz Channel Raster

- It breaks the NR release 15/16 specifications (e.g. ARFCN and GSCN compliance).

4: 300 kHz Channel Raster

- It breaks the NR release 15/16 specifications (e.g. GSCN compliance).

# Observations

Based on the analysis in our companion document R4-2003462, our observations are:

Observation 1: The proposed 30 kHz channel raster for band n48 aligns with the Δf_global 15 kHz global raster defined for frequencies above 3 GHz and also with the LTE channel raster.

Observation 2: The 30 kHz channel raster may require  +/- 10 kHz channel alignment offset

Observation 3: The offsetToCarrier would require further analysis.

Observation4: The 15 kHz channel raster for band n48 aligns with the Δf_global 15 kHz global raster defined for frequencies above 3 GHz and also with the LTE channel raster.

Observation 5: The 15 kHz channel raster may require  +/- 5 kHz channel alignment offset

Observation 6: The proposed 100 kHz channel raster for band n48 does not align with the Δf_global 15 kHz global raster defined for frequencies above 3 GHz.

Observation 7: Since 300 kHz is a multiple of both 15 and 30 kHz channel rasters, this case becomes an implementation of either one or both of the 15 or 30 kHz channel rasters.

Observation 8: The proposed 100 kHz channel raster for band n48 does not align with existing GSCN Index values and/or SSB definitions such that each FCC PAL 10 MHz could contain an SSB.

# Conclusions

Proposal 1: A 15 kHz channel raster for band n48 complies with LTE channel alignment in terms of PRB alignment. No changes to existing draft of TS38.101-1-g30_v002 are necessary to allow this alignment.

Proposal 2: A 30 kHz channel raster for band n48 complies with LTE channel alignment in terms of PRB alignment. No changes to existing draft of TS38.101-1-g30_v002 are necessary to allow this alignment.

Proposal 3: Further analysis is required on the offsetToCarrier parameters.