3GPP TSG-RAN WG4 Meeting #94bis-e	R4-2005075

Online, 20th April – 30th April 2020

Source:	Ericsson

Title:	TP to TR 37.716.21-21: Addition of DC configurations for DC_3_n8-n78

Agenda item:	8.7.2

Document for:	Approval

# Background

This text proposal for TR 37.716-21-21 to add DC_3_n8-n78 configurations as defined in WID [1].

# Text Proposal

##### ---Start of changes---

## 6.99	DC_3_n8-n78

### 6.99.1	Operating bands for DC

Table 6.99.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n8-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.99.2	Configuration for DC

Table 6.99.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A_n8A-n78A | DC_3A_n8A DC_3A_n78A | 3A | CA_n8A-n78A |

### 6.99.3	Co-existence studies

Co-existence analysis for DC_3_n8 shows that there are IMD3 and IMD5 impact from DC_3_n8 UL to NR Band n78 DL.

Co-existence analysis for DC_3_n78 shows that there is no impact from DC_3_n78 UL to NR Band n8 DL.

### 6.99.4	∆TIB and ∆RIB values

For DC_3_n8-n78 the TIB,c and RIB,c values are same as for DC_3-8_n78 and are given in the tables below.

Table 6.99.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n8-n78 | 3 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 6.99.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n8-n78 | 3 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 6.99.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD3&IMD5 produced by Band 3 and NR band n8 that impact the reference sensitivity of NR band n78. The required MSD is shown in the following Table.

Table 6.99.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A_n8A-n78A | 3 | 1740 | 5 | 25 | 1835 | N/A | N/A |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n78 | 3540 | 10 | 50 | 3540 | 16.3 | IMD3 |

##### ---End of changes---

# Reference

[1]	RP-200080, “Revised WID on EN-DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)”, LG Electronics

[2]	R4-2002654, “MSD results for new EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16” , LG Electronics