3GPP TSG-RAN WG4 Meeting #94bis-e	R4-2005076

Online, 20th April – 30th April 2020

Source:	Ericsson

Title:	TP to TR 37.716.21-21: Addition of DC configurations for DC_7_n1-n40

Agenda item:	8.7.2

Document for:	Approval

# Background

This text proposal for TR 37.716-21-21 to add DC_7_n1-n40 configurations as defined in WID [1].

# Text Proposal

##### ---Start of changes---

## 6.100	 DC_7_n1-n40

### 6.100.1	Operating bands for DC

Table 6.100.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n1-n40 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

### 6.100.2	Configuration for DC

Table 6.100.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A_n1A-n40A | DC_7A_n1A DC_7A_n40A | 7A | CA_n1A-n40A |

### 6.100.3	Co-existence studies

Co-existence analysis for DC_7_n1 shows that there is no impact from DC_7_n1 UL to NR Band n40 DL.

Co-existence analysis for DC_7_n40 shows that there are IMD3 and IMD5 impact from DC_7_n40 UL to NR Band n1 DL.

### 6.100.4	∆TIB and ∆RIB values

For DC_7_n1-n40 the TIB,c and RIB,c values are reused from CA_1-7-40 and are given in the tables below.

Table 6.100.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n1-n40 | n1 | 0.6 |
|  | 7 | 0.8 |
|  | n40 | 0.9 |

Table 6.100.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n1-n40 | n1 | 0 |
|  | 7 | 0.3 |
|  | n40 | 0.8 |

### 6.100.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD3&IMD5 produced by Band 7 and NR band n40 that impact the reference sensitivity of NR band n1. The required MSD is shown in the following Table.

Table 6.100.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n1A-n40A | 7 | 2540 | 5 | 25 | 2660 | N/A | N/A |
|  | n40 | 2335 | 5 | 25 | 2335 | N/A | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | 15.2 | IMD3 |

##### ---End of changes---

# Reference

[1]	RP-200080, “Revised WID on EN-DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)”, LG Electronics

[2]	R4-2002654, “MSD results for new EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16” , LG Electronics