3GPP TSG-RAN WG4 Meeting #94bis-e                                                         R4-2005136

Online, 20th April-30th April, 2020

Source:	Samsung, KDDI

Title:	TP for TR 37.716-31-11: DC_1-3-41_n78(*)

Agenda item:			8.5.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.716-31-11 to include DC_1-3-41_n78(*) according to the request in [1].

# 2. Reference

- RP-2000102, Revised WID on Dual Connectivity (EN-DC) of 3 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL).

# 3. Text Proposal

<Start of Text Proposal>

## 5.1.5		DC_1-3-41_n78 and DC_1-3-41_n78(*)

### 5.1.5.1	Operating bands for EN-DC

Table 5.2B.4.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-41_n78 | CA_1-3-41 | n78 |  |
| DC_1-3-41_n78(*) | CA_1-3-41 | n78 |  |

### 5.1.5.2	Configuration for EN-DC

Table 5.5B.4.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A-41A_n78A | DC_1A_n78A DC_3A_n78A DC_41A_n78A | CA_ 1A-3A-41A | n78 |
| DC_1A-3A-41C_n78A | DC_1A_n78A DC_3A_n78A DC_41A_n78A | CA_ 1A-3A-41C | n78 |
| DC_1A-3A-41A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_41A_n78A | CA_ 1A-3A-41A | CA_n78(2A) |
| DC_1A-3A-41C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_41A_n78A DC_41C_n78A | CA_ 1A-3A-41C | CA_n78(2A) |

### 5.1.5.3	∆TIB and ∆RIB values

Table 6.2B.4.2.3.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 41 | 0.5 |
|  | n78 | 0.8 |

Table 7.3B.3.3.3-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-41-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 41 | 0 |
|  | n78 | 0.5 |

<End of Text Proposal>