3GPP TSG-RAN WG4 Meeting # 94-e-Bis 												R4-2005399

Electronic Meeting, 20 – 30 Apr., 2020

Agenda item:			6.10

Source:	Huawei, HiSilicon

Title:	Email discussion summary for [94e Bis][117] SRVCC_NR_to_UMTS_RRM

Document for:	Information

Introduction

This email thread discusses the SRVCC core part and performance part in agenda 6.10.1 and 6.10.2.

List of candidate target of email discussion for 1st round and 2nd round:

- 1st round: Invite companies to review the recommended WF in each sub-topic, and provide comments. If no comment is received for a CR, it will be recommended to be agreed in the summary for the 1st round.

- 2nd round: TBA

Topic #1: SRVCC core part

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2003097 | Ericsson | Per UE measurement gap configuration with an FR2 serving cell and GP12-23 is specified as having a purpose of “non NR RAT” after SRVCC updates. However GP12-23 cannot be used for measuring any non NR-RAT (e.g. LTE, WCDMA). Correct this entry of the gap applicability table to have measurement purpose “FR2 only” |

Open issues summary

Please directly provide comments for each CR in section 1.3.2.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |

CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2003097 | Nokia: The draft CR can be endorsed. |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | No need |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2003097 | Suggest to be “endorsed” |

Discussion on 2nd round (if applicable)

N/A

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Topic #2: SRVCC test case

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2003096 | Ericsson | Event triggered reporting test for WCDMA measurements in NR FR1 |
| R4-2004307 | Huawei, Hisilicon | Test case for NR to UTRA FDD Inter-RAT handover |

Open issues summary

Please directly provide comments for each CR in section 2.3.2.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 2-1: Sub topic 2-2: …. Others: |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2003096 | Huawei, HiSilicon: - There are multiple “LTE” in the test. It shall be “UTRAN”. - In Table A.6.6.5.1.1-1, only UTRA FDD duplex mode is configured for SRVCC, thus there shall be 3 configurations. - Table A.6.6.5.1.1-4 is empty (I guess it refers to the below table). |
|  | Ericsson : Thanks for careful checking, we can update it for 2nd round to address the issues identified by Huawei. |
|  | Nokia: The related test spec should be added on cover page. Table A.6.6.5.1.1-2 contains term “E-UTRAN Ec/Io threshold”, which needs to be replaced with CPICH Ec/Io threshold. |
| R4-2004307 | Ericsson : CR is fine for Ericsson. |
|  | Nokia: There are several appearances of “LTE” or “E-UTRAN” in the CR, whilst the handover to UTRAN is targeted. Table A.6.3.1.6-1 should have only 3 entries, aligning to Table A.6.6.5.1.1-1 as commented above. In Table A.6.3.1.6-2, the unit for b2-Threshold2-UTRA is dB rather than dBm. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | No need |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2003096 | to be revised |
| R4-2004307 | to be revised |

Discussion on 2nd round (if applicable)

Please directly provide comments for each CR in section 2.5.1.

CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2005333 (revised from R4-2003096) | Company A |
|  | Company B |
| R4-2005334 (revised from R4-2004307) | Company A |
|  | Company B |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2005333 (revised from R4-2003096) | agreeable |
| R4-2005334 (revised from R4-2004307) | agreeable |