3GPP TSG-RAN WG4 Meeting # 94-e-Bis 												R4-2005484

Electronic Meeting, 20 – 30 Apr., 2020

Agenda item:			6.5.2

Source:	Samsung

Title:	IAB-MT power dynamic range

Document for:	Approval

Introduction

For BS, the transmission is assumed with equal PSD within targeted spectrum channel(s), except limited power boosting/deboosting on necessary PHY layer channel in FR1. And it assumed the BS transmission would be always on in DL time slot. Based on this scenario the dynamic range requirement applies for BS is defined as:

- RE power control dynamic range for BS type 1-C/H/O

- Total power dynamic range for BS type 1-C/H/O and 2-O

In addition, the OFF power and transient time is defined to guarantee the co-existence

While on UE side min TX power, OFF TX power, and Transmit ON/OFF time mask are defined to ensure the interference from own Transmission to victim system reception in nearby environment to an acceptable level similar as BS mask related requirement. Furthermore, it is assumed that the UE should adjust it output power according to power control comment from network, that is why the power control requirement as below applies for UE as well.

- Absolute power control

- Relative power control

- Aggregated power control

With respect to IAB-MT there is no doubt on necessity of dynamic range requirement for at least some IAB-MT class with the open issue to what level it should be. Furthermore, power control also is believed as necessary at least for some IAB-MT class by several companies to enable to adjust with environment changes with existing Donor/parent and the switching to other candidate donor/parent if legacy connection is not good enough. On the other hand, several companies  support consider BS framework for dynamic range for IAB-MT as dynamic changes in backhaul link conditions are not expected for all IAB-MT classes..

And this WF is targeting to gather agreements in RAN4#94-bis-e meeting on topics of

- IAB-MT output power dynamics

  - Tx dynamic range

    - Relative dynamic range or not?

    - IAB-MT specfic or not?

  - Power control

    - Necessity and applicability of power control requirement for IAB-MT

Agreement

- Definition of IAB-MT Tx dynamic range

- Agreement: Relative Tx power dynamic range is derived by delta of maximum EIRP level and minimum EIRP level on declared beam direction within the same channel allocation for FR2 IAB-MT

- FFS whether this definition applies only when IAB-MT transmits in uplink time slot

        FFS on whether Total power dynamic range applies for IAB-MT

- IAB-MT Tx dynamic range

- It is agree to define relative Tx power dynamic range with dependency on  IAB-MT class with candidate options as below:

- Wide area IAB-MT Tx dynamic range to:

  - Option 1: 0 dB

  - Option 12: 5dB Tx dynamic range

  - Option 3: 10dB Tx dynamic range

- [local area] IAB-MT Tx dynamic range:

- Option 1: 10dB Tx dynamic range

- Option 2: 20dB Tx dynamic range

- IAB-MT power control

- It is agreed that need of Power control accuracy requirement can be different for different IAB-MT classes. It is FFS which framework would be applied depending on relative Tx dynamic range to be agreed above, i.e. how small the Tx dynamic range should be to result in no need to define power control accuracy requirements

- Wide area IAB-MT and [Local Area IAB-MT] FFS if based on UE framework as

- Absolute power control

- Relative power control

- Aggregated power control

Reference

[1] R4-2003774, IAB-Node classification and related output power requirements

[2] R4-2004165, IAB-MT Tx power dynamic range

[3] R4-2004150, Further discussion on IAB-MT power requirement

[4]R4-2005029, IAB-MT transmitter RF requirement

[5] R4-2004168, IAB MT power control requirement

[6] R4-2003312, Discussion of IAB MT ON/OFF time mask

[7] R4-2004647, Definition of IAB-MT dynamic range

[8] R4-2003609, IAB MT Tx Requirements

[9] R4-2005437, Email discussion summary for [94e Bis][207] NR_IAB_RF_Part_1