3GPP TSG RAN WG4 Meeting #94bis-e	R4-2005636

Electronic meeting, 	20th – 30th April 2020							Revision of R4-2003549

Agenda Item:	6.4.2

Source:	Futurewei

Title:	TP on Indevice Coexistence

Document for:	Approval

# Introduction

This contribution provides text proposal for the section 6 Evaluation of Indevice Coexistence in the TR 38.886 [1]. The scenario description for evaluation are described.

************* Start of the TP in sub clause 6 of TR 38.886 ***************

# 6	Evaluation of In-device Coexistence

## 6.2 UE architecture considerations

For in-device coexistence, the UE architecture and implementation assumptions need to be considered.  For this short-term switching period analysis, intra-band operation without dual PA capability can be the baseline.   It may be possible that a better switching delay performance is possible by considering separate RF chains with dual PAs. However, for this analysis the baseline performance considering single PA is considered.

## 6.3 Switching time analysis

The switching time between NR SL and LTE SL for short-term TDM operation was analyzed as part of this study.  The general ON / OFF time mask requirements in subclause 6.3.3.2 in TS 38.101-1 is applicable for NR sidelink also.  In addition to the above general ON / OFF time masks, the additional switching period of about [150us] is applicable between the NR SL and LTE SL.  Considering that the NR V2X CBW is 10/20/30/40 MHz and that of LTE V2X is 10/20MHz, the spectrum span for contiguous and non-contiguous transmissions would be similar, the switching time can be similar.

<End of Changes>

# References

TR 38.886. Title: V2X Services based on NR; User Equipment (UE) radio transmission and reception