TSG-RAN Working Group 4 (Radio) meeting #94-e-bis	   R4-2005725

Electronic meeting, 20-30 April 2020

Source:	Ericsson

Title:	TP on Inclusion of NR-U standalone combinations in TR 38 716-01-01: NR intra band CA for xCC DL/yCC UL including contiguous and non-contiguous spectrum, (x>=y)

Agenda item:	6.1.3

Document for:	Approval

# Introduction

We have provided a discussion paper in [1] where we have provided description about standalone NR-U CA combinations. In this contribution, we propose the corresponding text proposals for the TR 38.716-01-01. This TP is a revised version of originally proposed TP in [2].

# TP

This section includes the proposed texts in this TR.

=========Start of the TP==============

## 5.6	CA_2DL_n7B_ 1UL_n7A

### 5.6.1	Operating band for CA

Table 5.6.1-1: intra-band contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n7 | n7 |

### 5.6.2	Channel bandwidths per operating band for CA

Table 5.6.2-1: Supported bandwidth combinations for CA_2DL_n1B_1UL _n1A

| NR CA Configuration | Uplink Configurations | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n7B | - | 10, 15, 20 | 10, 15, 20, 30, 35, 40 | 50 | 0 |

### 5.6.3	Co-existence studies

There are no co-existence issues for this combination.

## 5.7	CA_n46

### 5.7.1	Operating band for CA

Table 5.7.1-1: intra-band contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n46 | n46 |

### 5.7.2	Channel bandwidths per operating band for CA

Table 5.7.2-1: Supported bandwidth combinations for CA_n46

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_n46B | CA_n46A | 20, 40, 60 | 20, 40 |  |  |  | 100 |  |
| CA_n46C | CA_n46A | 60, 80 | 60, 80 |  |  |  | 160 |  |
| CA_n46D | CA_n46A | 60, 80 | 80 | 80 |  |  | 240 |  |
| CA_n46E | CA_n46A | 80 | 80 | 80 | 80 |  | 320 |  |
| CA_n46G | CA_n46A | 40, 60 | 40 | 40 |  |  | 140 |  |
| CA_n46H | CA_n46A | 40, 80 | 40 | 40 | 40 |  | 200 |  |
| CA_n46I | CA_n46A | 60 | 40 | 40 | 40 | 40 | 220 |  |

### 5.7.3	Co-existence studies

There are no co-existence issues for this combination.

### 5.7.4 REFSENS

In Rel-13 LAA, only CA was allowed involving band 46, thus we did not have any REFSENS values introduced in the REFSENS table in Section 7 of the spec.

In Rel-16, we will also have standalone NR-U operations. Following LAA specification, REFSENS for 15kHz SCS with 20MHz CBW can be reused as -90dBm. However, the other REFENS numbers need to be investigated.

|  | Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band |  | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n46 |  | 15 |  |  |  | -89.7 |  |  | -86.6 |  |  |  |  |  |  | TDD |
|  |  | 30 |  |  |  | -89.9 |  |  | -86.7 |  | -84.8 |  | -83.6 |  |  |  |
|  |  | 60 |  |  |  | -90.1 |  |  | -86.9 |  | -85.0 |  | -83.6 |  |  |  |

### 5.7.5 A-MPR studies

Since there will be wideband UL operation (UL CBW larger than 20MHz), A-MPR studies are required too.

# 6	Intra-Band Non-Contiguous Carrier Aggregation FR1: Specific Band Combination Part

## 6.1	CA_2DL_n66(2A)_1UL_n66A

=========End of the TP======================

# Proposal

It is proposed that the text in this TP can be approved for version 0.8.0 of the NR intra band CA for xCC DL/yCC UL including contiguous and non-contiguous spectrum, (x>=y) TR 38 716-01-01.

# References

- R4-2003413, “Introduction of standalone NR-U combinations in Rel-16”, Ericsson

- R4-2003415, “TP on Inclusion of NR-U standalone combinations in TR 38 716-01-01”, Ericsson